3GPP TSG-RAN5 Meeting #100	R5-234587r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0322 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new generic procedure MCData CO call release keeping the pre-established session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New MCData test procedures needed a generic procedure to release a call that was using a pre-established session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MCData generic procedure 5.3C.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3C.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 1) Corrected a typo in step 3 2) Changed step 7 to end the MSRP connection and added Notes 3) Corrected contents in Table 5.3C.13.4-3 4) Corrected Derivation Path in Table 5.3C.13.4-4 5) Added Table 5.3C.13.4-5 for SIP 200 (OK) from the UE |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.3C.13	MCData CO call release keeping the pre-established session

5.3C.13.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.13.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.13.3	Procedure

Table 5.3C.13.3-1: MCData CO call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) send a SIP REFER message with method “BYE” to release the MCData session and keep the pre-established session? | --> | SIP REFER | - | P |
| 2 | The SS (MCData server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 3 | The SS (MCX Server) sends a SIP re-INVITE to verify the release of the MCData call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 4a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 4a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 5 | Check: Does the UE (MCX client) respond to the SIP re-INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 6 | The SS (MCX server) sends a SIP ACK in response to the SIP 200 (OK) message. | <-- | SIP ACK | - | - |
| 7 | The SS waits 3s for the client to close the MSRP TCP connection. (NOTE 1, 2) | - | - | - | - |
| 7 | The SS waits 2 seconds before the SS releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. | - | - | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection. |  |  |  |  |  |

5.3C.13.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.13.4-1: SIP REFER (step 1, Table 5.3C.13.3-1)

Derivation Path: Table 5.5.2.12-1, condition METHOD-BYE

Table 5.3C.13.4-2: SIP 200 (OK) (step 2, Table 5.3C.13.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition REFER-RSP

Table 5.3C.13.4-3: SIP re-INVITE from the SS (step 3, Table 5.3C.13.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_SDS, re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | session identity of the pre-established session | TS 24.282 [31] clause 9.2.5.4.2.1 |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "application/vnd.3gpp.mcdata-info+xml" |  | TS 24.282 [31] clause 9.2.5.4.2.1 |  |
| Message-body |  |  |  |  |
| MCData-Info Message | MCData-Info message as described in Table 5.3C.13.4-4 |  |  |  |
| MIME body part | not present | SDP message |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info message as described in Table 5.3C.13.4-4 |  |  |  |
| Editor’s Note: TS 24.282 [31] clause 9.2.5.1.2 does not clearly specify the header fields of the INVITE and therefore the default header fields are used |  |  |  |  |

Table 5.3C.13.4-4: MCData-Info (Table 5.3C.13.4-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| anyExt |  |  |  |  |
| mcdata-communication-state | "terminated" |  | TS 24.282 [31] clause 9.2.5.4.2.1 |  |

Table 5.3C.13.4-5: SIP 200 (OK) from the UE (step 5, Table 5.3C.13.3-1)

| Derivation Path: Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Message-body | not present |  |  |  |

<END OF CHANGES>