3GPP TSG-RAN5 Meeting #100	R5-234622r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2875 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default ProSe PC5 signalling messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Default ProSe PC5 signalling messages for NR Sidelink Relay testing are missing from the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding default ProSe PC5 signalling messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR Sidelink Relay will not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update editorial errors |  |  |  |  |  |  |  |  |

4.7.6	Contents of UE Policy Delivery messages

<Skip unchanged sections>

–	UE POLICY PROVISIONING REJECT

Table 4.7.6-6: UE POLICY PROVISIONING REJECT

| Derivation Path: TS 24.587 Table 7.2.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | The same value as the value set in UE POLICY PROVISIONING REQUEST message. |  |  |
| UE POLICY PROVISIONING REJECT message identity |  | ’0000 0110’B |  |  |
| UPDS cause |  | '0001 1111'B | Request rejected, unspecified |  |

4.7.7	Contents of ProSe PC5 signalling messages

–	PROSE DIRECT LINK ESTABLISHMENT REQUEST

Table 4.7.7-1: PROSE DIRECT LINK ESTABLISHMENT REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK ESTABLISHMENT REQUEST message identity |  | '0000 0001'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Source user info |  |  |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| UE PC5 unicast signalling security policy |  | '0000 0010'B | Signalling integrity protection required, Signalling ciphering not needed. |  |
| MIC |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| ProSe identifiers |  | Not Present |  |  |
| Key establishment information container |  | Not Present |  |  |
| Nonce_1 |  | Not Present |  |  |
| MSB of KNRP-sess ID |  | Not Checked |  | Tx |
|  |  | '5C 00'H |  | Rx |
| Target user info |  | Not Checked |  | Tx |
| Target user info |  |  |  | Rx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application layer ID contents |  | '00 00 11 00'H |  |  |
| KNRP ID |  | Not Present |  |  |
| Relay service code |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| UE identity |  | Not Checked |  | Tx |
|  |  | SUCI of the UE |  | Rx |
| User security key ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| HPLMN ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| UTC-based counter LSB |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK ESTABLISHMENT ACCEPT

Table 4.7.7-2: PROSE DIRECT LINK ESTABLISHMENT ACCEPT

| Derivation Path: TS 24.554 [58] Table 10.3.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK ESTABLISHMENT ACCEPT message identity | '0000 0010'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Source user info |  |  |  |
| Length of Application layer ID contents | '04'H |  |  |
| Application Layer ID 1 | '00 00 02 00'H | Application Layer ID in target UE side |  |
| Configuration of UE PC5 unicast user plane security protection | '0000 0000'B | User plane integrity protection and ciphering is off |  |
| QoS flow descriptions |  |  |  |
| Length of PC5 QoS flow descriptions contents | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0001'B |  |  |
| Operation Code | '001'B | Create new PC5 QoS flow description |  |
| PII | 1 | Associated ProSe identifiers field is included |  |
| Number of parameters | 5 |  |  |
| E | 1 | parameters list is included |  |
| Associated ProSe identifiers |  |  |  |
| Length of ProSe identifier contents | '04'H |  |  |
| ProSe identifier 1 | '00 00 00 01'H |  |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 22 | Sensor sharing, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |
| Parameter identifier | '02'H | GFBR |  |
| Length of parameter contents | 3 |  |  |
| Parameter contents | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Parameter 3 |  |  |  |
| Parameter identifier | '03'H | MFBR |  |
| Length of parameter contents | 3 |  |  |
| Parameter contents | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Parameter 4 |  |  |  |
| Parameter identifier | '04'H | Averaging window |  |
| Length of parameter contents | 2 |  |  |
| Parameter contents | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |
| Parameter identifier | '06'H | Default priority level |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 4 |  |  |
| QoS rules | Not Present |  |  |
| IP address configuration | Not Checked |  | Tx |
| IP address configuration |  |  | Rx |
| IP address configuration IEI | '57'H |  |  |
| IP address configuration content | '0000 0001'B | IPv6 Router |  |
| Target link local IPv6 address | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK ESTABLISHMENT REJECT

Table 4.7.7-3: PROSE DIRECT LINK ESTABLISHMENT REJECT

| Derivation Path: TS 24.554 [58] Table 10.3.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK ESTABLISHMENT REJECT message identity | '0000 0011'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause | '0110 1111'B | Protocol error, unspecified |  |
| Back-off value | Not Present |  |  |
| EAP message | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK RELEASE REQUEST

Table 4.7.7-4: PROSE DIRECT LINK RELEASE REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.4.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK RELEASE REQUEST message identity | '0000 0111'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause | Not Checked |  | Tx |
|  | '0110 1111'B | Protocol error, unspecified | Rx |
| MSBs of KNRP ID | Not Checked |  | Tx |
|  | '00 00'H |  | Rx |
| Back-off value | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK RELEASE ACCEPT

Table 4.7.7-5: PROSE DIRECT LINK RELEASE ACCEPT

| Derivation Path: TS 24.554 [58] Table 10.3.5.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK RELEASE ACCEPT message identity | '0000 1000'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSBs of KNRP ID | Not Checked |  | Tx |
|  | '00 00'H |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK MODIFICATION REQUEST

Table 4.7.7-6: PROSE DIRECT LINK MODIFICATION REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.6.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK MODIFICATION REQUEST message identity |  | '0000 0100'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Link modification operation code |  | '0000 0011'B | Add new PC5 QoS flow(s) to the existing ProSe direct link |  |
| QoS flow descriptions |  |  |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0010'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| PII |  | 1 | Associated ProSe identifiers field is included |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated ProSe identifiers |  |  |  |  |
| Length of ProSe identifier contents |  | '04'H |  |  |
| ProSe identifier 1 |  | '00 00 00 02'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 23 | Platooning between UEs, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0000 1010'B | 10 * 4Mbps = 40Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0001 0100'B | 20 * 4Mbps = 80Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 3 |  |  |
| QoS rules |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK MODIFICATION ACCEPT

Table 4.7.7-7: PROSE DIRECT LINK MODIFICATION ACCEPT

| Derivation Path: TS 24.554 [58] Table 10.3.7.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK MODIFICATION ACCEPT message identity |  | '0000 0101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| QoS flow descriptions |  |  |  |  |
| PC5 QoS flow descriptions IEI |  | '79'H |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0010'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| PII |  | 1 | Associated ProSe identifiers field is included |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated ProSe identifiers |  |  |  |  |
| Length of ProSe identifier contents |  | '04'H |  |  |
| ProSe identifier 1 |  | '00 00 00 02'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 23 | Platooning between UEs, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0000 1010'B | 10 * 4Mbps = 40Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0001 0100'B | 20 * 4Mbps = 80Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 3 |  |  |
| QoS rules |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK KEEPALIVE REQUEST

Table 4.7.7-8: PROSE DIRECT LINK KEEPALIVE REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.8.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK KEEPALIVE REQUEST message identity | '0000 1001'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Keep-alive counter | '00 00 00 00'H | Increase by 1 after each keep-alive procedure. |  |
| Maximum inactivity period | Not Checked |  | Tx |
|  |  |  | Rx |
| Maximum inactivity period IEI | '55'H |  |  |
| Maximum inactivity period contents | '00 00 00 06'H |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK KEEPALIVE RESPONSE

Table 4.7.7-9: PROSE DIRECT LINK KEEPALIVE RESPONSE

| Derivation Path: TS 24.554 [58] Table 10.3.9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK KEEPALIVE RESPONSE message identity | '0000 1010'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Keep-alive counter | '00 00 00 01'H |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK AUTHENTICATION REQUEST

Table 4.7.7-10: PROSE DIRECT LINK AUTHENTICATION REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK AUTHENTICATION REQUEST message identity | '0000 1011'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Key establishment information container | Not checked |  | Tx |
|  |  |  | Rx |
| Length of key establishment information container contents | '00'H |  |  |
| Key establishment information container contents | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK AUTHENTICATION RESPONSE

Table 4.7.7-11: PROSE DIRECT LINK AUTHENTICATION RESPONSE

| Derivation Path: TS 24.554 [58] Table 10.3.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK AUTHENTICATION RESPONSE message identity | '0000 1100'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Key establishment information container | Not checked |  | Tx |
|  |  |  | Rx |
| Length of key establishment information container contents | '00'H |  |  |
| Key establishment information container contents | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK AUTHENTICATION REJECT

Table 4.7.7-12: PROSE DIRECT LINK AUTHENTICATION REJECT

| Derivation Path: TS 24.554 [58] Table 10.3.12.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PROSE DIRECT LINK AUTHENTICATION REJECT message identity | '0000 1101'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause value | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK SECURITY MODE COMMAND

Table 4.7.7-13: PROSE DIRECT LINK SECURITY MODE COMMAND

| Derivation Path: TS 24.554 [58] Table 10.3.13.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK SECURITY MODE COMMAND message identity |  | '0000 1110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Selected security algorithms |  | '0000 0000'B |  |  |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| UE PC5 unicast signalling security policy |  |  |  |  |
| UE PC5 unicast signalling security policy IEI |  | '59'H |  |  |
| Signalling integrity protection policy |  | '000'B | Signalling integrity protection not needed |  |
| Signalling ciphering policy |  | '000'B | Signalling ciphering not needed. |  |
| Nonce_2 |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| LSB of KNRP-sess ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| Key establishment information container |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| MSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| GPI |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| EAP message |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK SECURITY MODE COMPLETE

Table 4.7.7-14: PROSE DIRECT LINK SECURITY MODE COMPLETE

| Derivation Path: TS 24.554 [58] Table 10.3.14.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK SECURITY MODE COMPLETE message identity |  | '0000 1111'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| UE PC5 unicast user plane security policy |  | '0000 0000'B | Signalling integrity protection not needed, Signalling ciphering not needed. |  |
| QoS flow descriptions |  |  |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0001'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters |  | '05'H |  |  |
| E |  | '1'B | parameters list is included |  |
| Associated ProSe identifiers |  |  |  |  |
| Length of ProSe identifier contents |  | '12'H |  |  |
| ProSe identifier 1 |  | '00 00 00 00 00 00 00 00 00 00 00 01'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | '01'H |  |  |
| Parameter contents |  | '0001 0110'B |  |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | '03'H |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | '03'H |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | '02'H |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | '01'H |  |  |
| Parameter contents |  | '0000 0100'B |  |  |
| IP address configuration |  | Not Checked |  | Tx |
|  |  |  |  | Rx |
| IP address configuration IEI |  | '57'H |  |  |
| IP address configuration content |  | '0000 0001'B | IPv6 Router |  |
| Target link local IPv6 address |  | Not Checked |  | Tx |
|  |  | Not present |  | Rx |
| LSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not present |  | Rx |
| QoS rules |  | Not Checked |  | Tx |
|  |  | Not present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK SECURITY MODE REJECT

Table 4.7.7-15: PROSE DIRECT LINK SECURITY MODE REJECT

| Derivation Path: TS 24.554 [58] Table 10.3.15.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK SECURITY MODE REJECT message identity |  | '0001 0000'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |
| RAND |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| AUTS |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK REKEYING REQUEST

Table 4.7.7-16: PROSE DIRECT LINK REKEYING REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.16.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK REKEYING REQUEST message identity |  | '0001 0001'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| Key establishment information container |  | Not Checked |  |  |
| Nonce_1 |  | Not Checked |  |  |
| MSBs of KNRP-sess ID |  | Not Checked |  |  |
| Re-authentication indication |  |  |  |  |
| Re-authentication indication IEI |  | '56'H |  |  |
| Re-authentication indication contents |  | '0000 0000'B | KNRP is not requested to be refreshed |  |
| User security key ID |  | Not Checked |  |  |
| HPLMN ID |  | Not Checked |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK REKEYING RESPONSE

Table 4.7.7-17: PROSE DIRECT LINK REKEYING RESPONSE

| Derivation Path: TS 24.554 [58] Table 10.3.17.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK REKEYING RESPONSE message identity |  | '0001 0010'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST

Table 4.7.7-18: PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST

| Derivation Path: TS 24.554 [58] Table 10.3.18.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message identity |  | '0001 0011'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| MSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Source layer-2 ID |  | Not checked |  | Tx |
|  |  | '00 00 10'H | New Layer-2 ID in initiating UE side | Rx |
| Source user info |  | Not checked |  | Tx |
|  |  |  |  | Rx |
| Application layer ID IEI |  | ‘27’H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H | New application Layer ID in initiating UE side |  |
| Source link local IPv6 address |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT

Table 4.7.7-19: PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT

| Derivation Path: TS 24.554 [58] Table 10.3.19.1.1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message identity |  | '0001 0100'B |  |  |  |  |  |
| Sequence number |  | Not Checked |  | 0~255, uniquely identify a PC5 signalling message being sent or received |  | Tx |  |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. |  | 0~255, uniquely identify a PC5 signalling message being sent or received |  | Rx |  |
| LSB of KNRP-sess ID |  | Not checked |  |  |  | Tx |  |
|  |  | '00'H |  |  |  | Rx |  |
| MSB of KNRP-sess ID |  | Not checked |  |  |  | Tx |  |
|  |  | '00'H |  |  |  | Rx |  |
| Source layer-2 ID |  | Not checked |  |  |  | Tx |  |
|  |  | '00 00 20'H |  |  |  | Rx |  |
| Target layer-2 ID |  | '00 00 10'H |  |  |  | Tx |  |
|  |  | Same as the source layer-2 ID in PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition |  |  |  | Rx |  |
| Target user info |  | Not Present |  |  |  | Rx AND (NOT Update_user_info) |  |
| Target user info |  |  |  |  |  |  |  |
| Application layer ID IEI |  | '28'H |  |  |  |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |  |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H |  | New application Layer ID in initiating UE side |  | Tx |  |
|  |  | Same as the source user info in PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition |  | New application Layer ID in initiating UE side |  | Rx AND Update_user_info |  |
| Target link local IPv6 address |  | Not Present |  |  |  |  |  |
| Source user info |  | Not checked |  |  |  | Tx |  |
|  |  |  |  |  |  | Rx |  |
| Application layer ID IEI |  | '27'H |  |  |  |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |  |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H |  | New application Layer ID in target UE side |  |  |  |
| Source link local IPv6 address |  | Not Present |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST. |

–	PROSE DIRECT LINK IDENTIFIER UPDATE ACK

Table 4.7.7-20: PROSE DIRECT LINK IDENTIFIER UPDATE ACK

| Derivation Path: TS 24.554 [58] Table 10.3.20.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK IDENTIFIER UPDATE ACK message identity |  | '0001 0101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Target layer-2 ID |  | '00 00 20'H |  | Tx |
|  |  | Same as the source layer-2 ID in PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition |  | Rx |
| Target user info |  | Not Present |  | Rx AND (NOT Update_user_info) |
| Target user info |  |  |  |  |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H | New application Layer ID in target UE side | Tx |
|  |  | Same as the source user info in PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition | New application Layer ID in target UE side | Rx AND Update_user_info |
| Target link local IPv6 address |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST. |

–	PROSE DIRECT LINK IDENTIFIER UPDATE REJECT

Table 4.7.7-21: PROSE DIRECT LINK IDENTIFIER UPDATE REJECT

| Derivation Path: TS 24.554 [58] Table 10.3.21.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK IDENTIFIER UPDATE REJECT message identity |  | '0001 0110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK MODIFICATION REJECT

Table 4.7.7-22: PROSE DIRECT LINK MODIFICATION REJECT

| Derivation Path: TS 24.554 [58] Table 10.3.22.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK MODIFICATION REJECT message identity |  | '0000 0110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	PROSE DIRECT LINK AUTHENTICATION FAILURE

Table 4.7.7-23: PROSE DIRECT LINK AUTHENTICATION FAILURE

| Derivation Path: TS 24.554 [58] Table 10.3.23.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PROSE DIRECT LINK AUTHENTICATION FAILURE message identity |  | '0001 0111'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Key establishment information container |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |