3GPP TSG-RAN5 Meeting #100	R5-234695r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2882 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for registration of NR sidelink U2N Relay / Relay UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test procedure for registration of NR sidelink U2N Relay / Relay UE side is missing from the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test procedure for registration of NR sidelink U2N Relay / Relay UE side |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay / Relay UE side will be not implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | '4.9.X' shall be resolved according to TS38.508-1 CR 2880 and '4.9.Y' shall be resolved according to TS38.508-1 CR 2881 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:Updating the initial conditions and messages. |  |  |  |  |  |  |  |  |

4.9.Z	Test procedure for registration of NR sidelink U2N Relay / Relay UE side

4.9.Z.1	Scope

The purpose of this procedure is testing relay UE behavior when remote UE of NR sidelink U2N Relay is performing registration procedure.

4.9.Z.2	Procedure description

4.9.Z.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

-	1 NR-SS-UE

-	1 GNSS simulator

User Equipment:

-	The UE is in state 3N-A.

4.9.Z.2.2	Procedure sequence

Table 4.9.Z.2.2-1: Procedure for registration of NR sidelink U2N Relay / Relay UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). |  |  |  |  |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.X is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.Y is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3 | The NR-SS-UE1 transmits an RRCSetupRequest message. | <-- | PC5-RRC: RRCSetupRequest | - | - |
| 4 | The UE transmits a SidelinkUEInformationNR message. | --> | NR RRC: SidelinkUEInformationNR | - | - |
| 5 | The SS transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - |  |
| 7 | The UE relays the RRCSetupRequest message received at step 3. | --> | NR RRC: RRCSetupRequest | - | - |
| 8 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 9 | The UE relays the RRCSetup message received at step 8. | --> | PC5-RRC: RRCSetup | - | - |
| 10 | The NR-SS-UE1 transmits an RRCSetupComplete message. | <-- | PC5-RRC: RRCSetupComplete | - | - |
| 11 | The UE relays the RRCSetupComplete message received at step 10. | --> | NR RRC: RRCSetupComplete | - | - |
| 12 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 13 | The UE relays the SecurityModeCommand message received at step 12. | --> | PC5-RRC: SecurityModeCommand | - | - |
| 14 | The NR-SS-UE1 transmits a SecurityModeComplete message. | <-- | PC5-RRC: SecurityModeComplete | - | - |
| 15 | The UE relays the SecurityModeComplete message received at step 14. | --> | NR RRC: SecurityModeComplete | - | - |
| 16 | The SS transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | The UE relays the RRCReconfiguration message received at step 16. | --> | PC5-RRC: RRCReconfiguration | - | - |
| 18 | The NR-SS-UE1 transmits an RRCReconfigurationComplete message | <-- | PC5-RRC: RRCReconfigurationComplete | - | - |
| 19 | The UE relays the RRCReconfigurationComplete message received at step 18. | --> | NR RRC: RRCReconfigurationComplete | - | P |

4.9.Z.3	Specific message contents

Table 4.9.Z.3-1: RRCSetupRequest (steps 3 and 7, Table 4.9.Z.2.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-23 with condition L2RemoteUE

Table 4.9.Z.3-2: SidelinkUEInformationNR (Step 4, Table 4.9.Z.2.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-28A with condition L2RelayUE

Table 4.9.Z.3-3: RRCReconfiguration (step 5, Table 4.9.Z.2.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RelayUE and RELAY

Table 4.9.Z.3-4: RRCReconfiguration (steps 16 and 17, Table 4.9.Z.2.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition L2RemoteUE and RELAY