3GPP TSG-RAN5 Meeting #100	R5-234696r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2883 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Generic procedures for NR sidelink U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Generic procedure of NR sidelink U2N Relay UE should be clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding of generic procedure of NR sidelink U2N Relay UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Remote UE will not be implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4A.2, 4.4A.5, 4.5.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | '4.9.X' shall be resolved according to TS38.508-1 CR 2880 and '4.9.Y' shall be resolved according to TS38.508-1 CR 2881 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Adding UE state description; Update editorial errors. |  |  |  |  |  |  |  |  |

4.4A.2	Test states and associated 5GC and RRC protocol states

Table 4.4A.2-0: 5GC and RRC/N3AN protocol states for UE Switched Off

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 0-A | - | - | - | - | - | UE switched off. No change to PLMN stored in the USIM |
| 0N-B | NR | - | - | - | - | UE switched off with the PLMN under test stored in the USIM |
| 0E-B | E-UTRA | - | - | - | - |  |
| 0W-B | WLAN | - | - | - | - |  |

Table 4.4A.2-1: 5GC and RRC/N3AN protocol states for IDLE

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 1N-A | NR | NR RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1E-A | E-UTRA | EUTRA RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1W-A | WLAN | Ipsec_SA_Released | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 1N-B | NR | NR RRC_IDLE | 5GMM-IDLE | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-2: 5GC and RRC protocol states for INACTIVE

| 5GS state ID | Connectivity | RRC state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 2N-A | NR | NR RRC_INACTIVE | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 2E-A | E-UTRA | EUTRA RRC_ INACTIVE | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-3: 5GC and RRC/N3AN protocol states for CONNECTED

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 3N-A | NR | NR RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3E-A | E-UTRA | EUTRA RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3W-A | WLAN | Ipsec_SA_Established | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |
| 3N-B | NR | NR RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

Table 4.4A.2-4: 5GC and RRC/N3AN protocol states for Out of Coverage

| 5GS state ID | Connectivity | RRC/N3AN state | 5GMM modes | 5GMM sublayer | 5GSM sublayer |
| --- | --- | --- | --- | --- | --- |
| 4-A | - | - | - | - | - |
| 4N-A | NR | NR RRC_ CONNECTED | 5GMM-CONNECTED | 5GMM-REGISTERED | PDU SESSION INACTIVE |
|  |  |  |  |  | PDU SESSION ACTIVE |

<skip unchanged sections>

4.4A.5	Mapping of test state IDs and test parameters to generic procedures, generic procedure parameters and specific message conditions

Depending on the test case preamble requested test state ID and test parameters the SS shall:

1> use the applicable generic procedure as specified in Table 4.4A.5-1 using the:

2> applicable generic procedure parameters as specified in Table 4.4A.5-1 and Table 4.4A.5-2; and

2> applicable message conditions as specified in Table 4.4A.5-2.

Table 4.4A.5-1: Test state ID mapping to generic procedures and Connectivity generic procedure parameter

| Test state ID |  |  |  |  |  |  |  | Generic Procedure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC state field |  | Connectivity field |  |  | Variant field |  |  | Name |  | Generic procedure parameter (Note 1) |  |  | Clause |
| 0 |  |  | - |  |  | A |  |  | SWITCHED_OFF |  |  |  | 4.5.5 |
| 0 |  |  | N |  |  | B |  |  | SWITCHED_OFF |  | Connectivity=NR |  | 4.5.5 |
| 0 |  |  | E |  |  | B |  |  | SWITCHED_OFF |  | Connectivity=E-UTRA |  | 4.5.5 |
| 0 | W |  |  | B |  |  | SWITCHED_OFF |  |  |  |  | Connectivity=WLAN | 4.5.5 |
| 1 |  | N |  |  | A |  |  | RRC_IDLE |  | Connectivity=NR |  |  | 4.5.2 |
| 1 |  | E |  |  | A |  |  | RRC_IDLE |  | Connectivity=E-UTRA |  |  | 4.5.2 |
| 1 | W |  |  | A |  |  | Ipsec_SA_Released |  |  |  |  | Connectivity=WLAN | 4.5.2 |
| 1 | N |  |  | B |  |  | RRC_IDLE_with_SL |  |  |  |  | Connectivity=NR, Sidelink=On | 4.5.2 |
| 2 |  | N |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=NR |  |  | 4.5.3 |
| 2 |  | E |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=E-UTRA |  |  | 4.5.3 |
| 3 |  | N |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=NR |  |  | 4.5.4 |
| 3 |  | E |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=E-UTRA |  |  | 4.5.4 |
| 3 | W |  |  | A |  |  | Ipsec_SA_Established |  |  |  |  | Connectivity=WLAN | 4.5.4 |
| 3 | N |  |  | B |  |  | RRC_CONNECTED_with_SL |  |  |  |  | Connectivity=NR, Sidelink=On | 4.5.4 |
| 4 | - |  |  | A |  |  | Out_of_Coverage |  |  |  |  | Sidelink = On | 4.5.7 |
| 4 | N |  |  | A |  |  | Out_of_Coverage_with_Relay |  |  |  |  | Connectivity=NR U2N Sidelink Relay | 4.5.8 |
| Note 1: In addition to the Connectivity parameter specified in this table the applicable additional generic procedure parameters and conditions as stated in Table 4.4A.5-2 shall be used |  |  |  |  |  |  |  |  |  |  |  |  |  |

<skip unchanged sections>

4.5.8	NR sidelink U2N Relay

4.5.8.1	Initiation

The SS shall:

1>	If sidelink is On

2>	use neither NR cell nor E-UTRA cell;

2>	use 1 GNSS simulator;

2>	use 1 NR-SS-UE;

2>perform the procedure according to the table 4.5.8.2-1: NR sidelink U2N Relay;

4.5.8.2	Procedures

Table 4.5.8.2-1: NR sidelink U2N Relay

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Same as table 4.5.7.2-1, steps 1a1-1A. | - | - | - | - |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.X is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.Y is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3a1-4 | Same as table 4.5.7.2-1, steps 3a1-4. | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | PC5-RRC: RRCSetupRequest | - | - |
| 6 | The NR-SS-UE transmits an RRCSetup message. | <-- | PC5-RRC: RRCSetup | - | - |
| 7 | The UE transmits an RRCSetupComplete message. | --> | PC5-RRC: RRCSetupComplete | - | - |
| 8 | The NR-SS-UE transmits a SecurityModeCommand message. | <-- | PC5-RRC: SecurityModeCommand | - | - |
| 9 | The UE transmits a SecurityModeComplete message. | --> | PC5-RRC: SecurityModeComplete | - | - |
| 10 | The NR-SS-UE transmits an RRCReconfiguration message. | <-- | PC5-RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | PC5-RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 12a1-12a3 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 12a1 | IF connected without release is not present THEN, the NR-SS-UE transmits an RRCRelease message. | <-- | PC5-RRC: RRCRelease | - | - |
| 12a2 | The UE sends a RemoteUEInformationSidelink message. | --> | PC5-RRC: RemoteUEInformationSidelink | - | - |
| 12a3 | The NR-SS-UE transmits a UuMessageTransferSidelink message. | <-- | PC5-RRC: UuMessageTransferSidelink | - | - |

4.5.8.3	Specific message contents

All specific message contents shall be according subclause 4.6 with the exceptions below.

Table 4.5.8.3-1: Message RRCSetup (steps 6, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-21 with condition L2RemoteUE

Table 4.5.8.3-2: Message RRCReconfiguration (steps 10, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-13 with condition L2RemoteUE and RELAY

Table 4.5.8.3-3: Message RRCRelease (steps 12a1, Table 4.5.8.2-1)

Derivation path: Table 4.6.1-16 with condition L2RemoteUE

<End of Changed Section>