3GPP TSG-RAN5 Meeting #100	R5-234703r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3951 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NEDC TC 8.2.6.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to TP, the NR Cell 1 in which SIB1 includes idleModeMeasurementsEUTRA; - There are not 5GS state 3N-A in TS 38.508-1 Table 4.4A.2-1; - AT step 14, there are not IE measResultIdleEutra in the RRCResumeComplete message according to TS 38.331; - In the Table 8.2.6.3.4.3.3-5 & Table 8.2.6.3.4.3.3-6, the value of some IEs is missing; - Editorial errors |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added condition EMR_EUTRA for SIB1 in the System Simulator; - Updated Table 4.4A.2-1 to Table 4.4A.2-3; - Updated IE measResultIdleEutra to measResultIdleEUTRA; - Added them according to TS 38.331; - Modifed them |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:updated the IE value in the table 8.2.6.3.4.3.3-10 |  |  |  |  |  |  |  |  |

<Start of Changed Section>

8.2.6.3.4	Idle / Inactive measurements / Inactive mode / NE-DC / RRCRelease configuration

8.2.6.3.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message including suspendConfig and measIdleConfig with measIdleCarrierListEUTRA while being connected to a serving cell in which SIB1 includes idleModeMeasurementsEUTRA and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message including suspendConfig and measIdleConfig with measIdleCarrierListEUTRA while being connected to a serving cell in which SIB1 includes idleModeMeasurementsEUTRA and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives RRCResume message with idleModeMeasurementReq after successful measurement procedure }

then { UE reports measurement results in RRCResumeComplete message }

(3)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message including suspendConfig and measIdleConfig without measIdleCarrierListEUTRA while being connected to a serving cell in which SIB1 includes idleModeMeasurementsEUTRA and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE does not report measurement results in UEInformationResponse message }

}

8.2.6.3.4.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clauses 5.3.8.3, 5.7.8.1, 5.7.8.1a, 5.7.8.2a, 5.7.8.3, 5.3.13.4 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

[TS 38.331, clause 5.7.8.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 38.331, clause 5.7.8.1a]

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveEUTRA-MeasReport:

3>	if the SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListEUTRA:

4>	store or replace the measIdleCarrierListEUTRA of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListEUTRA in VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA:

3>	for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

4>	if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry:

5>	perform measurements in the carrier frequency and bandwidth indicated by carrierFreqEUTRA and allowedMeasBandwidth within the corresponding entry;

5>	if the reportQuantitiesEUTRA is set to rsrq:

6>	consider RSRQ as the sorting quantity;

5>	else:

6>	consider RSRP as the sorting quantity;

5>	if the measCellListEUTRA is included:

6>	consider cells identified by each entry within the measCellListEUTRA to be applicable for idle/inactive mode measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive measurement results for the measurement quantities indicated by reportQuantitiesEUTRA;

5>	store the derived measurement results as indicated by reportQuantitiesEUTRA within the measReportIdleEUTRA in VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThresholdEUTRA is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdEUTRA;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.3.13.4]

The UE shall:

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

4>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

2>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.2.6.3.4.3	Test Description

8.2.6.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2. Condition EMR_EUTRA used for SIB1.

-	E-UTRA Cell 1, suitable for inter-frequency measurements. System information combination 1 as defined in 36.508 [7] clause 4.4.3.1.1.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.2.6.3.4.3.2	Test procedure sequence

Table 8.2.6.3.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig with measIdleCarrierListEUTRA to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 66 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 3 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 4 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 5 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 6 | UE transmits RRCResumeComplete message with idleMeasAvailable to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 7 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 8 | Check: Does the UE transmit UEInformationResponse including measResultIdleEUTRA? | --> | NR RRC: UEInformationResponse | 1 | P |
| 9 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig with measIdleCarrierListEUTRA to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 10 | Wait for 66 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 11 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 12 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 13 | SS transmits RRCResume message with idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 14 | Check: Does UE transmit RRCResumeComplete message including measResultIdleEUTRA to SS. | --> | NR RRC: RRCResumeComplete | 2 | P |
| 15 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig without measIdleCarrierListEUTRA to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 16 | Wait for 66 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 17 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 18 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 19 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 20 | UE transmits RRCResumeComplete message without idleMeasAvailable to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 21 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 22 | Check: Does the UE transmit UEInformationResponse without measResultIdleEUTRA? | --> | NR RRC: UEInformationResponse | 3 | P |

8.2.6.3.4.3.3	Specific message contents

Table 8.2.6.3.4.3.3-1: RRCRelease (steps 1, 9 and 15, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-16 with condition NR_RRC_INACTIVE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 SetupRelease { |  |  |  |
| setup | MeasIdleConfigDedicated1 | Table 8.2.6.3.4.3.3-2 | step 1 and 9 |
|  | MeasIdleConfigDedicated2 | Table 8.2.6.3.4.3.3-3 | step 15 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.4.3.3-2: MeasIdleConfigDedicated1 (Table 8.2.6.3.4.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 { | 1 entry |  |  |
| MeasIdleCarrierEUTRA-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreqEUTRA-r16 | ARFCN value corresponding to E-UTRA cell 1 |  |  |
| allowedMeasBandwidth-r16 | mbw50 |  |  |
| measCellListEUTRA-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange { | 1 entry |  |  |
| EUTRA-PhysCellIdRange[1] SEQUENCE { |  | entry 1 |  |
| start | EUTRA-PhysCellId corresponding to E-UTRA cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuatitiesEUTRA-r16 | both |  |  |
| qualityThresholdEUTRA-r16 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not Present |  |  |
| } |  |  |  |

Table 8.2.6.3.4.3.3-3: MeasIdleConfigDedicated2 (Table 8.2.6.3.4.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| measIdleCarrierListEUTRA-r16 | Not Present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not Present |  |  |
| } |  |  |  |

Table 8.2.6.3.4.3.3-4: RRCResume (step 13, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508 [4], table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.3.4.3.3-5: RRCResumeComplete (step 6 and 20, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | Any allowed value |  | step 6 |
|  |  | Not present |  | step 20 |
| measResultIdleEUTRA-r16 |  | Not checked |  |  |
| measResultIdleNR-r16 |  | Not checked |  |  |
| scg-Response-r16 |  | Not checked |  |  |
| ue-MeasurementsAvailable-r16 |  | Not checked |  |  |
| mobilityHistoryAvail-r16 |  | Not checked |  |  |
| mobilityState-r16 |  | Not checked |  |  |
| needForGapsInfoNR-r16 |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.3.4.3.3-6: RRCResumeComplete (step 14, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | Not checked |  |  |
| measResultIdleEUTRA-r16 |  | MeasResultIdleEUTRA |  |  |
| measResultIdleNR-r16 |  | Not checked |  |  |
| scg-Response-r16 |  | Not checked |  |  |
| ue-MeasurementsAvailable-r16 |  | Not checked |  |  |
| mobilityHistoryAvail-r16 |  | Not checked |  |  |
| mobilityState-r16 |  | Not checked |  |  |
| needForGapsInfoNR-r16 |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.3.4.3.3-7: UEInformationRequest (steps 7 and 21, Table 8.2.6.3.4.3.2-1)

Derivation Path: TS 38.508-1 [4], table 4.6.1-32A with condition IDLE

Table 8.2.6.3.4.3.3-8: UEInformationResponse (step 8, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleEUTRA-r16 | MeasResultIdleEUTRA |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.4.3.3-9: UEInformationResponse (step 22, Table 8.2.6.3.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.4.3.3-10: MeasResultIdleEUTRA (Table 8.2.6.3.4.3.3-6 and Table 8.2.6.3.4.3.3-8)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResultIdleEUTRA-r16 ::= SEQUENCE { |  |  |  |
| measResultsPerCarrierListIdleEUTRA-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleEUTRA-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleEUTRA-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreqEUTRA-r16 | ARFCN value corresponding to E-UTRA cell 1 |  |  |
| measResultsPerCellListIdleEUTRA-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleEUTRA-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleEUTRA-r16[1] SEQUENCE { |  | entry 1 |  |
| eutra-PhysCellId-r16 | EUTRA-PhysCellId corresponding to E-UTRA cell 1 |  |  |
| measIdleResultEUTRA-r16 SEQUENCE { |  |  |  |
| rsrp-ResultEUTRA-r16 | Any allowed value |  |  |
| rsrq-ResultEUTRA-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>