3GPP TSG-RAN5 Meeting #100	R5-234763r2

# Toulouse, France, Aug 21 - 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex C in 36521-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | In the updated WP for RA5#100, Annex C has been allocated to “Downlink Physical Channels”, and Annex C in 36521-4 needs to be added accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex C in 36521-4 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: According to TS 36.102 Annex B, Section C.2, C.3.0, C.3.1 and C.3.2 are updated, and Section 3.3 is added R2: Update C.2 to merge the corresponding configurations for category M1 and NB-IoT NTN from TS 36.521-1 Annex C. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Annex C (normative):
Downlink Physical Channels

This annex specifies the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

C.0	Downlink signal levels

The downlink power settings in Table C.0-1 or Table C.0-2 are used unless otherwise specified in a test case.

For the UE has one Rx antenna, the downlink signal is applied to it.

Table C.0-1: Default Downlink power levels for category M1

|  | Unit | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Number of RBs |  | 6 |
| Channel BW Power | dBm | -66 |
| RS EPRE | dBm/15kHz | -85 |
| Note 1: The channel bandwidth powers and RB allocations are informative, based on -85dBm/15kHz RS_EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed in this calculation, but allocation may vary during setup. Note 2: The power level is specified at each UE Rx antenna. |  |  |

Table C.0-2: Default Downlink power levels for category NB1 and NB2

|  | Unit |  |
| --- | --- | --- |
| Subcarriers |  | 12 |
| Channel BW Power | dBm | -74 |
| NRS EPRE | dBm/15kHz | -85 |
| Note 1: The channel bandwidth power is informative, based on -85dBm/15kHz NRS_EPRE, then scaled according to the number of subcarriers and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed in this calculation, but allocation may vary during setup. Note 2: The power level is specified at the UE Rx antenna |  |  |

The default signal level uncertainty is +/-3dB at each test port, for any level specified. If the uncertainty value is critical for the test purpose, a tighter uncertainty is specified for the related test case in Annex F.

C.1	General

Table C.1-1 describes the mapping of downlink physical channels and signals to physical resources for category M1 FDD.

Table C.1-3 describes the mapping of downlink physical channels and signals to physical resources for category NB1 and NB2.

Table C.1-1: Mapping of downlink physical channels and signals to physical resources for category M1 FDD

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| RS | Symbols 0, 4 of each subframe for antenna port 0 & 1 Symbol 1 of each subframe for antenna port 2 & 3 | Downlink system bandwidth dependent | Mapping rule is specified in TS 36.211 6.10.1.2 - CELL_ID = 0 |
| PBCH | Symbols 0 to 3 of slot 1 of subframe 0 of each radio frame | Occupies 72 subcarriers centred on the DC subcarrier | Mapping rule is specified in TS 36.211 Section 6.6.4 (Note 2) |
| PSS | Symbol 6 of slot 0 and 10 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS 36.211 Section 6.11.1.2 |
| SSS | Symbol 5 of slots 0 and 10 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS 36.211 Section 6.11.2.2 |
| PCFICH | Symbol 0 of each subframe | Downlink system bandwidth dependent. Maps into 4 REGs uniformly spread in the frequency domain over the whole system bandwidth. | Mapping rule is specified in TS 36.211 Section 6.7.4 (Note 1) - CELL_ID = 0 |
| PHICH | Symbol 0 of each subframe | Downlink system bandwidth dependent. Each PHICH group maps into 3 REGs in the frequency domain on the REGs not assigned to PCFICH over the whole system bandwidth | Mapping rule is specified in TS 36.211 Section 6.9.3 (Note 1) - CELL_ID = 0 - Ng = 1 - Normal PHICH duration -Number of PHICH groups = 1(BW=1.4MHz)/2(BW=3MHz)/ 4(BW=5MHz)/7(BW=10MHz)/ 10(BW=15MHz)/13(BW=20MHz) Not required for UE category M1 testing |
| PDCCH | Symbols 0, 1, 2, 3 of each subframe for 1.4 MHz Symbols 0, 1, 2, of each subframe for 3 and 5 MHz Symbols 0, 1 of each subframe for 10, 15 and 20 MHz | The remaining REGs not allocated to both PCFICH and PHICH are used for PDCCH | Mapping rule is specified in TS 36.211 Section 6.8.5 (Note 1) |
| MPDCCH | Same as PDSCH | For Subframe 0, subcarriers corresponding to resource elements not allocated to RS, PSS, SSS and PBCH (core set and repetitions) and reserved for PBCH antenna ports 3 and 4 on the 72 central subcarriers. For Subframe 5, subcarriers corresponding to resource elements not allocated to RS, PSS and SSS. For Subframe 9, subcarriers corresponding to resource elements not allocated to RS, PSS, SSS and PBCH (repetitions). For other subframes, subcarriers corresponding to resource elements not allocated to RS. | Mapping rule is specified in TS 36.211 Section 6.8B.5 (Note 1) Only required for UE category M1 testing |
| PDSCH | All remaining OFDM symbols of each subframe not allocated to PDCCH | For Subframe 0, REs not allocated to RS, PSS, SSS and PBCH, is allocated to PDSCH For Subframe 5, REs not allocated to RS, PSS and SSS, is allocated to PDSCH For other subframes, REs not allocated to RS, is allocated to PDSCH | Note that there are reserved REs that are not used for transmission of any physical channels (Note 3 ) & (Note 4) which need to be taken into account when allocating REs to PDSCH |
| Note 1: In case a single cell-specific RS is configured, cell-specific RS shall be assume to be present on antenna ports 0 and 1 for the purpose of mapping a symbol-quadruplet to a REG (resource-element group). (See TS 36.211 Section 6.2.4). Note 2: PBCH is mapped into RE assuming RS from 4 antennas are used at the eNB transmitter, irrespective of the actual number of Tx antenna. Resource elements assumed to be reserved for RS but not used for transmission of RS shall not be used for transmission of any physical channel. (See TS 36.211 Section 6.6.4). Note 3: In slot 0 and slot 10 of each radio frame, there are reserved REs for PSS and SSS that are not used for transmission of any physical channels. (See TS 36.211 Section 6.11.1.2 & 6.11.2.2). Note 4: REs used for RS transmission on any of the antenna ports in a slot shall not be used for any transmission on any other antenna port in the same slot and set to zero. (See TS 36.211 Section 6.10.1.2). |  |  |  |

Table C.1-2: FFS

Table C.1-3: Mapping of downlink physical channels and signals to physical resources for category NB1 and NB2

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPBCH | NPBCH is transmitted in subframe 0 in every radio frame. NPBCH consists of 8 independently decodable blocks of 80 ms duration. The time interval where MIB remains unchanged is 640 ms. NPBCH does not use the first 3 symbols in a subframe in in-band operation. For stand-alone and guard-band, the first 3 symbols (of the subframe transmitting NPBCH) contain no NPBCH. | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS 36.211 [8] sub clause 10.2.4.4 |
| NPSS | NPSS is transmitted in subframe 5. NPSS uses the last 11 OFDM symbols of subframes in which NB-PSS occurs for normal CP. NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS NSSS periodicity is 10ms. | NPSS is mapped to sub-carriers #0-10 of the NB-IoT carrier | Mapping rule is specified in TS 36.211 [8] sub clause 10.2.7.1.2 |
| NSSS | NSSS is transmitted in subframe 9. NSSS uses the last 11 OFDM symbols of subframes in which NB-SSS occurs for normal CP. NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS NSSS periodicity is 20ms. | The number of subcarriers for NSSS is 12 | Mapping rule is specified in TS 36.211 [8] sub clause 10.2.7.2.2 |
| NPDCCH | NPDCCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier. NPDCCH are not be mapped onto resources elements used for NRS. NPDCCH are not overlapped with PBCH, PSS, SSS, or CRS. One or two NPDCCHs can be transmitted in a subframe. In in-band, first 3 OFDM symbols are not used for NPDCCH. In stand-alone and guard-band, all OFDM symbols are available for NPDCCH. NPDCCH and NPDSCH are multiplexed only based on TDM at subframe level: - It means that only cross subframe scheduling is supported - The start of an NPDCCH search space is >=4ms after the end of the last NPDCCH search space | NPDCCH is transmitted on an aggregation of one or two consecutive narrowband control channel elements (NCCEs), where a narrowband control channel element corresponds to 6 consecutive subcarriers in a subframe where NCCE 0 occupies subcarriers 0 through 5 and NCCE 1 occupies subcarriers 6 through 11 | Mapping rule is specified in TS36.211 [8] sub clause 10.2.5.5 |
| NPDSCH | The start of NB-PDSCH transmission is >=4ms later than the end of its associated DL assignment. NPDSCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier. NPDSCH resource elements should be different from the ones used for NRS. NPDSCH resource elements should be different from the ones used for CRS. | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS 36.211 [8] sub clause 10.2.3.4 |
| NRS | Narrowband reference signals are transmitted in all NB-IoT downlink subframes in a cell supporting NPDSCH transmission. NRS is not transmitted in subframes that are not NB-IoT downlink subframes, except if these subframes contain NPBCH or NPDSCH carrying SystemInformationBlockType1-NB where NRS shall be transmitted Narrowband reference signals are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS [when no NB-IoT downlink subframes configuration has not been established]. The narrowband reference signals shall not be mapped to subframes containing NPSS or NSSS. NRS are transmitted on one or two antenna ports 0 to 1 (Figure 10.2.6.2-1 in TS 36.211 [8]). | 2 subcarriers per antenna port in any OFDM symbol where NRS is transmitted |  |

C.2	Set-up

Table C.2-1 describes the downlink Physical Channels that are required for connection set up.

Table C.2-1: Downlink Physical Channels required
for category M1 connection set-up

Physical Channel

PBCH

SSS

PSS

MPDCCH

PDSCH

Table C.2-2 describes the configuration of PDSCH and MPDCCH before measurement for FDD.

Table C.2-2: PDSCH and MPDCCH configuration for FDD

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| Allocated resource blocks |  | [6] |  |
| MCS Index |  | - | TB Size with transmitting message in 1TTI |
| Number of HARQ processes, CE Mode A | Processes | [8] |  |
| Number of HARQ processes, CE Mode B | Processes | [2] |  |
| Maximum number of HARQ transmission |  | [5] |  |
| Aggregation level | CCE | [2] | Note 4 |
| DCI Format for PDSCH |  | Format 6-1A | CE Mode A |
| DCI Format for PDSCH |  | Format 6-1B | CE mode B |
| DCI Format for PUSCH |  | Format 6-0A | CE Mode A |
| DCI Format for PUSCH |  | Format 6-0B | CE mode B |
| Note 1: [2] symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW. [3] symbols allocated to PDCCH for 5 MHz and 3 MHz. [4] symbols allocated to PDCCH for 1.4 MHz. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [8]. Note 3: Void. Note 4: For MPDCCH using SI-RNTI, Aggregation level: a)Tables C.3.0-3, C.3.1-3, and C.3.2-3 for RF tests b)Table A.2.1-1 of 36.521-3 for RRM tests. |  |  |  |

Table C.2-3: Downlink Physical Channels required for connection set-up, category NB1 and NB2

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| NPBCH | NPBCH_RA = 0 dB |  |
|  | NPBCH_RB = 0 dB |  |
| NPSS | NPSS_RA = 0 dB |  |
| NSSS | NSSS_RA = 0 dB |  |
| NPDCCH | NPDCCH_RA = 0 dB |  |
|  | NPDCCH_RB = 0 dB |  |
| NPDSCH | NPDSCH_RA = 0 dB |  |
|  | NPDSCH_RB = 0 dB |  |
| Note 1: No boosting is applied. |  |  |

Table C.2-4 describes the configuration of NPDSCH and NPDCCH before measurement for category NB1 and NB2.

Table C.2-4: NPDSCH and NPDCCH configuration for category NB1 and NB2

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| Allocated subcarriers |  | [FFS] |  |
| MCS Index |  | - | TB Size with transmitting message in 1TTI |
| Number of HARQ processes | Processes | 1 |  |
| Maximum number of HARQ transmission |  | 1 |  |
| Aggregation level | NCCE | 2 |  |
| DCI Format for NPDSCH |  | Format N1 |  |
| DCI Format for NPUSCH |  | Format N0 |  |
| Note 1: Reference signal NRS, Synchronization signals NPSS, NSSS and NPBCH allocated as per TS 36.211 [8] |  |  |  |

C.3	Connection

The following clauses describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

C.3.0	Measurement of Transmitter Characteristics

Table C.3.0-1 is applicable for measurements on the Transmitter Characteristics (clause 6).

Table C.3.0-1: Downlink Physical Channels transmitted for category M1 during a connection (FDD)

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = 0 dB |  |
|  | PBCH_RB = 0 dB |  |
| PSS | PSS_RA = 0 dB |  |
| SSS | SSS_RA = 0 dB |  |
| MPDCCH | MPDCCH_RA = 0 dB | Only required for UE category M1 testing |
|  | MPDCCH_RB = 0 dB |  |
| PDSCH | PDSCH_RA = 0 dB |  |
|  | PDSCH_RB = 0 dB |  |

NOTE 1:	No boosting is applied.

Table C.3.0-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Transmitted power spectral density | dBm/15 kHz | Test specific | 1. shall be kept constant throughout all OFDM symbols |
| Cell-specific reference signal power ratio |  | 0 dB |  |

Table C.3.0-3: PDCCH Aggregation Level (in CCE-s)

| Bandwidth | DCI for DL (SI-RNTI) | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) | Notes |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 4 | 1 | 1 | Note 1 |

Table C.3.0-3a: MPDCCH Aggregation Level (in ECCE-s)

| Bandwidth | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) |
| --- | --- | --- |
| 1.4 MHz | 2 | 4 |

C.3.1	Measurement of Receiver Characteristics

Unless otherwise stated, Table B.3.1-1 is applicable for measurements on the Receiver Characteristics (clause 7).

Table B.3.1-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | EPRE Ratio |
| --- | --- |
| PBCH | PBCH_RA = 0 dB |
|  | PBCH_RB = 0 dB |
| PSS | PSS_RA = 0 dB |
| SSS | SSS_RA = 0 dB |
| PDSCH | PDSCH_RA = 0 dB |
|  | PDSCH_RB = 0 dB |
| OCNG | OCNG_RA = 0 dB |
|  | OCNG_RB = 0 dB |

NOTE 1:	No boosting is applied.

Table B.3.1-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Transmitted power spectral density Ior | dBm/15 kHz | Test specific | 1. Ior shall be kept constant throughout all OFDM symbols |
| Cell-specific reference signal power ratio ERS/Ior |  | 0 dB |  |

C.3.2	Measurement of Performance requirements

Table C.3.2-1 is applicable for measurements in which uniform RS-to-EPRE boosting for all downlink physical channels, unless otherwise stated.

Table C.3.2-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = A +  |  |
|  | PBCH_RB = B +  |  |
| PSS | PSS_RA = 0 (Note 3) |  |
| SSS | SSS_RA = 0 (Note 3) |  |
| MPDCCH | MPDCCH_RA = A+δ | Only required for UE category M1 testing |
|  | MPDCCH_RB = B+δ |  |
| PDSCH | PDSCH_RA = A |  |
|  | PDSCH_RB = B |  |
| OCNG | OCNG_RA = A +  |  |
|  | OCNG_RB = B +  |  |

NOTE 1:	A= B = 0 dB means no RS boosting.

NOTE 2:	OCNG are not defined downlink physical channels in [4].

NOTE 3:	Assuming PSS and SSS transmitted on a single antenna port.

NOTE 4: A, B, , and δ are test specific.

Table C.3.2-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Total transmitted power spectral density | dBm/15 kHz | Test specific | 1. shall be kept constant throughout all OFDM symbols |
| Cell-specific reference signal power ratio |  | Test specific | 1. Applies for antenna port p |
| Energy per resource element EPRE |  | Test specific | 1. The complex-valued symbols and defined in TS 36.211 [3] shall conform to the given EPRE value. |

Table C.3.2-3: PDCCH Aggregation Level (in CCE-s) for PDSCH demodulation and PMI performance tests

| Bandwidth | DCI for DL (SI-RNTI) | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) | Notes |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 4 | 4 | 2 | Note 1 |
| Note 1: No DL data allocated on subframe 5 |  |  |  |  |

Table C.3.2-3a: MPDCCH Aggregation Level (in ECCE-s) for PDSCH demodulation tests

| Bandwidth | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) | Notes |
| --- | --- | --- | --- |
| 1.4 MHz | 16 | 16 | Note 1, 2 |
| Note 1: No DL data allocated on subframe 5 Note 2: For using 2PRB for MPDCCH, aggregation level 2 and 4 are used to transmit DCI for DL(C-RNTI) and UL(C-RNTI), respectively. |  |  |  |

Table C.3.2-4: PDCCH Aggregation Level for CQI and RI performance tests (in CCE-s)

| Bandwidth | DCI for DL (SI-RNTI) | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) | Notes |
| --- | --- | --- | --- | --- |
| 1.4 MHz |  |  |  |  |
| Note 1: 3 symbols allocated to PDCCH Note 2: No DL data allocated on subframes 0 and 5 for FDD |  |  |  |  |

Table C.3.2-4a: MPDCCH Aggregation Level for CQI tests (in ECCE-s)

| Bandwidth | DCI for DL (C-RNTI) | DCI for UL (C-RNTI) | Notes |
| --- | --- | --- | --- |
| 1.4 MHz |  |  |  |
| Note 1: No DL data allocated on subframes 0 and 5 for FDD Note 2: For using 2PRB for MPDCCH, aggregation level 2 and 4 are used to transmit DCI for DL(C-RNTI) and UL(C-RNTI), respectively. |  |  |  |

C.3.3	Measurement of Receiver Characteristics for Narrowband IoT

For the performance requirements for Narrowband IoT, the power allocation for the physical channels is listed in Table B.3.3-1

Table C.3.3-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | EPRE Ratio for one NRS antenna port | EPRE Ratio for two NRS antenna ports |
| --- | --- | --- |
| NPBCH | 0 dB | -3 dB |
| NPDCCH | 0 dB | -3 dB |
| NPDSCH | 0 dB | -3 dB |
| NPSS | 0 dB | 0 dB |
| NSSS | 0 dB | 0 dB |

NOTE 1:	Assuming NPSS and NSSS transmitted on one NRS antenna port.

Table C.3.3-2: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value | Note |
| --- | --- | --- | --- |
| Transmitted power spectral density Ior | dBm/15 kHz | Test specific | Ior shall be kept constant throughout all OFDM symbols |
| Narrowband reference signal power ratio ECRS/Ior |  | 0 dB | Applicable for Stand-alone operation |
| Narrowband reference signal power over cell-specific reference signal power ENRS/ERS |  | 0 dB | Applicable for In-band operation |

<< END OF CHANGES >>