3GPP TSG-RAN5 Meeting #100	R5-234842r1

# Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2406 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum requirement of AMPR for CA_n77(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test of AMPR for UL CA_n77(2A) is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding minimum requirement of AMPR for UL CA_n77(2A). No test details are needed as there is not power backoff requirement is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.0.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Correcting the Word file name by removing invalid characters |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.3.0.2	UE additional maximum output power reduction for Intra-band non-contiguous CA

6.2A.3.0.1.2.0	General

Table 6.2A.3.1.2-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NC_NS value. The CA_NC_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band non-contiguous CA bands with NS_xy indicated or configured in multiple uplink serving cells, except CA_NC_NS_01 that indicates the general emission requirements for intra-band non-contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.2-2.  For any NR CA band not listed in Table 6.2A.3.0.1.2-2 the network signalling label CA_NC_NS_01 applies.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_55 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |

For UEs configured with intra-band non-contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3700-3980 MHz and NS_01 or NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NC_NS_01.

Table 6.2A.3.1.2-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NC_NS_01 | CA_NC_NS_04 |  |  |  |  |  |  |
| CA_n77 | CA_NC_NS_01 | CA_NC_NS_55 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2A.3.1.2.1	AMPR for CA_NC_NS_04 (CA_n41(2A))

For intra-band non-contiguous CA_n41(2A) and it receives IE CA_NC_NS_04, the UE determins the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified into 2 types: AMPR to meet -25dBm/MHz and -13dBm/MHz. The A-MPR defined in this clause is used instead of MPR defined in 6.2A.2.2, not additively, so CA MPR=0 when CA_NC_NS_04 is signaled.

The UE determins the AMPR type as follows:

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

A-MPRIM3 defined in Clause 6.2A.3.1.2.1.2

Else

A-MPRIM3 defined in Clause 6.2A.3.1.2.1.1

where

-	LCRB1 is for CC1 which is the component carrier with lower frequency

-	LCRB2 is for CC2 which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.2.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.2.

6.2A.3.1.2.1.1	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band non-contiguous CA_n41(2A) power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

AMPRIM3=MAWhere MA is defined as follows

MA = 	12; 	0 ≤ B < 1.08

12; 	1.08 ≤ B < 2.16

11; 	2.16 ≤ B < 3.24

10;       3.24 ≤ B < 5.04

9; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.38

7;        16.38 ≤ B < 21.78

6; 	      21.78 ≤ B

6.2A.3.1.2.1.2	AMPRIM3 to meet -13dBm/MHz

AMPR in this clause is for intra-band non-contiguous CA_n41(2A) power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

AMPRIM3=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

6.2A.3.0.3	UE additional maximum output power reduction for inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

Unless specified in Table 6.2A.3.0.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.0.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

< End of Changes >