3GPP TSG-RAN5 Meeting #100	R5-234852r1

# Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2889 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default message for unified TCI state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure of unified TCI state is specified under NR feMIMO WI. The relevant messages need to be introduced to 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding default messages for unified TCI state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message configuration is missing for unified TCI state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Removing IEs without default value according to R5-233931 |  |  |  |  |  |  |  |  |

<Start of Changes>

4.6.3	Radio resource control information elements

–	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL, RF AND Scell_Add |
| pusch-Config | Not present |  | RESUME |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
|  | ConfiguredGrantConfig |  | CG_Config |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| sl-PUCCH-Config-r16 | Not present |  |  |
| sl-PUCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| cp-ExtensionC2-r16 | Not present |  |  |
| cp-ExtensionC3-r16 | Not present |  |  |
| useInterlacePUCCH-PUSCH-r16 | Not present |  |  |
| pucch-ConfigurationList-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 CHOICE { |  |  | SharedSpectrum |
| setup | LBT-FailureRecoveryConfig |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r16 | Not present |  |  |
| configuredGrantConfigToReleaseList-r16 | Not present |  |  |
| configuredGrantConfigType2DeactivationStateList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add Scell |
| SharedSpectrum | Operation with shared spectrum channel access |
| CG_Config | Configured Grant Configuration |

–	ControlResourceSet

Table 4.6.3-28: ControlResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 24 RBs of the BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList | Not present |  |  |
| tci-StatesPDCCH-ToReleaseList | Not present |  |  |
| tci-PresentInDCI | Not present |  |  |
| pdcch-DMRS-ScramblingID | Not present |  |  |
| rb-Offset-r16 | Not present |  |  |
| tci-PresentDCI-1-2-r16 | Not present |  |  |
| coresetPoolIndex-r16 | Not present |  |  |
| controlResourceSetId-v1610 | Not present |  |  |
| } |  |  |  |

–	CSI-SSB-ResourceSet

Table 4.6.3-47: CSI-SSB-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetId | CSI-SSB-ResourceSetId |  |  |
| csi-SSB-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index { | 1 entry |  |  |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

–	PDSCH-Config

Table 4.6.3-100: PDSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB | Not present |  |  |
| tci-StatesToAddModList | Not present |  | MBS_Multicast |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] | TCI-State | entry 1 |  |
| } |  |  |  |
| tci-StatesToReleaseList | Not present |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
|  | resourceAllocationType0 |  | Used_for_Type0 |
| pdsch-TimeDomainAllocationList | Not present |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| rateMatchPatternGroup1 | Not present |  |  |
| rateMatchPatternGroup2 | Not present |  |  |
| rbg-Size | config1 |  |  |
| mcs-Table | Not present | qam64 per default |  |
| maxNrofCodeWordsScheduledByDCI | Not present |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  |  |
| } |  |  |  |
| } |  |  |  |
| zp-CSI-RS-ResourceToAddModList | Not present |  |  |
| zp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToAddModList | Not present |  |  |
| sp-ZP-CSI-RS-ResourceSetsToReleaseList | Not present |  |  |
| p-ZP-CSI-RS-ResourceSet | Not present |  |  |
| maxMIMO-Layers-r16 | Not present |  |  |
| minimumSchedulingOffsetK0-r16 | Not present |  |  |
| antennaPortsFieldPresenceDCI-1-2-r16 | Not present |  |  |
| aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16 | Not present |  |  |
| aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 | Not present |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 | Not present |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 | Not present |  |  |
| dmrs-SequenceInitializationDCI-1-2-r16 | Not present |  |  |
| harq-ProcessNumberSizeDCI-1-2-r16 | Not present |  |  |
| mcs-TableDCI-1-2-r16 | Not present |  |  |
| numberOfBitsForRV-DCI-1-2-r16 | Not present |  |  |
| pdsch-TimeDomainAllocationListDCI-1-2-r16 | Not present |  |  |
| prb-BundlingTypeDCI-1-2-r16 | Not present |  |  |
| priorityIndicatorDCI-1-2-r16 | Not present |  |  |
| rateMatchPatternGroup1DCI-1-2-r16 | Not present |  |  |
| rateMatchPatternGroup2DCI-1-2-r16 | Not present |  |  |
| resourceAllocationType1GranularityDCI-1-2-r16 | Not present |  |  |
| vrb-ToPRB-InterleaverDCI-1-2-r16 | Not present |  |  |
| referenceOfSLIVDCI-1-2-r16 | Not present |  |  |
| resourceAllocationDCI-1-2-r16 | Not present |  |  |
| priorityIndicatorDCI-1-1-r16 | Not present |  |  |
| dataScramblingIdentityPDSCH2-r16 | Not present |  |  |
| pdsch-TimeDomainAllocationList-r16 | Not present |  |  |
| repetitionSchemeConfig-r16 | Not present |  |  |
| repetitionSchemeConfig-v1630 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Used_for_Type0 | Used for RF performance test cases |
| MBS_Multicast | Used for MBS Multicast reception |

–	ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | BWP-DownlinkDedicated with condition Scell_Add |  | Scell_Add |
|  | BWP-DownlinkDedicated with condition MBS_Multicast |  | MBS_Multicast |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | DLBWP_add |
| BWP-Downlink[1] | BWP-Downlink |  |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  | ULBWP_add |
| BWP-Uplink[1] | BWP-Uplink |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |
| Scell_Add | Add SCell |
| MBS_Multicast | Used for MBS Multicast reception |
| DLBWP_add | A dedicated DL BWP is configured |
| ULBWP_add | A dedicated UL BWP is configured |

–	TCI-State

Table 4.6.3-190: TCI-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { |  |  |  |
| tci-StateId | TCI-StateId |  |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-Id | Not present |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB-Index |  |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| qcl-Type2 | Not present |  |  |
| } |  |  |  |

–	TCI-UL-State

Table 4.6.3-191A: TCI-UL-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-UL-State-r17 ::= SEQUENCE { |  |  |  |
| tci-UL-StateId-r17 | TCI-UL-StateId |  |  |
| servingCellId-r17 | Not present |  |  |
| bwp-Id-r17 | Not present |  |  |
| referenceSignal-r17 CHOICE { |  |  |  |
| ssb-Index-r17 | SSB-Index |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| ul-powerControl-r17 | Not present |  |  |
| pathlossReferenceRS-Id-r17 |  |  |  |
| } |  |  |  |

–	TCI-UL-StateId

Table 4.6.3-191B: TCI-UL-StateId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-UL-State-Id-r17 | 0 |  |  |

–	Uplink-PowerControl

Table 4.6.3-202A: Uplink-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Uplink-PowerControl-r17 ::= SEQUENCE { |  |  |  |
| ul-powercontrolId-r17 | 1 |  |  |
| p0AlphaSetforPUSCH-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | alpha08 |  |  |
| closedLoopIndex-17 | i0 |  |  |
| } |  |  |  |
| p0AlphaSetforPUCCH-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | Not present |  |  |
| closedLoopIndex-17 | i0 |  |  |
| } |  |  |  |
| p0AlphaSetforSRS-r17 SEQUENCE { |  |  |  |
| p0-r17 | 0 |  |  |
| alpha-r17 | alpha08 |  |  |
| closedLoopIndex-17 | i0 |  |  |
| } |  |  |  |
| } |  |  |  |

< End of Changes >