3GPP TSG-RAN5 Meeting #100 	R5-234901r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5468 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of DL RMC for Band 46 BW 10 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rx test cases, CA_46C is tested with BW 10 MHz + BW 20 MHz and BW 20 MHz + BW 10 MHz. However, there is no definition of DL RMC for Band 46 BW 10 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DL RMC definition for Band 46 BW 10 MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_46C will not be tested with BW 10 MHz + BW 20 MHz and BW 20 MHz + BW 10 MHz in Rx test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1, A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2311492 R4-2313447 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Tdoc number of related RAN4 CR was corrected. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

A.3.1	General

The number of available channel bits varies across the sub-frames due to PBCH and PSS/SSS overhead. The payload size per sub-frame is varied in order to keep the code rate constant throughout a frame.

No user data is scheduled on subframes #5 in order to facilitate the transmission of system information blocks (SIB).

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a) A is a valid TB size (according to TS 36.213 [10] clause 7.1.7) assuming an allocation of NRB resource blocks

b) C is the number of Code Blocks calculated according to section 5.1.2 of TS 36.212 [9].

3.	If there is more than one A that minimizes the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

4.	For TDD, the measurement channel is based on DL/UL configuration ratio of 2DL+DwPTS (12 OFDM symbol): 2UL.

A.3.1.1	Overview of DL reference measurement channels

In Table A.3.1.1-1 are listed the DL reference measurement channels specified in annexes A.3.2 to A.3.17 of this release of TS 36.521-1. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for testing are annexes A.3.2 to A.3.17 as appropriate.

Table A.3.1.1-1: Overview of DL reference measurement channels

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |
| TDD, Receiver requirements |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-2 |  | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 5 | QPSK | 1/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 15 | QPSK | 1/3 | 75 |  | ≥ 1 |  |
| TDD | Table A.3.2-2 |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.2-2a |  | 1.4 | QPSK | 1/3 | 6 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 3 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 5 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 10 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 15 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2a |  | 20 | QPSK | 1/3 | 14 |  | 0 |  |
| TDD | Table A.3.2-2b |  | 1.4 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2b |  | 3 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2b |  | 5 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2b |  | 10 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2b |  | 15 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2b |  | 20 | QPSK | 1/3 | 4 |  | M1 |  |
| TDD | Table A.3.2-2c |  | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.2-2c |  | 20 | QPSK | 1/3 | 100 |  | ≥ 1 |  |
| TDD | Table A.3.2-2d |  | 1.4 | QPSK | 1/3 | 4 |  | M2 |  |
| TDD | Table A.3.2-2d |  | 3 | QPSK | 1/3 | 8 |  | M2 |  |
| TDD | Table A.3.2-2d |  | 5 | QPSK | 1/3 | 16 |  | M2 |  |
| TDD | Table A.3.2-2d |  | 10 | QPSK | 1/3 | 16 |  | M2 |  |
| TDD | Table A.3.2-2d |  | 15 | QPSK | 1/3 | 16 |  | M2 |  |
| TDD | Table A.3.2-2d |  | 20 | QPSK | 1/3 | 16 |  | M2 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |
| TDD, Receiver requirements, Maximum input level for UE Categories 3-5 |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.2-4 |  | 1.4 | 64QAM | 3/4 | 6 |  | - |  |
| TDD | Table A.3.2-4 |  | 3 | 64QAM | 3/4 | 15 |  | - |  |
| TDD | Table A.3.2-4 |  | 5 | 64QAM | 3/4 | 25 |  | - |  |
| TDD | Table A.3.2-4 |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4 |  | 15 | 64QAM | 3/4 | 75 |  | - |  |
| TDD | Table A.3.2-4 |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| TDD | Table A.3.2-4d |  | 10 | 64QAM | 3/4 | 50 |  | - |  |
| TDD | Table A.3.2-4d |  | 20 | 64QAM | 3/4 | 100 |  | - |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |

A.3.2	Reference measurement channel for receiver characteristics

Unless otherwise stated, Tables A.3.2-1, A.3.2-1a, A.3.2-1b, A.3.2-2, A.3.2-2a and A.3.2-2b  are applicable for measurements on the Receiver Characteristics (clause 7) with the exception of subclause 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2-3, A.3.2-3a, A.3.2-3b, A.3.2-4, A.3.2-4a and A.3.2-4b are applicable for subclause 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2-1, A.3.2-1a, A.3.2-1b, A.3.2-2, A.3.2-2a and A.3.2-2b also apply for the modulated interferer used in Clauses 7.5, 7.6 and 7.8 with test specific bandwidths.

For transmissions in TDD Band 46, Table A.3.2-2c is applicable for measurements of Receiver Characteristics (clause 7) except for the Maximum Input Level (clause 7.4A) for which Table A.3.2-4d and Table A.3.2-7 apply. For these measurements, the discovery signals measurement timing configuration (DMTC) periodicity shall be set at dmtc-Periodicity = 40 ms with an offset dmtc-Offset = 0 for the channel and the DRS shall be transmitted in the first subframe of each DMTC occasion. Furthermore, no PBCH is transmitted and the PDSCH is also scheduled in subframe #5.

<<Unchanged tables skipped>>

Table A.3.2-2c Fixed Reference Channel for Receiver Requirements (TDD Band 46)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 |
| Allocated resource blocks |  | 50 | 100 |
| Uplink-Downlink Configuration |  | N/A | N/A |
| Subcarriers per resource block |  | 12 | 12 |
| Allocated subframes per Radio Frame (D) |  | 8 | 8 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of HARQ Processes | Processes | N/A | N/A |
| Maximum number of HARQ transmissions |  | N/A | N/A |
| Information Bit Payload per Sub-Frame |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 | Bits | 4392 | 8760 |
| For Sub-Frame 1,2 | Bits | N/A | N/A |
| For Sub-Frame 0,5 | Bits | 4392 | 8760 |
| Transport block CRC | Bits | 24 | 24 |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 | Bits | 1 | 2 |
| For Sub-Frame 1,2 | Bits | N/A | N/A |
| For Sub-Frame 0,5 | Bits | 1 | 2 |
| Binary Channel Bits Per Sub-Frame |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 | Bits | 13800 | 27600 |
| For Sub-Frame 1,2 | Bits | N/A | N/A |
| For Sub-Frame 0,5 | Bits | 13512 | 27312 |
| Max. Throughput averaged over 1 frame | kbps | 3513.6 | 7008 |
| UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal and Synchronization signals allocated as per TS 36.211 [8]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

<<Unchanged tables skipped>>

Table A.3.2-4d: Fixed Reference Channel for Maximum input level for UE Categories ≥ 3 (TDD Band 46)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 |
| Allocated resource blocks |  | 50 | 100 |
| Uplink-Downlink Configuration |  | N/A | N/A |
| Subcarriers per resource block |  | 12 | 12 |
| Allocated subframes per Radio Frame (D) |  | 8 | 8 |
| Modulation |  | 64QAM | 64QAM |
| Target Coding Rate |  | 3/4 | 3/4 |
| Number of HARQ Processes | Processes | N/A | N/A |
| Maximum number of HARQ transmissions |  | N/A | N/A |
| Information Bit Payload per Sub-Frame |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 | Bits | 30576 | 61664 |
| For Sub-Frame 1,2 | Bits | N/A | N/A |
| For Sub-Frame 0,5 | Bits | 30576 | 61664 |
| Transport block CRC | Bits | 24 | 24 |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 |  | 5 | 11 |
| For Sub-Frame 1,2 |  | N/A | N/A |
| For Sub-Frame 0,5 |  | 5 | 11 |
| Binary Channel Bits Per Sub-Frame |  |  |  |
| For Sub-Frames 3,4,6,7,8,9 | Bits | 41400 | 82800 |
| For Sub-Frame 1,2 | Bits | N/A | N/A |
| For Sub-Frame 0,5 | Bits | 40536 | 81936 |
| Max. Throughput averaged over 1 frame | kbps | 24460.8 | 49331.2 |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz. Note 2: Reference signal, Synchronization signals allocated as per TS 36.211 [8]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

<<End of change>>