3GPP TSG-RAN5 Meeting # 100	R5-234937r1

# Tolouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2897 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test frequencies for band n18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for band n18 are undefined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for band n18 are defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing frequencies for band n18 will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.18 (New), 4.3.1.1.19 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Remove unnecessary text added just before Table 4.3.1.1.1.18-1 |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.15 to 4.3.1.1.1.17	FFS

###### 4.3.1.1.1.18 	Reference test frequencies for NR operating band n18

Table 4.3.1.1.1.18-1: Test frequencies for NR operating band n18 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 862.5 | 172500 | 860.25 | 172050 | 0 | 15 | 2157 | 172590 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 846.89 | 169378 | 102 |  | 2168 | 173530 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 872.5 | 174500 | 779.53 | 155906 | 504 |  | 2182 | 174530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 817.5 | 163500 | 815.25 | 163050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 729.53 | 145906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 827.5 | 165500 | 824.17 | 164834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 865 | 173000 | 860.32 | 172064 | 0 | 15 | 2158 | 172610 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 867.5 | 173500 | 844.46 | 168892 | 102 |  | 2162 | 173050 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 870 | 174000 | 774.6 | 154920 | 504 |  | 2169 | 173550 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 820 | 164000 | 815.32 | 163064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 727.1 | 145420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 819.24 | 163848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 867.5 | 173500 | 860.39 | 172078 | 0 | 15 | 2156 | 172570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.39 | 163078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.18-2: Test frequencies for NR operating band n18 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 865 | 173000 | 860.68 | 172136 | 0 | 15 | 2164 | 173090 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 867.5 | 173500 | 826.46 | 165292 | 102 |  | 2168 | 173530 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 870 | 174000 | 684.24 | 136848 | 504 |  | 2175 | 174030 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 820 | 164000 | 815.68 | 163136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 822.5 | 164500 | 636.74 | 127348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 825 | 165000 | 818.52 | 163704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 867.5 | 173500 | 860.66 | 172132 | 0 | 15 | 2162 | 173050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 822.5 | 164500 | 815.66 | 163132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  | - | - | - | - | - | - |
|  |  |  | High |  |  |  |  |  |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.19	FFS

###### 4.3.1.1.1.20	Reference test frequencies for NR operating band n20

## <<< END OF CHANGES >>>