3GPP TSG-RAN5 Meeting #100	R5-235006r2

# Toulouse, France,21st -25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3976 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new enhanced SNPN test case 6.5.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New enhanced SNPN test case for ‘SNPN / cell reselection / SNPN to PLMN’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new enhanced SNPN test case for ‘SNPN / cell reselection / SNPN to PLMN’ Revision 1: Change bars enabled for new test. Revision 2: Editorial corrections. Renumbering of steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not interoperate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.9 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 6.5.2.6.3.3-7: REGISTRATION ACCEPT (Step 35, Table 6.5.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CAG information list |  |  |  |
| Length of CAG information list contents | ‘0000 0000 0000 0000’B |  |  |
| Entry 1 | Not present |  |  |

#### <<< Start of change>>>>

#### 6.5.3.9	SNPN / cell reselection / SNPN to PLMN

6.5.3.9.1	Test Purpose (TP)

(1)

with { UE in SNPN access mode is in limited service state and supports accessing a PLMN }

ensure that {

when { UE needs to make an emergency call and there is no available SNPN supporting emergency services }

then {  UE stops operating in SNPN access mode and attempts to camp on a cell of a PLMN  }

}

6.5.3.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122, clause 5.2.8. Unless otherwise stated these are Rel-17 requirements.

[TS 23.122, clause 3.5]

There are a number of situations in which the MS is unable to obtain normal service from a PLMN or SNPN. These include:

a)	Failure to find a suitable cell of the selected PLMN or of the selected SNPN;

…

When in the limited service state, with the exception of performing an initial registration for emergency services, no LR requests are made until a valid entry of the "list of subscriber data" is present and either a suitable cell is found or a manual network reselection is performed. In the limited service state, the presence of the MS need not be known to the SNPN on whose cell it has camped. If the MS needs to make an emergency call, the MS supports accessing a PLMN, and there is no available SNPN supporting emergency services, the MS shall stop operating in SNPN access mode and attempt to camp on a cell of a PLMN so that emergency calls can be made. After an emergency call is released, the MS may re-start operating in SNPN access mode and perform SNPN selection.

There are also other conditions under which only emergency calls or access to RLOS may be made if the MS does not operate in SNPN access mode. These are shown in table 2 in clause 5. ProSe communications can be initiated in case of PLMN if necessary (see 3GPP TS 24.334 [51] or 3GPP TS 24.554 [80]) when in the limited service state due to items a) or c) or f). V2X communication over PC5 can be initiated if necessary (see 3GPP TS 24.386 [59] or 3GPP TS 24.587 [75]) when in the limited service state due to items a) or c) or f).

6.5.3.9.3	Test description

6.5.3.9.3.1	Pre-test conditions

System Simulator:

-	1 SNPN cell NR Cell 1, is configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-27 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-     NR Cell 3, does not broadcast any SNPN ID.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is [not] provisioned with a “list of subscriber data” to allow access to SNPN identified by NR cell 1 only.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A).

6.5.3.9.3.2	Test procedure sequence

Table 6.5.3.9.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1” & “T2”, is applied at the points indicated in the Main behaviour description in Table 6.5.3.9.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.3.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -78 |  |

Table 6.5.3.9.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | “Off” |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |

Table 6.5.3.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.3.9.3.2-1/2. | - | - | - | - |
| 2 | Power on the UE. |  |  |  |  |
| 3 | Check: Does the UE send a RRCSetupRequest on NR Cell 1 in the next 2 minutes? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 4 | SS adjusts cell levels according to row T2 of table 6.5.3.9.3.2-1/2. |  |  |  |  |
| 5 | Cause the UE to originate Emergency call. (Note 1) | - | - | - | - |
| 6 | The UE transmits an RRCSetupRequest message on NR Cell 3 with establishmentCause set to 'emergency'. | --> | NR RRC: RRCSetupRequest | 1 | P |
| 7 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 8 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message.with IE 5GS registration type set to "Emergency registration" or “Initial registration”. (Note 1) | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 9-17 | Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1 | - | - | - | - |
| 18 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 19 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message | --> | NR RRC: ULInformationTransfer 5G MM: REGISTRATION COMPLETE | - | - |
| 20 | The UE transmits an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 1 | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST |  |  |
| - | EXCEPTION: Steps 2b22a1 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| - | EXCEPTION: The behaviour in step 21a1 occurs in parallel with steps 22-26 | - | - | - | - |
| 21a1 | IF 5GS registration type is set as Initial Registration in step 8, THEN the generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| 22-26 | Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. | - | - | - | - |
| 27 | Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. | - | - | - | - |
| Note 1: This can be done by an AT/MMI command. Note 2: 5GS registration type can set to either "initial registration" according to TS 24.501 clause5.2.2.3.3 or "emergency registration" according to TS 24.501 clause 5.5.1.2. |  |  |  |  |  |

6.5.3.9.3.3	Specific message contents

Table 6.5.3.9.3.3-1: SIB1 for NR Cell 1 (preamble)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo | CellAccessRelatedInfo |  |  |
| } |  |  |  |

Table 6.5.3.9.3.3-2: CellAccessRelatedInfo (Table 6.5.3.9.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-28 with condition eNPN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| cellReservedForOtherUse | true |  |  |
| snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 { | 1 entry |  |  |
| SNPN-AccessInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| extCH-Supported-r17 | true |  |  |
| extCH-WithoutConfigAllowed-r17 | true |  |  |
| onboardingEnabled-r17 | true |  |  |
| imsEmergencySupportForSNPN-r17 | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |