3GPP TSG-RAN5 Meeting #100	R5-235078r2

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5248 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to legacy NB-IoT cases for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Qualcomm Incorporated, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 36.304 Cl. 5.3.1, cellBarred IE will be ignored in case of NTN. Therefore the TP5 and TP6 are not applicable to IoT NTN UE. 5.3.1 Cell status and cell reservations Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-BR message or SystemInformationBlockType1-NB message) TS 36.331 [3] by means of the following fields: - cellBarred (IE type: "barred" or "not barred") This field indicates if the cell is barred for connectivity to EPC. This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR. This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1-BR or SIB1-NB. And accoring to TS 36.331 Cl. 6.7.2, cellBarred-r13 in SIB1-NB shall always be set to barred. This conflicts with the setting in T5 and T6. Barred means the cell is barred for connectivity to NTN, as defined in TS 36.304 [4]. E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIB1-NB was specified for NTN test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN UE will fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: specifying the message content to replace exceptional handling in test procedure in TC 22.2.4. r2: R5-235111 merged and simplified the title in Table 22.2.4.3.3-6A. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.2.4	NB-IoT / Cell selection / Qrxlevmin and Qqualmin / Serving cell becomes non-suitable (S<0 or barred or Srxlev > 0 and Squal < 0)

22.2.4.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria which are not fulfilled (Srxlev < 0)}

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(2)

with { UE in RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell except the cell selection criteria which are not fulfilled (Srxlev > 0 AND Squal < 0)}

then { the UE does not consider the cell suitable and no camping on this cell can take place }

}

(3)

with { UE in RRC_IDLE state }

ensure that {

when { a cell fulfils all requirements for a suitable cell including the cell selection criteria for a cell which are also fulfilled (Srxlev > 0 AND Squal > 0)}

then { the UE considers the cell suitable and camps on it }

}

(4)

with { UE in RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (Srxlev < 0)and there is a suitable neighbour cell (Srxlev > 0) }

then { UE selects the suitable neighbour cell }

}

(5)

with { UE in RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable neighbour cell}

then { UE selects the suitable neighbour cell }

}

(6)

with { UE in RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (Srxlev > 0 and Squal < 0)and there is a suitable neighbour cell (Srxlev > 0 and Squal > 0) }

then { UE selects the suitable neighbour cell }

}

22.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in:  TS 36.300, clause 10.1.1.1 and TS 36.304, clause 4.1, 5.1.2.2, 5.2.1,  5.2.3.1, 5.2.3.2a , 5.2.4.6, 5.2.8a and 5.3. Unless otherwise stated these are Rel-13 requirements.1.

[TS 36.300, clause 10.1.1.1]

Cell selection:

-	The UE NAS identifies a selected PLMN and equivalent PLMNs;

-	The UE searches the E-UTRA frequency bands and for each carrier frequency identifies the strongest cell. It reads cell system information broadcast to identify its PLMN(s):

-	The UE may search each carrier in turn ("initial cell selection") or make use of stored information to shorten the search ("stored information cell selection").

-	The UE seeks to identify a suitable cell; if it is not able to identify a suitable cell it seeks to identify an acceptable cell. When a suitable cell is found or if only an acceptable cell is found it camps on that cell and commence the cell reselection procedure:

-	A suitable cell is one for which the measured cell attributes satisfy the cell selection criteria; the cell PLMN is the selected PLMN, registered or an equivalent PLMN; the cell is not barred or reserved and the cell is not part of a tracking area which is in the list of "forbidden tracking areas for roaming";

-	An acceptable cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred;

[TS 36.304, clause 4.1]

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

The UE will, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN [5].

[TS 36.304, clause 5.1.2.2]

The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find available PLMNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to. If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value), provided that the following high quality criterion is fulfilled:

1.	For an E-UTRAN and NB-IoT cell, the measured RSRP value shall be greater than or equal to -110 dBm.

Found PLMNs that do not satisfy the high quality criterion, but for which the UE has been able to read the PLMN identities are reported to the NAS together with the RSRP value. The quality measure reported by the UE to NAS shall be the same for each PLMN found in one cell.

...

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

[TS 36.304, clause 5.2.1]

UE shall perform measurements for cell selection and reselection purposes as specified in [10].

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on idle mode measurements and cell selection criteria.

In order to speed up the cell selection process, stored information for several RATs may be available in the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in [10].

The NAS is informed if the cell selection and reselection results in changes in the received system information relevant for NAS.

For normal service, the UE shall camp on a suitable cell, tune to that cell's control channel(s) so that the UE can:

-	Receive system information from the PLMN; and

-	receive registration area information from the PLMN, e.g., tracking area information; and

-	receive other AS and NAS Information; and

-	if registered:

-	receive paging and notification messages from the PLMN; and

-	initiate transfer to connected mode.

[TS 36.304, clause 5.2.3.1]

The UE shall use one of the following two cell selection procedures:

a)	Initial Cell Selection

This procedure requires no prior knowledge of which RF channels are E-UTRA or NB-IoT carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected.

[TS 36.304, clause 5.2.3.2a]

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – Qrxlevmin – Pcompensation - Qoffsettemp Squal = Qqualmeas – Qqualmin - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [TS 36.101]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [TS 36.101] |

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise and for NB-IoT this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.304, clause 5.2.8a]

In this state, the UE shall attempt to find a suitable cell of any PLMN to camp on and searching first for a high quality cell, as defined in subclause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state until a suitable cell is found.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-NB message) [3] by means of two fields:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN.

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	UEs assigned to Access Class 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to “reserved”.

-	UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 1:	ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN; ACs 12, 13, 14 are only valid for use in the home country [4].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is “barred” due to being unable to acquire the MasterInformationBlock (or MasterInformationBlock-NB), the SystemInformationBlockType1 (or SystemInformationBlockType1-NB), or the SystemInformationBlockType2 (or SystemInformationBlockType2-NB):

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else

-	If the cell is a CSG cell:

-	the UE may select another cell on the same frequency if the selection/reselection criteria are fulfilled.

-	else

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

22.2.4.3	Test description

22.2.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 2 on frequency f1 but have different tracking areas according to TS 36.508 Table 8.1.4.2-6. Ncell 3 on frequency f2 according to TS 36.508 Table 8.1.4.2-6.

-	System information combination 2 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 10 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (state 1) ac-NB cording to [18 TS 36.508].

22.2.4.3.2	Test procedure sequence

Table 22.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1" , "T2", "T3" , "T4", "T5" and "T6" are applied at the points indicated in the Main behaviour description in Table 22.2.4.3.2-2

Table 22.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NCell 1 | Ncell 2 | Ncell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific NRS EPRE | dBm/15kHz | -95 | "Off" | "Off" | The power level value is such to satisfy Srxlev Ncell 1 < 0 but the UE is able to read the PLMN identity |
|  | Qrxlevmin | dBm | -84 | - | - |  |
|  | Pcompensation | dB | 0 | - | - |  |
| T2 | Cell-specific NRS EPRE | dBm/15kHz | -95 | "Off" | "Off" | The power level value is such to satisfy Srxlev Ncell 1 > 0 and Squal Ncell 1 < 0 but the UE is able to read the PLMN identity |
|  | RSRQ | dB | -32 | - | - |  |
|  | Noc | dBm/15kHz | -75 | - | - |  |
|  | Qrxlevmin | dBm | -106 | - | - |  |
|  | Qqualmin | dB | -18 | - | - |  |
|  | Pcompensation | dB | 0 | - | - |  |
| T3 | Cell-specific NRS EPRE | dBm/15kHz | -65 | "Off" | "Off" | The power level is such that Srxlev Ncell 1 > 0 and Squal Ncell 1 > 0 |
|  | RSRQ | dB | -5 | - | - |  |
|  | Noc | dBm/15kHz | -75 | - | - |  |
| T4 | Cell-specific NRS EPRE | dBm/15kHz | "Off" | -85 | "Off" | SrxlevNcell1 < 0 |
|  | Srxlev* | dB | - | 25 | - | Ncell 2 becomes the strongest Ncell |
| T5 | Cell-specific NRS EPRE (FDD) | dBm/15kHz | "Off" | -85 | -91 | SrxlevNcell 2 > 0, SrxlevNcell 3 > 0 , RNcell 3 < RNcell 2 |
|  | Srxlev* (FDD) | dB | - | 25 | 19 |  |
|  | Cell-specific NRS EPRE (TDD) | dBm/15kHz | "Off" | -85 | -89 |  |
|  | Srxlev* (TDD) | dB | - | 25 | 21 |  |
|  | cellBarred (NOTE 3) | - | - | barred | notBarred | Serving Ncell becomes barred |
| T6 | Cell-specific NRS EPRE | dBm/15kHz | "Off" | -85 | -97 | SrxlevNcell 3 > 0 and SqualNcell 3 < 0 |
|  | RSRQ | dB | - | -3.28 | -26 |  |
|  | Qrxlevmin | dBm | - | -92 | -106 |  |
|  | Qqualmin | dB | - | -20 | -20 |  |
|  | Pcompensation | dB | - | 0 | 0 |  |
|  | Srxlev* | dB | - | 7 | 9 | Ncell 2 is suitable Ncell |
|  | Squal* | dB | - | 16.72 | -6 | Squal = Qqualmeas – Qqualmin |
|  | Noc | dBm/15kHz | - | "Off" | -75 |  |
|  | cellBarred (NOTE 3) |  | - | notBarred | notBarred |  |
| NOTE 1: The downlink signal level uncertainty is specified in TS 36.508 section 8.1.3.4.1 NOTE 2: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. NOTE 3: E-UTRAN always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell. |  |  |  |  |  |  |

Table 22.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS re-adjusts the cell-specific reference signal level of Ncell 1 according to row "T1" in table 22.2.4.3.2-1. | - | - | - | - |
| 0B | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 1 within the next 60 s? | --> | RRCConnectionRequest-NB | 1 | F |
| 3 | SS re-adjusts the cell-specific reference signal level of Ncell 1 level according to row "T2" in table 22.2.4.3.2-1. | - | - | - | - |
| 3A | Wait for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 4 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 1 within the next 60 s? | --> | RRCConnectionRequest-NB | 2 | F |
| 5 | SS re-adjusts the cell-specific reference signal level of Ncell 1 level according to row "T3" in table 22.2.4.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE send an RRCConnectionRequest-NB on Ncell 1? | --> | RRCConnectionRequest-NB | 3 | P |
| 7-18 | Steps 3 to 14 of the registration procedure described in TS 36.508 subclause 8.1.5.2.3 are performed on Ncell 1. | - | - | - | - |
| 18A | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRCConnectionRelease-NB | - | - |
| 19 | SS re-adjusts the cell-specific reference signal level of Ncell 1 and Ncell 2 according to row "T4" in table 22.2.4.3.2-1. | - | - | - | - |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 4 | P |
| 21 | SS changes Ncell 2 and Ncell 3 signal level and SIB1-NB IE cellBarred-r13 or cellBarred-NTN according to row "T5" in table 22.2.4.3.2-1 and transmits a Paging-NB message including systemInfoModification-r13. The systemInfoValueTag-r13 / systemInfoValueTag-r15 in the MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is increased. | <-- | Paging-NB | - | - |
| 22 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 5 | P |
| 23 | SS changes the SIB1-NB IE cellBarred-r13 or cellBarred-NTN according to row T6 in table 22.2.4.3.2-1 for Ncell 2 and waitst for 1.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 23A | SS re-adjusts the cell-specific reference signal level of Ncell 2 and Ncell 3 according to row "T6" in table 22.2.4.3.2-1. | - | - | - | - |
| 24 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 6 | P |

22.2.4.3.3	Specific message contents

Table 22.2.4.3.3-0: MasterInformationBlock-NB for Ncell 1 (all steps, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemInfoValueTag-r13 | The value is increased by 1 in step 0A, step 3, step 19. |  |  |
| } |  |  |  |

Table 22.2.4.3.3-0A: MasterInformationBlock-TDD-NB for Ncell 1 (all steps, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-TDD-NB-r15 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag-r15 | The value is increased by 1 in step 0A, step 3, step 19. |  |  |
| } |  |  |  |

Table 22.2.4.3.3-1: SystemInformationBlockType1-NB for Ncell 1 (step 0A, Table  22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| CellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -42 (-84 dBm) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-2: SystemInformationBlockType1-NB for Ncell 1 (step 3, Table  22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| CellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -53 (-106 dBm) |  |  |
| q-Qualmin-r13 | -18 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-3: SystemInformationBlockType1-NB for Ncells 1 and 2 (step 19, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -55 (-110 dBm) |  |  |
| } |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1.. maxSI-Message-NB-r13)) OF |  |  |  |
| { |  |  |  |
| SystemInfoValueTagSI-r13[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-4: SystemInformationBlockType3-NB for Ncells 1 and 2 (step 19, table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -55 (-110 dBm) |  |  |
| t-Reselection-r13 | s21 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-5: MasterInformationBlock-NB for Ncell 2 (step 21 and 23, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemInfoValueTag-r13 | The value is increased by 1 |  |  |
| } |  |  |  |

Table 22.2.4.3.3-5A: MasterInformationBlock-TDD-NB for Ncell 2 (step 21 and 23, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-TDD-NB-r15 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag-r15 | The value is increased by 1 |  |  |
| } |  |  |  |

Table 22.2.4.3.3-6: SystemInformationBlockType1-NB for Ncell 2 (step 21, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| cellBarred-r13 | barred |  |  |
| intraFreqReselection-r13 | allowed |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-6A: SystemInformationBlockType1-NB for Ncell 2 (step 21 under NTN, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 with condition NTN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| cellBarred-r13 | barred |  |  |
| intraFreqReselection-r13 | allowed |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1350 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1430 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1450 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1530 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1610 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1700 |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | barred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-7: Paging-NB (step 21, Table 22.2.4.3.2-2)

| Derivation path: 36.508 Table 8.1.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging-NB ::= SEQUENCE { |  |  |  |
| PagingRecordList-r13 | Not present |  |  |
| systemInfoModification-r13 | True |  |  |
| systemInfoModification-eDRX-r13 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 22.2.4.3.3-8: SystemInformationBlockType1-NB for Ncell 2 (step 23, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| cellBarred-r13 | notbarred |  | Ncell 2 |
| intraFreqReselection-r13 | notAllowed |  | Ncell 2 |
| } |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -46 (-92 dBm) |  | Ncell 2 |
| q-QualMin-r13 | -20 dB |  | Ncell 2 |
| } |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1.. maxSI-Message-NB-r13)) OF SystemInfoValueTagSI-r13 SEQUENCE { |  |  |  |
| SystemInfoValueTagSI-r13[1] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-8A: SystemInformationBlockType1-NB for Ncell 2 (step 23 when UE under test is NB-IoT over NTN, Table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.2-3 with condition NTN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| cellBarred-r13 | barred |  |  |
| intraFreqReselection-r13 | notAllowed |  |  |
| } |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -46 (-92 dBm) |  |  |
| q-QualMin-r13 | -20 dB |  |  |
| } |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1.. maxSI-Message-NB-r13)) OF SystemInfoValueTagSI-r13 SEQUENCE { |  |  |  |
| SystemInfoValueTagSI-r13[1] | 2 |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1350 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1430 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1450 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1530 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1610 |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1700 |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.2.4.3.3-9: SystemInformationBlockType3-NB for Ncell 2 (step 23, table 22.2.4.3.2-2)

| Derivation Path: 36.508 Table 8.1.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -46 (-92 dBm) |  | Ncell 2 |
| q-QualMin-r13 | -20 dB |  | Ncell 2 |
| t-Reselection-r13 | s21 |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]