3GPP TSG-RAN5 Meeting #100	R5-235087r2

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2912 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating PDCCH-ConfigCommon content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PEI channel is mapped to DCI format 2_7 and this configuration was missing from PDCCH-ConfigCommon. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DCI format 2_7 configuration to PDCCH-ConfigCommon. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PEI configuration will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-235087->R5-235087r1: - removed MBS configuration from PEI case - removed aggregationLevel4-r17 and aggregationLevel4-r17 as there is CORESET of 24 RBS over 2 symbols, so 48RBs all together (n2 -> 4*6*2=48) - corrected the ASN.1 structure R5-235087r1->R5-235087r2: - updated aggregation level |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

[SECTION SKIPPED]

#### –	PDCCH-ConfigCommon

Table 4.6.3-96: PDCCH-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | ControlResourceSetZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB, BWP-Id1 |
| commonControlResourceSet | ControlResourceSet |  | EN-DC |
|  | Not present |  |  |
| searchSpaceZero | SearchSpaceZero | Initial BWP |  |
|  | Not present |  | SCell_Add InitialBWP_SIB, BWP-Id1 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
|  | 3 entries |  | PEI |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| SearchSpace[2] | SearchSpace with condition SISS | entry 2 |  |
| SearchSpace[3] | SearchSpace with condition PEI | entry 3 | PEI |
| } |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  | EN-DC |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| commonSearchSpaceList | Not present |  | SCell_Add |
| searchSpaceSIB1 | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| searchSpaceOtherSystemInformation | SearchSpaceId with condition SISS |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| pagingSearchSpace | 0 |  |  |
|  | Not present |  | EN-DC, SCell_Add |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
|  | Not present |  | SCell_Add |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| commonSearchSpaceListExt-r16 | Not present |  |  |
| sdt-SearchSpace-r17 | Not present |  |  |
| sdt-SearchSpace-r17 CHOICE { |  |  | SDT |
| existingSearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |
| searchSpaceMCCH-r17 | Not present |  |  |
| searchSpaceMCCH-r17 | SearchSpaceId with condition CSS |  | MBS_Broadcast |
| searchSpaceMTCH-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 | Not present |  |  |
| commonSearchSpaceListExt2-r17 SEQUENCE(SIZE (1..4)) OF SearchSpaceExt-v1700 { | 2 entries |  | MBS_Broadcast |
|  | 3 entries |  | PEI |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  | PEI |
| searchSpaceType-r17 SEQUENCE { |  |  | MBS_Broadcast |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[2] SEQUENCE { |  | entry 2 | MBS_Broadcast, PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 | Not present |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| SearchSpaceExt-v1700[3] SEQUENCE { |  | entry 3 | PEI |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 | Not present |  |  |
| dci-Format2-7-r17 SEQUENCE { |  |  |  |
| nrofCandidates-PEI-r17 SEQUENCE { |  |  |  |
| aggregationLevel4-r17 | n2 |  |  |
| aggregationLevel8-r17 | Not present |  |  |
| aggregationLevel16-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| firstPDCCH-MonitoringOccasionOfPO-v1710 | Not present |  |  |
| pei-ConfigBWP-r17 | Not present |  |  |
| pei-ConfigBWP-r17 SEQUENCE { |  |  | PEI |
| pei-SearchSpace-r17 | SearchSpaceId with condition PEI |  |  |
| firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { |  |  |  |
| sCS15KHZoneT[1] | 0 |  |  |
| sCS15KHZoneT[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| followUnifiedTCIstate-v1720 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| SCell_Add | Add SCell |
| InitialBWP_SIB | Configured via DownlinkConfigCommonSIB |
| BWP-Id1 | Additional BWP 1 |
| SDT | For SDT test cases |
| PEI | Paging Early Indication is configured in the cell. |
| MBS_Broadcast | NR Cell provide MBS Broadcast service |