3GPP TSG-RAN5 Meeting #100	R5-235122r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3174 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-SIM: NR MUSIM Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No postamble procedures have been defined for MUSIM test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced postamble procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MUSIM test cases could not be fully implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.8(new), 10.3.9(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: made clear that the USIMs can be removed in any order in clause 10.3.8. Editorial updates. |  |  |  |  |  |  |  |  |

## 10.3	On NR/5GC

### 10.3.1	UE postamble states and procedures

In order to bring the UE to the switched/powered off state, a procedure needs to be executed, which depends on the UE state at the end of test case body. The UE postamble start states and associated procedures are shown in figure 10.3.1-1.

Figure 10.3.1-1: UE postamble states and procedures for NR/5GC

If the test body ends by releasing an emergency call whilst in limited service, the UE may be in either NR REGISTERED or in NR DEREGISTERED state as the UE behaviour is not specified. The postamble procedures shall allow for both possible start states.

Figure 10.3.1-2: UE postamble states and procedures after emergency call release in limited service

### 10.3.2	Switch / Power off procedure in State 1N-A

Table 10.3.2-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Test procedure for Switch off / Power off in RRC_IDLE as specified in TS 38.508-1 [5] clause 4.9.6.1 | - | - |

### 10.3.3	Switch / Power off procedure in State 2N-A

Table 10.3.3-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Test procedure for Switch off / Power off in RRC_INACTIVE as specified in TS 38.508-1 [5] clause 4.9.6.2 | - | - |

### 10.3.4	Switch / Power off procedure in State 3N-A

Table 10.3.4-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Test procedure for Switch off / Power off in RRC_CONNECTED as specified in TS 38.508-1 [5] clause 4.9.6.3 | - | - |

### 10.3.4A	Switch / Power off procedure in State 3N-A with T3540 started

Table 10.3.4A-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Test procedure for Switch off / Power off in RRC_CONNECTED with T3540 started as specified in TS 38.508-1 [5] clause 4.9.6.3A | - | - |

### 10.3.5	Switch / Power off procedure in NR DEREGISTERED

Table 10.3.5-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Test procedure for Switch off / Power off in State DEREGISTERED as specified in TS 38.508-1 [5] clause 4.9.6.4 | - | - |

### 10.3.6	Switch / Power off procedure after EMERGENCY CALL RELEASED in RRC_IDLE

Table 10.3.6-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported. | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (NOTE 1) | - | - |
| 1a2 | Start T_Delay=5 sec | - | - |
| - | EXCEPTION: Steps 1a3a1 to 1a3b1 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - |
| 1a3a1 | The UE transmits an RRCSetupRequest message | --> | NR RRC: RRCSetupRequest |
| 1a3a2 | Stop T_Delay | - | - |
| 1a3a3 | SS transmits an RRCSetup message | <-- | NR RRC: RRCSetup |
| 1a3a4 | The UE transmits an RRCSetupComplete message including the DEREGISTRATION REQUEST message | --> | NR RRC: RRCSetupComplete 5GMM: DEREGISTRATION REQUEST |
| 1a3a5 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |
| 1a3b1 | T_Delay expires (NOTE 2) | - | - |
| 1b1 | ELSE power off UE (NOTE 3) | - | - |
| NOTE 1: USIM removal is a feasible alternative to switch off UE. NOTE 2: The UE has performed local deregistration NOTE 3: Power off is used when UE does not support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

### 10.3.7	Switch / Power off procedure after EMERGENCY CALL RELEASED in RRC_CONNECTED

Table 10.3.7-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a DEREGISTRATION REQUEST message with Deregistration type IE set to "re-registration not required". | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION REQUEST |
| 2 | Start T_Delay=5 sec. | - | - |
| - | EXCEPTION: Steps 3a1 – 3c3 describe optional behaviour that depends on the UE implementation. | - | - |
| 3a1 | The UE transmits a DEREGISTRATION ACCEPT message. | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION ACCEPT |
| 3a2 | Stop T_Delay. | - | - |
| 3a3 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |
| 3b1 | T_Delay expires. (NOTE 1) | - | - |
| 3b2 | The SS locally releases the RRC connection. | - | - |
| 3c1 | The UE transmits a 5GMM STATUS message | --> | NR RRC: ULInformationTransfer 5GMM: 5GMM STATUS |
| 3c2 | Stop T_Delay. | - | - |
| 3c3 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease |
| 4 | Test procedure for Switch off / Power off in State DEREGISTERED as specified in TS 38.508-1 [5] clause 4.9.6.4. | - | - |
| NOTE 1: The UE has performed local deregistration, so the SS shall also perform the deregistration procedure. |  |  |  |

Table 10.3.7-2: DEREGISTRATION REQUEST (Step 1, Table 10.3.7-1)

| Derivation Path: TS 38.508-1 [5], Table 4.7.1-14. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| De-registration type |  | ‘0001’B |  | Re-registration not required |  |  |  |

### 10.3.8	MUSIM Switch / Power off procedure, both USIMs in State 1N-A

Table 10.3.8-1: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1c1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported. | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE | - | - |
| 1a2 – 1a14 | The steps specified in Table 10.3.8-2 take place on the cell on which one of the USIMs is registered | - | - |
| 1a15 | The SS configures this cell as a "Non-Suitable Off cell". | - | - |
| 1a16 – 1a29 | The steps specified in Table 10.3.8-2 take place on the cell on which the other USIM is registered | - | - |
| 1b1 | ELSE IF pc_USIM_Removal THEN remove one of the USIMs (Note 1, Note 2) | - | - |
| 1b2 – 1b14 | The steps specified in Table 10.3.8-2 take place on the cell on which the first USIM to be removed is registered | - | - |
| 1b15 | The SS configures this cell as a "Non-Suitable Off cell". | - | - |
| 1b16 | Remove the second USIM |  |  |
| 1b17 – 1b30 | The steps specified in Table 10.3.8-2 take place on the cell on which the second USIM to be removed is registered | - | - |
| 1c1 | ELSE power off UE (Note 3) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: It is up to operator which USIM is removed first. Note 3: Power off is used when UE does not support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |

Table 10.3.8-2: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | The following messages are to be observed on the same cell | - | - |
| 1 | UE transmits an RRCSetupRequest message. | --> | RRC: RRCSetupRequest |
| 2 - 13 | Steps 1a3 to 1a5 of Test procedure for Switch off / Power off in RRC_IDLE as specified in TS 38.508-1 [5] clause 4.9.6.1 | - | - |

### 10.3.9	MUSIM Switch / Power off procedure, one USIM in State 1N-A and one USIM in State 3N-A

Table 10.3.9-1: USIM parameters

| Condition | Explanation |
| --- | --- |
| USIM A | The UE is in state 3N-A on this USIM |
| USIM B | The UE is in state 1N-A on this USIM |

Table 10.3.9-2: Switch / Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1c1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported. | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE | - | - |
| 1a2 – 1a7 | Steps 5a1Aa1 to 5a3 of test procedure for Switch off / Power off in RRC_CONNECTED as specified in TS 38.508-1 [5] clause 4.9.6.3 take place on the cell on which USIM A is registered | - | - |
| 1a8 | The SS configures this cell as a "Non-Suitable Off cell". | - | - |
| 1a9 – 1a22 | Steps 1a2 to 1a5 of Test procedure for Switch off / Power off in RRC_IDLE as specified in TS 38.508-1 [5] clause 4.9.6.1 take place on the cell on which USIM B is registered | - | - |
| 1b1 | OR IF pc_USIM_Removal THEN remove USIM A (Note 1) | - | - |
| 1b2 – 1b7 | Steps 5a1Aa1 to 5a3 of test procedure for Switch off / Power off in RRC_CONNECTED as specified in TS 38.508-1 [5] clause 4.9.6.3 take place on the cell on which USIM A is registered | - | - |
| 1b8 | The SS configures this cell as a "Non-Suitable Off cell". | - | - |
| 1b9 | Remove USIM B |  |  |
| 1b10 – 1b23 | Steps 1a2 to 1a5 of Test procedure for Switch off / Power off in RRC_IDLE as specified in TS 38.508-1 [5] clause 4.9.6.1 take place on the cell on which USIM B is registered | - | - |
| 1c1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE does not support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |