3GPP TSG-RAN WG5 Meeting # 100	R5-235128r2

# Toulouse, France, August 21 – 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0525 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on notation for SUL Physical Layer Baseline Implementation Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align with the discussion on the new dual SUL CA combination notations, a WF on the notation for single CA combinations was approved in RAN4. The core spec in RAN4 has also modified the notation for SUL CA configuration. The notation for SUL Physical Layer Baseline Implementation Capabilities should be corrected accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the notation for SUL Physical Layer Baseline Implementation Capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The notation for SUL Physical Layer Baseline Implementation Capabilities will be inconsistent with core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: - CR-3GU inconsistency correction. None of the CR impact tick box for UICC apps, ME, RAN or CN shall be set in the coversheet. 2nd revision: - Correct CA_n1A_n78A-n80A to CA_n1A-n78A-n80A in Table A.4.3.2C.3-3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

A.4.3.2C	NR SUL Physical Layer Baseline Implementation Capabilities

NOTE:	See Annex B for status of completed NR SUL configurations in this version of 3GPP UE conformance test specifications.

A.4.3.2C.1	General NR SUL capabilities

Table A.4.3.2C.1-1: Uplink NR SUL capabilities (for one or more of the supported NR SULconfigurations)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UL NR SUL with 2 carriers | 38.101-1, 5.5C |  |
| 2 | UL NR SUL with 3 carriers | 38.101-1, 5.5C |  |

A.4.3.2C.2	SUL band combinations without CA

Table A.4.3.2C.2-1: Supported SUL configurations without CA

| SUL configuration / Item (Note 1) | Release | Supported | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- |
| SUL_n41A-n83A | Rel-17 |  |  |  |
| SUL_n78A-n80A | Rel-15 |  |  |  |
| SUL_n78A-n84A | Rel-15 |  |  |  |
| SUL_n79A-n83A | Rel-17 |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-1, e.g. ‘SUL_n78A-n80A’ indicates SUL operation on NR bands n78 and n80 with UL CA Bandwidth Class A on both bands. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. The ULTxSwitching is only tested with 2 UL CCs, so UE is allowed to report ‘N’ by default for SUL configuration with > 2 component carriers. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |

A.4.3.2C.3		SUL band combinations with CA

Table A.4.3.2C.3-1: Supported SUL configurations with Intra-band non-contiguous CA

| NR SUL with CA configuration / Item (Note 1) | Release | Supported | Supported SUL configuration in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| TBD | TBD |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-2. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

Table A.4.3.2C.3-2: Supported SUL configurations with Intra-band contiguous CA

| NR SUL configuration / Item (Note 1) | Release | Supported | Supported SUL configuration in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| CA_n41C-n83A | Rel-17 |  |  |  |  |
| CA_n78C-n80A | Rel-17 |  |  |  |  |
| CA_n78C-n84A | Rel-17 |  |  |  |  |
| CA_n79C-n83A | Rel-17 |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-3,. e.g. ‘CA_n41C-n83A’ indicates SUL operation on NR bands n41 and n83 with DL CA Bandwidth Class C on band n41. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

Table A.4.3.2C.3-3: Supported SUL configurations with Inter-band CA

| NR SUL configuration / Item (Note 1) | Release | Supported | Supported SUL configuration in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n80A | Rel-17 |  |  |  |  |
| CA_n1A-n78A-n84A | Rel-17 |  |  |  |  |
| CA_n3A-n78A-n80A | Rel-17 |  |  |  |  |
| CA_n28A-n41A-n83A | Rel-17 |  |  |  |  |
| CA_n28A-n79A-n83A | Rel-17 |  |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-4. e.g. ‘CA_n1A-n78A-n84A’ indicates SUL operation on NR bands n1, n78 and n84 with DL CA Bandwidth Class A on bands n1 and n78. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

<< End of changes >>