3GPP TSG-RAN5 Meeting #100	R5-235161r1

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2918 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Mid test channel bandwidths in Table 4.3.1.0A-1 for the following bands are incorrect: - Band n3: Mid channel bandwidth defined missed update when CBW=50 MHz was included through WI NR_lic_bands_BW_R17-UEConTest. - Band n48: Mid channel bandwidth calculation missed that channel bandwidth 5MHz is restricted to operation when carrier is configured as an SCell part of DC or CA configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mid test channel bandwidths in Table 4.3.1.0A-1 for the following bands are corrected: - Band n3 Band n48: if calculation were done according to note 1, the result would be not allowed asymmetric channel bandwidths for DL and UL. Hence a new note has been added to identify exception to note 1 for mid channel bandwidth Scell in CA DL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect mid test channel bandwidths will remain and affect the conformance testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Changes highlighted in yellow are proposed in different CR due to different WI code. Recommended implementation order is: - R5-235157 (R15) - R5-235158 (R16) - R5-235159 (R16) - R5-235160 (R16) - R5-235161 (R17 this CR) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

## 4.3	Reference test conditions

### 4.3.1	Test frequencies

Editor's Note: n7 (Mid: 25MHz, High: 50MHz), n8 (High: 35MHz), n25 (High: 45MHz), n40 (High: 100MHz), n65 (High: 50MHz), n66 (High: 45MHz), n71 (High: 35MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 15 | 15 |
| n8 | 20 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 20 | 15 |
| n26 | 10 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 30 | 15 |
| n41 | 60 | 15 |
| n46 | 40 | N/A |
| n48 | 2011 | 15 |
| n50 | 20 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 108 |
| n70 | 15 | 10 |
| n71 | 10 | 10 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n86 | 203 | [15] |
| n91 | 103 | [10] |
| n92 | 15 | [15] |
| n93 | 103 | [10] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closer bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n259 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closer bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NR band, NTN

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n255 | 15 |
| n256 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.1 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

///End of changes