3GPP TSG-RAN WG5 Meeting #100 	R5-235178

# Toulouse, France, 21st – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2451 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add max UE power for ULCA with NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case for ULCA with NR-U bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 6.2F.1A UE maximum output power for CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.1A (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 change – update clause title to be “UE maximum output power for CA for shared spectrum access. |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 6.2F.1A	UE maximum output power for CA for shared spectrum access

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with 1.5dB MPR has been covered in [6.2F.2].

- MU and TT for >6GHz (band n96).

- Test state and generic procedure are TBD in 38.508-1

6.2F.1A.1	Test purpose

To verify that the error of the UE maximum output power in two uplink carrier aggregation does not exceed the range

prescribed by the specified nominal maximum output power and tolerance.

6.2F.1A.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access that support NR 2UL CA.

6.2F.1A.3	Minimum conformance requirements

#### 6.2F.1A.3.1	UE maximum output power for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2F.1A.3.1-1.

Table 6.2F.1A.3.1-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46A-n48A |  |  |  |  | 23 | +2/-32 |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2F.1A.

6.2F.1A.4	Test description

6.2F.1A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1 that are restricted to shared channel access. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2F.1A.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2F.1.4A.1-1: Test Configuration Table for uplink inter-band

NOTE: No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2F.1A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2F.1A.4.3.

6.2F.1A.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2F.1A.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. Symbols with transient periods are not under test.

6.2F.1A.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

6.2F.1A.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2F.1A.5-1.

Table 6.2F.1A.5-1 UE Output Power for inter-band CA (2 UL CA) test requirements

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46A-n48A |  |  |  |  | 23 | +2+TT/-32 -TT |  |  |

Table 6.2F.1A.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 5.925GHz | 5.925GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB | TBD |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB | TBD |

<< End of changes >>