3GPP TSG-RAN5 Meeting # 100	R5-235181r1

# Toulouse, France, 21st August 2023 – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to 38.521-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Applicability of core requirements to different types of satellites is not clear. - Clarifications required for delay conditions in case of zero Doppler conditions. - Satellite orbit conditions not clear for demodulation requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Applicabiity of core requirements to different types of satellites clarified. - Clarifications added for delay conditions in case of zero Doppler conditions. - Satellite orbit conditions clarified for demodulation requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test requirements will remain misaligned with core requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.1, 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2314929. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - RAN4 dependency updated - Removed changes to frequency error test case as it will be further discussed in RAN4 - Removed changes to demod applicability |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed. Handheld power class 3 UE is assumed in Release 17 for satellite access.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

All requirements in this section, other than frequency error in clause 6.4.1, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured in Table TBD.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

All requirements in this section shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites..

All the parameters in clause 7 are defined using the UL reference measurement channels specified in 3GPP TS 38.101-1 [5] Annex A.2.2, the DL reference measurement channels specified in 3GPP TS 38.101-1 [5] Annex A.3.2 and using the set-up specified in 3GPP TS 38.101-1 [5] Annex C.3.1.

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

## 8.1	General

## All requirements in this section shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.

### 8.1.1	Relationship between minimum requirements and test requirements

## <<< END OF CHANGES 3>>>