3GPP TSG WG RAN5 Meeting #101	R5-236051

# Chicago, USA, 13 – 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Initial conditions update for multiple TX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. , Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Initial conditions requires update to implement proposal 1.2 endorsed in R5-236027. Remove editor’s notes of “Initial condition and call setup procedure support satellite access are TBD” to implement proposal 4 of R5-236027 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test procedure of TX cases and did some clean-up for editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are remained incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-236027 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.2A.1	UE maximum output power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2A.1.4	Test description

6.2A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on operating bands for satellite access that specified in sub-clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |
|  |  |  |  | HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
| 1.4MHz |  |  | QPSK | 2 |
| NOTE 1: Test Channel Bandwidth is checked separately for each E-UTRA satellite access band, the applicable channel bandwidths are specified in Table 5.3A-1. NOTE 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. NOTE 3: The RBstart of non-1RB allocation shall be RB #0 with narrowband index 0 for low and mid range, RB# (6 - RB allocation) with max narrowband index for high range test frequency. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.

Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.3.

<< End of changes >>

<<Start of changes >>

6.2A.2	UE maximum output power reduction for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2A.2.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.3.

10.	For UE supporting subPRB allocation, repeat step 1-6 with UL RMC according to Table 6.2A.2.4.1-2

<< End of changes >>

<<Start of changes >>

6.2A.3	UE additional maximum output power reduction for category M1 UE

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test Points analysis is TBD

- Addition to applicability spec is pending

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

6.2A.3.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.3.4.1-1 to Table 6.2A.3.4.1-2.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.3.4.3.

<< End of changes >>

<<Start of changes >>

6.2A.4	Configured transmitted Power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2A.4.4	Test description

6.2A.4.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, in Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to TS 36.521-1[14] Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.4.1-1

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.4.3.

<< End of changes >>

<<Start of changes >>

6.2B.1	UE maximum output power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2B.1.4	Test description

6.2B.1.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521 Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.1.4.3.

<< End of changes >>

<<Start of changes >>

6.2B.2	UE maximum output power reduction for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2B.2.4	Test description

6.2B.2.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1[14].

4.	The UL Reference Measurement channels are set according to Table 6.2B.2.4.1-1.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2.3F.4.3.

<< End of changes >>

<<Start of changes >>

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Test requirement is TBD

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.2B.3.4	Test description

6.2B.3.4.1	Initial condition

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.3.4.1-1 to Table 6.2B.3.4.1-2.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause8.1.5. Message contents are defined in clause 6.2B.3.4.3.

<< End of changes >>

<<Start of changes >>

6.2B.4	Configured transmitted Power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.2B.4.4	Test description

6.2B.4.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, in Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H of TS 36.521-1 [14].

4.	The UL Reference Measurement channel is set according to Table 6.2B.4.4.1-1

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.4.4.3.

<< End of changes >>

<<Start of changes >>

6.3A.1	UE Minimum output power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.1.4	Test description

6.3A.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3A.2	Transmit OFF power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

<< End of changes >>

<<Start of changes >>

6.3A.3.1	General ON/OFF time mask for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.3.1.4	Test description

6.3A.3.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.3.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.3.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

<< End of changes >>

<<Start of changes >>

6.3A.3.2.1	PRACH time mask for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.3.2.1.4	Test description

6.3A.3.2.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2. Message contents are defined in clause 6.3A.3.2.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3A.3.2.2	SRS time mask for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.3.2.2.4	Test description

6.3A.3.2.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.3.2.2.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

<< End of changes >>

<<Start of changes >>

6.3A.4.1	Power Control Absolute power tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.4.1.4	Test description

6.3A.4.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.4.1.4.1-1.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A. 2 AA. Message contents are defined in clause 6.3A.4.1.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

<< End of changes >>

<<Start of changes >>

6.3A.4.2	Power Control Relative power tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.4.2.4	Test description

6.3A.4.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A. 2 AA. Message contents are defined in clause 6.3A.4.2.4.3.

<< End of changes >>

<<Start of changes >>

6.3A.4.3	Aggregate power control tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3A.4.3.4	Test description

6.3A.4.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.3A.4.3.4.1-1 (PUCCH sub-test) and Table 6.3A.4.3.4.1-2 (PUSCH sub-test).

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.4.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.1	UE Minimum output power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.1.4	Test description

6.3B.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.2	Transmit OFF power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

<< End of changes >>

<<Start of changes >>

6.3B.3.1	General ON/OFF time mask for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.3.1.4	Test description

This test is covered by clause 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 and 6.3B.3.2 NPRACH time mask for category NB1 and NB2.

6.3B.3.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.3.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.3.2	NPRACH time mask for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.3.2.4	Test description

6.3B.3.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.2.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.4.1	Power Control Absolute power tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.4.1.4	Test description

6.3B.4.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.1.4.1-1.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.1.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.4.2.4	Test description

6.3B.4.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.2.4.3.

<< End of changes >>

<<Start of changes >>

6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

<< End of changes >>

<<Start of changes >>

6.3B.4.3.4	Test description

6.3B.4.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C and uplink signals according to Annex H.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.3.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.3.4.3.

<< End of changes >>

<<Start of changes >>

6.4A.2.1	Error Vector Magnitude (EVM) for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4A.2.1.4	Test description

6.4A.2.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.4A.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.4A.2.1.4.3.

<< End of changes >>

<<Start of changes >>

6.4A.2.2	Carrier leakage for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4A.2.2.4	Test description

6.4A.2.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.4A.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.4A.2.1.4.3.

<< End of changes >>

<<Start of changes >>

6.4A.2.3	In-band emissions for non allocated RB for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4A.2.3.4	Test description

6.4A.2.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to in Table 6.4A.2.3.4.1-1 (PUSCH sub-test) and Table 6.4A.2.3.4.1-2 (PUCCH sub-test).

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.4A.2.3.4.3

<< End of changes >>

<<Start of changes >>

6.4A.2.4	EVM equalizer spectrum flatness for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4A.2.4.4	Test description

6.4A.2.4.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.4A.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.4A.2.4.4.3.

<< End of changes >>

<<Start of changes >>

6.4B.2.1	Error Vector Magnitude (EVM) for Category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4B.2.1.4	Test description

6.4B.2.1.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 8.1.4.3

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.

4.	The UL Reference Measurement channels are set according to in Table 6.4B.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is State 2A-NB with CP CIoT Optimisation to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.4B.2.1.4.3.

<< End of changes >>

<<Start of changes >>

6.4B.2.2	Carrier leakage for Category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4B.2.2.4	Test description

6.4B.2.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 8.1.4.3

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.

4.	The UL Reference Measurement channels are set according to in Table 6.4B.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is State 2A-NB with CP CIoT Optimisation to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.4B.2.2.4.3.

<< End of changes >>

<<Start of changes >>

6.4B.2.3	In-band emissions for Category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.4B.2.3.4	Test description

6.4B.2.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 8.1.4.3

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to in Table 6.4B.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is State 2A-NB with CP CIoT Optimisation to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.4B.2.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5A.2	Occupied bandwidth for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.2.4	Test description

6.5A.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5A.3.2	Spectrum emission mask

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.3.2.4	Test description

6.5A.3.2.4.1	Initial conditions

<<Skip unchanged >>

- Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

- The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

- Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

- The UL Reference Measurement channels are set according to Table 6.5A.3.2.4.1-1.

- Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.

Ensure the UE is in State 3A-RF-CE according to TS 36.508[12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.3.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5A.3.4	Adjacent Channel Leakage Ratio for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.3.4.4	Test description

6.5A.3.4.4.1	Initial conditions

<<skip unchanged >>

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A3.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.3.4.4.3.

10.	For UE supporting subPRB allocation, repeat step 1-6 with UL RMC according to Table 6.5A3.4.4.1-2.

<< End of changes >>

<<Start of changes >>

6.5A.4.2	Transmitter Spurious emissions

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.4.2.4	Test description

6.5A.4.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.4.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5A.4.3	Spurious emission band UE co-existence

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.4.3.4	Test description

6.5A.4.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.4.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.4.3.4.3.

<< End of changes >>

<<Start of changes >>

6.5A.4.4	Additional spurious emissions

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5A.4.4.4	Test description

6.5A.4.4.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.4.4.4.1-1 to Table 6.5A.4.4.4.1-2a.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.4.4.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.2	Occupied bandwidth for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.2.4	Test description

6.5B.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12]Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.4.0

4.	The UL Reference Measurement channels are set according to Table 6.5B.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.3.2	Spectrum emission mask

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.3.2.4	Test description

6.5B.3.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [12] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.3.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.3.4.4	Test description

6.5B.3.4.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [12] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.3.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.3.4.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.4.2	Transmitter Spurious emissions

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.4.2.4	Test description

6.5B.4.2.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.4.2.4.1-1.

5.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

7.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

8.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.4.2.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.4.3	Spurious emission band UE co-existence

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.4.3.4	Test description

6.5B.4.3.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE to the UE antenna connectors as shown in Figure TS 36.508 [12] Annex A, Figure A.7 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.4.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.4.3.4.3.

<< End of changes >>

<<Start of changes >>

6.5B.4.4	Additional spurious emissions

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.5B.4.4.4	Test description

6.5B.4.4.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.4.4.4.1-1 to Table 6.5B.4.4.4.1-2 depending on network signal value.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5B.4.4.4.3.

<< End of changes >>

<<Start of changes >>

6.6B	Transmit intermodulation for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

6.6B.4	Test description

6.6B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.2 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.6B.4.3.

<< End of changes >>