3GPP TSG WG RAN5 Meeting #101	R5-236052

# Chicago, USA, 13 – 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Initial conditions update for multiple RX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. , Keysight Technologies UK Ltd ,CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Initial conditions requires update to implement proposal 1.2 endorsed in R5-236027. Remove editor’s notes of “Initial condition and call setup procedure support satellite access are TBD” to implement proposal 4 of R5-236027 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test procedure of RX cases and did some clean-up for editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are remained incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.4.1, 7.4A.4.1, 7.4B.4.1 7,5A.4.1, 7.5B.4.1 ,7.6A.2.4.1 7.6B.2.4.1, 7.9A.4.1, 7.9B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-236027 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

7.3B.4	Test description

7.3B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1[14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.3B.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.3B.4.3.

<< End of changes >>

<<Start of changes >>

7.4A	Maximum input level for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.4A.4	Test description

7.4A.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.4A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.4A.4.3.

<< End of changes >>

<<Start of changes >>

7.4B	Maximum input level for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.4B.4	Test description

7.4B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.4B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.4B.4.3.

<< End of changes >>

<<Start of changes >>

7.5A	Adjacent Channel Selectivity for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.5A.4	Test description

7.5A.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 7.5A.4.3.

<< End of changes >>

7.5B	Adjacent Channel Selectivity for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<<Start of changes >>

7.5B.4	Test description

7.5B.4.1	Initial conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5B.4.1-1.

5	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 7.5B.4.3.

<< End of changes >>

<<Start of changes >>

7.6A.2	In-band blocking for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.6A.2.4	Test Description

7.6A.2.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6A.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.6A.2.4.3.

<< End of changes >>

<<Start of changes >>

7.6B.2	In-band blocking for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Test Points analysis is TBD

- Test configuration is TBD

<< End of changes >>

<<Start of changes >>

7.6B.2.4	Test Description

7.6B.2.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect the SS to the UE antenna connector as shown in TS 36.508 [12] Annex A, Figure A.4 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C.0, C.1 and C.2, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channel is set according to Table 7.6B.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.6B.2.4.3.

<< End of changes >>

<<Start of changes >>

7.9A	Spurious emissions for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

7.9A.4	Test Description

7.9A.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9A.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.9A.4.3.

<< End of changes >>

<<Start of changes >>

7.9B	Spurious emissions for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

<< End of changes >>

<<Start of changes >>

7.9B.4	Test Description

7.9B.4.1	Initial Conditions

<<Skip unchanged >>

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9B.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other preconfigured means.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.9B.4.3.

<< End of changes >>