3GPP TSG-RAN5 Meeting #101	R5-236060

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

Agenda item:	5.3.9.9

Source:	Keysight Technologies

Title:	PC5 MU - Discussion on ACS Case 1 and IBB

Document for:	Discussion and Endorsement

# Introduction

The aim of this contribution is to progress in the MU analysis for Power Class 5 (PC5) devices in FR2.

Considering the agreement in [1] that all MU factors others than influence of noise will be reused from PC1 and that REFSENS is already defined in [3], this contribution will focus in the analysis for the following RX test cases:

- ACS Case 1 (Case 2 not testable)

- IBB

Noting that in those test cases, for certain bands and power classes, relaxations are required due to maximum achievable interference level, relaxations apart from MTSU will be analysed.

# Discussion

## MTSU

ACS Case 1 and IBB test cases are RX tests where influence of noise is not considered. Considering also the previously mentioned agreement in [1], all the MU factors will be reused from PC1 so resulting MTSU for these 2 tests will be exactly the same as already calculated for PC1 in FR2a (FR2c for PC5 still out of scope in RAN5. FR2b directly not applicable for PC5).

Proposal 1. For ACS Case 1 and IBB test cases, for FR2a, define MTSU for PC5 equal to the MTSU defined for PC1.

## Relaxation

In FR2a, as agreed in [4], the maximum feasible Interferer power is -52.8dBm. With such a value, testing CBW up to 100MHz, for PC3 there is no need to add any relaxation because right at 100MHz the required interferer power is -52.8dBm.

To know if any relaxation will be needed for PC5, the required interferer power for CBW = 100MHz will need to be calculated.

PC5 REFSENS for 100MHz in FR2a as defined in [5], section 7.3, is -89.6dBm.The interferer power as defined in [5], section 7.5 (Case 1) and section 7.6, for FR2a is REFSENS +35.5 dB. As a result, the required interferer power for PC5 in FR2a is -89.6dBm + 35.5dB = -54.1dBm that is smaller than the maximum feasible Interferer power of -52.8dBm. Therefore, no relaxation is required.

Proposal 2. For ACS Case 1 and IBB test cases, do not add any relaxation for PC5 in FR2a.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. For ACS Case 1 and IBB test cases, for FR2a, define MTSU for PC5 equal to the MTSU defined for PC1.

Proposal 2. For ACS Case 1 and IBB test cases, do not add any relaxation for PC5 in FR2a.

# References

- R5-232158, “On FR2 PC5 MU analysis”, Keysight, RAN5#100

- R5-231774, “PC5 measurement grids including the alternate antenna array assumptions”, Keysight, RAN5#98

- R5-230180, “PC5 MU - definition for REFSENS test case in 38.903”, Keysight, RAN5#98

- R5-197497, “Discussion on testability of ACS and IBB”, Rohde & Schwarz, Anritsu, Keysight Technologies, RAN5#84

- TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, V18.0.0