3GPP TSG-RAN5 Meeting #101	R5-236061

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0988 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 MU - ACS Case 1 and IBB update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per analysis made in R5-236060, for ACS and IBB test cases, MTSU for PC5 in FR2a can be defined the same as currently defined for PC1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined MTSU for PC5 for ACS and IBB test case. Editor’s note updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5, 7.6.2, F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## 7.5	Adjacent channel selectivity

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainty is FFS for power classes other than 1, 3 and 5.

- The test case is incomplete for band n259 and for band n262.

-	The minimum conformance requirements for Case 2 in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed.

-	For power class 1, if testing were extended beyond 100MHz, potential relaxation required is FFS.

7.5.1	Test purpose

Adjacent channel selectivity tests the UE’s ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

7.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

7.5.3	Minimum conformance requirements

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the Radiated Interface Boundary (RIB) when the AoA of the incident wave of the wanted signal and the interfering signal are both from the direction where peak gain is achieved.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The UE shall fulfil the minimum requirement specified in Table 7.5.3-1 for all values of an adjacent channel interferer up to –25 dBm. However, it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.3-2 and Table 7.5.3-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2, with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.5.3-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258, n261 | dB | 23 | 23 | 23 | 23 |
| ACS for band n259, n260 | dB | 22 | 22 | 22 | 22 |

Table 7.5.3-2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| PInterferer for band n259, n260 | dBm | REFSENS + 34.5 dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in subclause 7.3.2.3, which are applicable to different UE power classes. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 5: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

Table 7.5.3-3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| Power in Transmission Bandwidth Configuration for band n259, n260 | dBm | -45.5 | -45.5 | -45.5 | -45.5 |
| PInterferer | dBm | -25 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 4: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 7.5.

7.5.4	Test description

7.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and subcarrier spacing, are shown in Table 7.5.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.5.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | 50 MHz, 100 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration |  |  | Uplink Configuration |  |
|  | Modulation | RB allocation |  | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 7.3.2.4.1-1. NOTE 2: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.2 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The DL and UL Reference Measurement channels are set according to Table 7.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 7.5.4.3.

7.5.4.2	Test procedure

1.	 Set the UE in the Rx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.5.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.5.4.1-1. Since the UL has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

4.	Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.5.5-2 (Case 1, PC3) or Table 7.5.5-2a (Case 1, PC1) or Table 7.5.5-3 (Case 2), for at least the duration of the throughput measurement, where:

- MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

- Uplink power control window size = 1dB (UE power step size) + 1dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-2 [3], Table 6.3.4.3-2 and is 1dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.3-1.

5.	Perform Blocking measurement procedure as stated in Annex K.1.8 using Downlink signal level and Interferer signal level as defined in Table 7.5.5-2 (Case 1, PC3) or Table 7.5.5-2a (Case 1, PC1). Modulated interferer signal characteristics as defined in Annex D with frequency below the wanted signal. Measure throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat step 5 using an interfering signal frequency above the wanted signal in Case 1.

7.	Perform Blocking measurement procedure as stated in Annex K.1.8 using Downlink signal level and Interferer signal level as defined in Table 7.5.5-3 (Case 2). Modulated interferer signal characteristics as defined in Annex D with frequency below the wanted signal. Measure throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

8.	Repeat step 7 using an interfering signal frequency above the wanted signal in Case 2.

9.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.5.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.5.5	Test requirements

The requirement below shall only be considered if UE output power measured in the test procedure step 4 ends within the Uplink power control window.

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, under the conditions specified in Table 7.5.5-2, Table 7.5.5-2a, and also under the conditions specified in Table 7.5.5-3.

Table 7.5.5-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258, n261 | dB | 23 | 23 | 23 | 23 |
| ACS for band n259, n260, n262 | dB | 22 | 22 | 22 | 22 |

Table 7.5.5-2: Test parameters for adjacent channel selectivity, Case 1, PC3

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 | dBm | REFSENS + 14 dB |  |  |  |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | REFSENS + 14 - 1.8 dB NOTE 4 | REFSEN + 14 - 4.8 dB NOTE 4 | REFSENS + 14 dB | REFSENS + 14 dB |
| Power in Transmission Bandwidth Configuration for band n259 | dBm | REFSENS + 14 - TBD dB | REFSEN + 14 - TBD dB | REFSENS + 14 - TBD dB | REFSENS + 14 - TBD dB |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB NOTE 5 | REFSENS +35.5dB NOTE 5 |
| PInterferer for band n260, n262 | dBm | REFSENS + 34.5 - 1.8 dB NOTE 4 | REFSENS +34.5 - 4.8 dB NOTE 4 | REFSENS +34.5dB NOTE 5 | REFSENS +34.5dB NOTE 5 |
| PInterferer for band n259 | dBm | REFSENS + 34.5 - TBD dB | REFSENS +34.5 - TBD dB | REFSENS +34.5 - TBD dB | REFSENS +34.5 - TBD dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in subclause 7.3.2.5. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. NOTE 5: Core requirement cannot be tested due to testability issue. NOTE 6: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 7: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

Table 7.5.5-2a: Test parameters for adjacent channel selectivity, Case 1, PC1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n260, n261 | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| PInterferer for band n260 | dBm | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in subclause 7.3.2.5. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 5: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

Table 7.5.5-2b: Test parameters for adjacent channel selectivity, Case 1, PC5

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258 | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in subclause 7.3.2.5. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 5: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

Table 7.5.5-3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| Power in Transmission Bandwidth Configuration for band n259, n260 | dBm | -45.5 | -45.5 | -45.5 | -45.5 |
| PInterferer | dBm | -25 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3 with one sided dynamic OCNG Pattern TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 4: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

## <<< Skip unchanged sections >>>

### 7.6.2	In-band blocking

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement uncertainty is FFS for power classes other than 1, 3 and 5.

- The test case is incomplete for band n259 and for band n262.

- For power class 1, if testing were extended beyond 100MHz, potential relaxation required is FFS.

7.6.2.0	General

In-band blocking is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel.

7.6.2.1	Test purpose

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the spectrum equivalent to twice the channel bandwidth below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

7.6.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

7.6.2.3	Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1). The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.6.2.3-1: In-band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | N/A | N/A | N/A |
| PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | N/A | N/A | N/A |
| FInterferer(offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 | ≤ -1600 & ≥ 1600 NOTE 5 | ≤ -3200 & ≥ 3200 | ≤ -4000 & ≥ 4000 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 | FDL_low + 400 to FDL_high - 400 | FDL_low + 800 to FDL_high - 800 | FDL_low + 1600 to FDL_high - 1600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Section 7.3.2.3, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) and set-up according to Annex C. NOTE 4: Void. NOTE 5: The absolute value of the interferer offset FInterferer(offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [10] clause 7.6.2.

7.6.2.4	Test description

7.6.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 7.6.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2. The details of the OCNG patterns used are specified in Annex A.5.

Table 7.6.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Mid range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | 50 MHz, 100 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration |  |  | Uplink Configuration |  |
|  | Modulation | RB allocation |  | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 7.3.2.4.1-1. NOTE 2: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.2  for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The DL and UL Reference Measurement channels are set according to Table 7.6.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38-508-1 [10] clause 4.5. Message content are defined in clause 7.6.2.4.3.

7.6.2.4.2	Test procedure

1.	 Set the UE in the Rx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

4.	Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6.2.5-1, for at least the duration of the throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 1dB (UE power step tolerance) ) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-2 [3], Table 6.3.4.3-2 and is 1dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.3-1.

5.	Perform Blocking measurement procedure as stated in Annex K.1.8 using Downlink signal level and Interferer signal level as defined in Table 7.6.2.5-1. Modulated interferer signal characteristics as defined in Annex D. Measure throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

6.	Repeat step 5 using interfering signals specified in 7.6.2.5-1. The ranges are covered in steps equal to the interferer bandwidth. Interferer frequencies should be chosen starting with an offset nearest to the centre frequency and sweep outwards towards the band edges. In order to ensure that full range is tested for interferer frequency, run last test steps at frequency equal to FInterferer range limit defined at the corresponding band edge.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

Table 7.6.2.4.2-1: Example for interferer frequencies

|  | Lower frequency | Upper frequency |
| --- | --- | --- |
| Band n257 | 26500.00 MHz | 29500.00 MHz |
| Band n257 Midrange | 27999.96 MHz |  |
| SCS | 120 kHz |  |
| CHBW | 100 MHz |  |
| Interferer (1st :most inner) | FFS | FFS |
| Interferer (2nd) | FFS | FFS |
| : | : | : |
| Interferer (13th) | FFS | FFS |
| Interferer (last step) NOTE 1 | FFS | FFS |
| Outer limit for in band blocking | FFS | FFS |
| Number of test frequencies | 14 | 14 |
| NOTE 1: Adjusted interferer frequency in the last step will be out of outer limit but should be tested. |  |  |

7.6.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.6.2.5	Test requirement

The requirement below shall only be considered if UE output power measured in the test procedure step 4 ends within the Uplink power control window.

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A with parameters specified in Table 7.6.2.5-1.

Table 7.6.2.5-1: In-band blocking test requirement for PC3

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for bands n257, n258, n261 | dBm | REFSENS + 14dB |  |  |  |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | REFSENS + 14 - 1.8 dB NOTE 7 | REFSENS + 14 - 4.8 dB NOTE 7 | REFSENS + 14 dB | REFSENS + 14 dB |
| Power in Transmission Bandwidth Configuration for band n259 | dBm | REFSENS + 14 - TBD dB | REFSENS + 14 - TBD dB | REFSENS + 14 - TBD dB | REFSENS + 14 - TBD dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB NOTE 8 | REFSENS + 35.5 dB NOTE 8 |
| PInterferer for band n260 | dBm | REFSENS + 34.5 - 1.8 dB NOTE 7 | REFSENS + 34.5 - 4.8 dB NOTE 7 | REFSENS + 34.5 dB NOTE 8 | REFSENS + 34.5 dB NOTE 8 |
| PInterferer for band n259 | dBm | REFSENS + 34.5 - TBD dB | REFSENS + 34.5 - TBD dB | REFSENS + 34.5 - TBD dB | REFSENS + 34.5 - TBD dB |
| FInterferer(offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Section 7.3.2.5, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: Void. NOTE 5: The absolute value of the interferer offset FInterferer(offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. NOTE 7: Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. NOTE 8: Core requirement cannot be tested due to testability issue. NOTE 9: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. |  |  |  |  |  |

Table 7.6.2.5-1a: In-band blocking test requirement for PC1

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for bands n257, n258, n260, n261 | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |
| PInterferer for band n260 | dBm | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| FInterferer(offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Section 7.3.2.5, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: VOID. NOTE 5: The absolute value of the interferer offset FInterferer(offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. |  |  |  |  |  |

Table 7.6.2.5-1b: In-band blocking test requirement for PC5

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for bands n257, n258 | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |
| FInterferer(offset) | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Section 7.3.2.5, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG pattern OP.1.TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 4: VOID. NOTE 5: The absolute value of the interferer offset FInterferer(offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. |  |  |  |  |  |

## <<< Skip unchanged sections >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 Max Device size ≤ 30 cm ±5.36 dB (FR2a, FR2b, NTC testing) [±5.95] dB (FR2c NTC testing) ±5.61 dB (FR2a, FR2b, ETC testing) TBD (FR2c ETC testing) PC1 Max Device size ≤ 30 cm ±5.58 dB (FR2a, FR2b, NTC testing) ±5.83 dB (FR2a, FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.58 dB (FR2a, NTC testing) ± 5.83 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |
| 7.3.4 EIS spherical coverage | PC3 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) [±5.66] dB (Max Device size ≤ 30 cm, FR2c) PC1 ±5.07 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC5 ±5.07 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.19-2 in TR 38.903) |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.4 Reference sensitivity power level for CA (5DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.5 Reference sensitivity power level for CA (6DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.6 Reference sensitivity power level for CA (7DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.7 Reference sensitivity power level for CA (8DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.3.1 EIS spherical coverage for CA (2DL CA) | TBD |  |
| 7.3A.3.2 EIS spherical coverage for CA (3DL CA) | TBD |  |
| 7.3A.3.3 EIS spherical coverage for CA (4DL CA) | TBD |  |
| 7.3A.3.4 EIS spherical coverage for CA (5DL CA) | TBD |  |
| 7.3A.3.5 EIS spherical coverage for CA (6DL CA) | TBD |  |
| 7.3A.3.6 EIS spherical coverage for CA (7DL CA) | TBD |  |
| 7.3A.3.7 EIS spherical coverage for CA (8DL CA) | TBD |  |
| 7.4 Maximum input level | TBD |  |
| 7.4A.1 Maximum input level for CA (2DL CA) | TBD |  |
| 7.4A.2 Maximum input level for CA (3DL CA) | TBD |  |
| 7.4A.3 Maximum input level for CA (4DL CA) | TBD |  |
| 7.4A.4 Maximum input level for CA (5DL CA) | TBD |  |
| 7.4A.5 Maximum input level for CA (6DL CA) | TBD |  |
| 7.4A.6 Maximum input level for CA (7DL CA) | TBD |  |
| 7.4A.7 Maximum input level for CA ((DL CA) | TBD |  |
| 7.5 Adjacent channel selectivity | PC3 ±8.08 dB (Max Device size ≤ 30 cm, FR2a, FR2b) TBD (Max Device size ≤ 30 cm, FR2c) PC1 ±8.31 dB (Max Device size ≤ 30 cm, FR2a, FR2b) PC5 ±8.31 dB (Max Device size ≤ 30 cm, FR2a) | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |
| 7.5A.1 Adjacent channel selectivity for CA (2UL CA) | TBD |  |
| 7.5A.2 Adjacent channel selectivity for CA (3UL CA) | TBD |  |
| 7.5A.3 Adjacent channel selectivity for CA (4UL CA) | TBD |  |
| 7.5A.4 Adjacent channel selectivity for CA (5UL CA) | TBD |  |
| 7.5A.5 Adjacent channel selectivity for CA (6UL CA) | TBD |  |
| 7.5A.6 Adjacent channel selectivity for CA (7UL CA) | TBD |  |
| 7.5A.7 Adjacent channel selectivity for CA (8UL CA) | TBD |  |
| 7.6.2 In-band blocking | Same as 7.5 |  |
| 7.6A.2.1 In-band blocking for CA (2UL CA) | TBD |  |
| 7.6A.2.2 In-band blocking for CA (3UL CA) | TBD |  |
| 7.6A.2.3 In-band blocking for CA (4UL CA) | TBD |  |
| 7.6A.2.4 In-band blocking for CA (5UL CA) | TBD |  |
| 7.6A.2.5 In-band blocking for CA (6UL CA) | TBD |  |
| 7.6A.2.6 In-band blocking for CA (7UL CA) | TBD |  |
| 7.6A.2.7 In-band blocking for CA (8UL CA) | TBD |  |
| 7.9 Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: For Band n257, n258, n260, n261: ±5.64dB (6GHz ≤ f < 12.75GHz) ±5.60dB (12.75GHz ≤ f < 23.45GHz) ±6.11dB (23.45GHz ≤ f < 40.8GHz) ±7.65dB (40.8GHz ≤ f < 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) PC1: For Band n257, n258, n260, n261: ±5.63dB (6GHz ≤ f < 12.75GHz) ±5.59dB (12.75GHz ≤ f < 23.45GHz) ±6.10dB (23.45GHz ≤ f < 40.8GHz) ±7.64dB (40.8GHz ≤ f < 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.25-1 in TR 38.903) |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table 5.1-2. |  |  |

## <<< END OF CHANGES >>>