3GPP TSG-RAN5 Meeting #101	R5-236069

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1669 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MUs - Annex F updates in 38.521-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT for FR2 NSA are defined just pointing to the MU and TT defined for the corresponding FR2 SA test. During FR2 MU discussions, it has been observed to be missing for some test cases despite MU and TT for corresponding SA tests are already defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the test cases 6.2B.1.4D.1 and 6.2B.1.4D.2, defined MU and TT in annex F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP |  | Same as clause 6.2.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage |  | Same as clause 6.2.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.1.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.1.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage |  | Same as clause 6.2A.1.2.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.2.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.2.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) -Spherical Coverage |  | Same as clause 6.2A.1.2.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.3.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.3 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.3.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.3 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.4 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.4 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.5.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.5 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.5.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.5 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.6.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.6 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.6.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) –Spherical Coverage |  | Same as clause 6.2A.1.2.6 in TS 38.521-2 |  |  |  |
| 6.2B.1.4D.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO |  | Same as clause 6.2D.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4D.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage |  | Same as clause 6.2D.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.2.1 UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.2.2 UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.2.3 UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.2.4 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 |  | Same as clause 6.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4a UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.1 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.3.2 UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.3.3 UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.3.4 UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC |  | Same as clause 6.2B.1.1 |  |  |  |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.2B.1.2 |  |  |  |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | Same as clause 6.2B.1.3 |  |  |  |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.4_1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 |  | Same as clause 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.4 Minimum Output Power for EN-DC Interband including FR2 |  | Same as clause 6.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.1 Minimum output power for inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.2 Minimum output power for inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.3 Minimum output power for inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 |  | Same as clause 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4 Transmit OFF Power for inter-band EN-DC including FR2 |  | Same as clause 6.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4_1.1 Transmit OFF Power for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.2 Transmit OFF Power for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.3 Transmit OFF Power for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 |  | Same as clause 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.4 Transmit ON/OFF time mask for inter-band EN-DC including FR2 |  | Same as clause 6.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.4_1.1 Transmit ON/OFF time mask for inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.3A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.4.3 PRACH Time Mask for inter-band EN-DC within FR1 |  | Same as clause 6.3.3.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.1 Absolute power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.2 Absolute power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.3 Absolute power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.4 Absolute power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.2.1 Relative power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.2 Relative power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.3 Relative power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.4 Relative power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.3.1 Aggregate power tolerance for intra-band contiguous EN-DC |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.2 Aggregate power tolerance for intra-band non-contiguous EN-DC |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.3 Aggregate power tolerance for inter-band EN-DC within FR1 |  | Same as clause 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.4 Aggregate power tolerance for inter-band EN-DC including FR2 |  | Same as clause 6.3.4.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.1 Frequency Error for intra-band contiguous EN-DC |  | TBD |  |  |  |
| 6.4B.1.2 Frequency Error for intra-band non-contiguous EN-DC |  | TBD |  |  |  |
| 6.4B.1.3 Frequency Error for inter-band EN-DC within FR1 |  | Same as clause 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.4 Frequency Error for inter-band EN-DC including FR2 |  | Same as clause 6.4.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.1 Frequency Error for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.4A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.2 Frequency Error for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.4A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.3 Frequency Error for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.4A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.1. |  |  |  |
| 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.3 In-band Emissions for intra-band contiguous EN-DC |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] for ENBW ≤100MHz. Uplink power measurement for steps 2 and 8 same as 6.2B.1.1. Uplink power measurement for steps 4, 6, 10, and 12 same as 6.3B.1.1. |  |  |  |
| 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] Uplink power measurement for step 2 and step 4 same as 6.2B.1.2. Uplink power measurement for step 6 and step 8 same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] Uplink power measurement for steps 2 and 8 same as 6.2B.1.2. Uplink power measurement for steps 4, 6, 10, and 12 same as 6.3B.1.2. |  |  |  |
| 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non-contiguous EN-DC |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.2 in TS 38.521-1 [8] Uplink power measurement for step 2 and step 4 same as 6.2B.1.3. Uplink power measurement for step 6 and step 8 same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.3 in TS 38.521-1 [8] Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2B.1.3. Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3B.1.3. |  |  |  |
| 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 |  | Same as clause 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.4.1 Error Vector Magnitude for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.2 Carrier Leakage for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.3 In-band Emissions for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.4 EVM Equalizer Flatness for inter-band EN-DC including FR2 |  | Same as clause 6.4.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.5 EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC |  | 1.5% of aggregated channel bandwidth |  |  |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC |  | Same as clause 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 |  | Same as clause 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 |  | Same as clause 6.5.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO |  | Same as clause 6.5D.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC |  | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC |  | TBD |  |  |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.1NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.2 UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as clause 6.5.2.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 |  | Same as clause 6.5.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.5.2.1.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 |  | Same as clause 6.5.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.4 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs) |  | Same as clause 6.5A.2.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.5 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs) |  | Same as clause 6.5A.2.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.6 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs) |  | Same as clause 6.5A.2.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.7 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs) |  | Same as clause 6.5A.2.2.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 |  | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (2 CCs) |  | Same as clause 6.5.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) |  | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) |  | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) |  | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) |  | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 |  | Same as clause 6.5.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) |  | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) |  | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) |  | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 |  | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as clause 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as clause 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | MU for NR CC downlink power same as clause 7.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as clause 7.4A.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] for NR CC Same as clause 7.5 in TS 36.521 [10] Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.5 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.3 Adjacent Channel Selectivity for EN-DC within FR1 (5 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.3_1.4 Adjacent Channel Selectivity for EN-DC within FR1 (6 CCs) |  | Same as clause 7.5A in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.1 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.2 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.3 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.5B.4_1.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (6 CCs) |  | Same as clause 7.5A in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.2.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.2 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | TBD |  |  |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.3 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.3.1 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.6.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | Same as clause 7.6A.4.1 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | Same as clause 7.6.4 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | MU for NR CC downlink power same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.3. |  |  |  |
| 7.7B.3_1.1 Spurious Response for EN-DC within FR1 (3 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.7B.3_1.2 Spurious Response for EN-DC within FR1 (4 CCs) |  | Same as clause 7.7 in TS 38.521-1 [8] for each component carrier. Uplink power measurement same as 6.2B.1.2. |  |  |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 |  | MU for NR CC downlink power same as clause 7.8.2 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.2. |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC in FR1 |  | Same as clause 7.8.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.1 Wideband Intermodulation for EN-DC within FR1 (3 CCs) |  | Same as clause 7.8A.2.1 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.2 Wideband Intermodulation for EN-DC within FR1 (4 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.3 Wideband Intermodulation for EN-DC within FR1 (5 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.8B.2.3_1.4 Wideband Intermodulation for EN-DC within FR1 (6 CCs) |  | Same as clause 7.8A.2 in TS 38.521-1 [8] Uplink power measurement for same as 6.2B.1.3. |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as clause 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as clause 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as clause 7.9 in TS 38.521-2 [9] |  |  |  |

# F.2	Interpretation of measurement results (normative)

TBD

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

## F.3.1	Measurement of test environments

TBD

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | Same as 6.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.2 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.1 in TS 38.521-1 [8]. |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.3 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.2 in TS 38.521-1 [8]. |  |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.1.4 UE Maximum Output Power for Inter-Band EN-DC including FR2 |  | Same as 6.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TR |  | Same as 6.2.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage |  | Same as 6.2.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.1.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - EIRP and TRP |  | Same as 6.2A.1.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.1.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (2 NR CCs) - Spherical Coverage |  | Same as 6.2A.1.2.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.2.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.2.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 NR CCs) - Spherical Coverage |  | Same as clause 6.2A.1.2.2 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.3.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.3 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.3.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.3 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.4 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.4 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.5.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.5 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.5.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (6 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.5 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.6.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) - EIRP and TRP |  | Same as clause 6.2A.1.1.6 in TS 38.521-2 |  |  |  |
| 6.2B.1.4_1.6.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (7 NR CCs) – Spherical Coverage |  | Same as clause 6.2A.1.2.6 in TS 38.521-2 |  |  |  |
| 6.2B.1.4D.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO |  | Same as clause 6.2D.1.1 in TS 38.521-2 |  |  |  |
| 6.2B.1.4D.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 for UL MIMO- Spherical coverage |  | Same as clause 6.2D.1.2 in TS 38.521-2 |  |  |  |
| 6.2B.2.1 UE Maximum Output Power reduction for Intra-Band Contiguous EN-DC |  | Same as 6.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.2B.2.2 UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.2.3 UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.2.4 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 |  | Same as clause 6.2.2 and 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4a UE Maximum Output Power reduction enhancements for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.2.4_1.1 UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.2A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.2B.3.1 UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC |  | Same as 6.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.2B.3.2 UE Additional Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.3.3 UE Additional Maximum Output Power reduction for Inter-Band EN-DC within FR1 |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.3.4 UE Additional Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.2B.4.1.1 Configured Output Power Level for Intra-Band Contiguous EN-DC |  | Same as 6.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.2B.4.1.2 Configured Output Power for Intra-Band Non-Contiguous EN-DC |  | Same as 6.2B.1.2 |  |  |  |
| 6.2B.4.1.3 Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.3_1 Configured Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |
| 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.2.4_1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.1 Minimum Output Power for intra-band contiguous EN-DC |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.2 Minimum output power for intra-band non-contiguous EN-DC |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.3 Minimum output power for inter-band EN-DC within FR1 |  | Same as 6.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.3B.1.4 Minimum Output Power for EN-DC Interband including FR2 |  | Same as 6.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.1 Minimum output power for inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.2 Minimum output power for inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.1.4_1.3 Minimum output power for inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.1 Transmit OFF Power for intra-band contiguous EN-DC |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.2 Transmit OFF Power for intra-band non-contiguous EN-DC |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.3 Transmit OFF Power for inter-band EN-DC within FR1 |  | Same as 6.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4 Transmit OFF Power for inter-band EN-DC including FR2 |  | Same as 6.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.1 Tx ON/OFF time mask for intra-band contiguous EN-DC |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.2.4_1.1 Transmit OFF Power for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.2 Transmit OFF Power for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.2.4_1.3 Transmit OFF Power for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.2 Tx ON/OFF time mask for intra-band non-contiguous EN-DC |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.3 Tx ON/OFF time mask for inter-band EN-DC within FR1 |  | Same as 6.3.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.3.4 Transmit ON/OFF time mask for inter-band EN-DC including FR2 |  | Same as 6.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.3.4_1.1 Transmit ON/OFF time mask for inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.3A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.3B.4.3 PRACH Time Mask for inter-band EN-DC within FR1 |  | Same as 6.3.3.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.1 Absolute power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.2 Absolute power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.3 Absolute power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.1.4 Absolute power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.2 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.2.1 Relative power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.2 Relative power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.3 Relative power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.2.4 Relative power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.3 in TS 38.521-2 [9] |  |  |  |
| 6.3B.8.3.1 Aggregate power tolerance for intra-band contiguous EN-DC |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.2 Aggregate power tolerance for intra-band non-contiguous EN-DC |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.3 Aggregate power tolerance for inter-band EN-DC within FR1 |  | Same as 6.3.4.4 in TS 38.521-1 [8] |  |  |  |
| 6.3B.8.3.4 Aggregate power tolerance for inter-band EN-DC including FR2 |  | Same as 6.3.4.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.1 Frequency Error for intra-band contiguous EN-DC |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.2 Frequency Error for intra-band non-contiguous EN-DC |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.3 Frequency Error for inter-band EN-DC within FR1 |  | Same as 6.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.1.4 Frequency Error for inter-band EN-DC including FR2 |  | Same as 6.4.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.1 Frequency Error for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 6.4A.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.2 Frequency Error for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 6.4A.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.1.4_1.3 Frequency Error for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 6.4A.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.1.1 Error Vector Magnitude for intra-band contiguous EN-DC |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.2 Carrier Leakage for intra-band contiguous EN-DC |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.3 In-band Emissions for intra-band contiguous EN-DC |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.1.4 EVM Equalizer Flatness for intra-band contiguous EN-DC |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.1 Error Vector Magnitude for intra-band non-contiguous EN-DC |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.2 Carrier Leakage for intra-band non-contiguous EN-DC |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.3 In-band Emissions for intra-band non-contiguous EN-DC |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.2.4 EVM Equalizer Flatness for intra-band non-contiguous EN-DC |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.1 Error Vector Magnitude for inter-band EN-DC within FR1 |  | Same as 6.4.2.1 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.2 Carrier Leakage for inter-band EN-DC within FR1 |  | Same as 6.4.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.3 In-band Emissions for inter-band EN-DC within FR1 |  | Same as 6.4.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.3.4 EVM Equalizer Flatness for inter-band EN-DC within FR1 |  | Same as 6.4.2.4 in TS 38.521-1 [8] |  |  |  |
| 6.4B.2.4.1 Error Vector Magnitude for inter-band EN-DC including FR2 |  | Same as 6.4.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.4.2.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.2 Carrier Leakage for inter-band EN-DC including FR2 |  | Same as 6.4.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.3 In-band Emissions for inter-band EN-DC including FR2 |  | Same as 6.4.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.4 EVM Equalizer Flatness for inter-band EN-DC including FR2 |  | Same as 6.4.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.4B.2.4.5 EVM spectral flatness for pi/2 BPSK modulation for inter-band EN-DC including FR2 (1 NR CC) |  | Same as 6.4.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 |  | Same as 6.5.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 |  | Same as 6.5.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) |  | TBD |  |  |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) |  | TBD |  |  |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO |  | Same as 6.5D.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as 6.5.2.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 |  | Same as 6.5.2.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.1 NR - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.3.3.2 UTRA - Adjacent channel leakage ratio for inter-band EN-DC within FR1 (1 NR CC) |  | Same as 6.5.2.4.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 |  | Same as 6.5.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC) |  | Same as clause 6.5.2.1.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 |  | Same as 6.5.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 CCs) |  | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 CCs) |  | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 CCs) |  | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.4 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 NR CCs) |  | Same as clause 6.5A.2.2.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.5 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (6 NR CCs) |  | Same as clause 6.5A.2.2.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.6 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (7 NR CCs) |  | Same as clause 6.5A.2.2.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.2.4.3_1.7 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (8 NR CCs) |  | Same as clause 6.5A.2.2.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 |  | Same as 6.5.3.1 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 |  | Same as 6.5.3.2 in TS 38.521-1 [8] |  |  |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as 6.5.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) |  | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) |  | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) |  | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) |  | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 |  | Same as 6.5.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) |  | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) |  | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) |  | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 |  | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |  |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) |  | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) |  | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) |  | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) |  | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 |  | Same as 6.5.4 in TS 38.521-1 [8] |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP |  | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.3.2 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.3A.1 in TS 38.521-1 [8] |  |  |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 |  | Same as 7.3.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.1 Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs) |  | Same as 7.3A.2.1 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.2 Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs) |  | Same as 7.3A.2.2 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.3 Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs) |  | Same as 7.3A.2.3 in TS 38.521-2 [9] |  |  |  |
| 7.3B.2.4_1.4 Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs) |  | Same as 7.3A.2.4 in TS 38.521-2 [9] |  |  |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 |  | Same as 7.3.4 in TS 38.521-2 [9] |  |  |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.4 in TS 38.521-1 [8] |  |  |  |
| 7.4B.3_1.1 Maximum Input Level for Inter-band EN-DC within FR1 (3 CCs) |  | Same as 7.4A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.1 Adjacent Channel Selectivity for intra-band contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.2 Adjacent Channel Selectivity for intra-band non-contiguous EN-DC (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3 Adjacent Channel Selectivity for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.1 Adjacent Channel Selectivity for EN-DC within FR1 (3 CCs) |  | Same as 7.5A.1 in TS 38.521-1 [8] |  |  |  |
| 7.5B.3_1.2 Adjacent Channel Selectivity for EN-DC within FR1 (4 CCs) |  | Same as 7.5 in TS 38.521-1 [8] |  |  |  |
| 7.5B.4 Adjacent Channel Selectivity for inter-band EN-DC including FR2 (2CCs) |  | Same as clause 7.5 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.1 Inband blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.2 Inband blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3 Inband blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.2 in TS 38.521-1 [8] |  |  |  |
| 7.6B.2.3_1.1 Inband blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.2 Inband blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.3_1.3 Inband blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.2.4 Inband blocking for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.6.2 in TS 38.521-2 [9] |  |  |  |
| 7.6B.2.4_1.1 Inband blocking for inter-band EN-DC including FR2 (3 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.2 Inband blocking for inter-band EN-DC including FR2 (4 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.3 Inband blocking for inter-band EN-DC including FR2 (5 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.2.4_1.4 Inband blocking for inter-band EN-DC including FR2 (6 CCs) |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6B.3.1 Out-of-band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.2 Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3 Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.3 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.1 Narrow band blocking for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.2 Narrow band blocking for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3 Narrow band blocking for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.6.4 in TS 38.521-1 [8] |  |  |  |
| 7.6B.4.3_1.1 Narrow band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.2 Narrow band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.4.3_1.3 Narrow band blocking for EN-DC within FR1 (5 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.7B.1 Spurious Response for intra-band contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.2 Spurious Response for intra-band non-contiguous EN-DC in FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.7B.3 Spurious Response for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.7 in TS 38.521-1 [8] |  |  |  |
| 7.6B.3.3_1.1 Out-of-band blocking for EN-DC within FR1 (3 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.6B.3.3_1.2 Out-of-band blocking for EN-DC within FR1 (4 CCs) |  | 0 dB |  | Wanted signal power + TT Interferer signal power unchanged T-put limit unchanged |  |
| 7.8B.2.1 Wideband Intermodulation for intra-band contiguous EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.2 Wideband Intermodulation for intra-band non-contiguous EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3 Wideband Intermodulation for inter-band EN-DC in FR1 |  | Same as 7.8.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.1 Wideband Intermodulation for EN-DC within FR1 (3 CCs) |  | Same as 7.8A.2.1 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.2 Wideband Intermodulation for EN-DC within FR1 (4 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.3 Wideband Intermodulation for EN-DC within FR1 (5 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.8B.2.3_1.4 Wideband Intermodulation for EN-DC within FR1 (6 CCs) |  | Same as 7.8A.2 in TS 38.521-1 [8] |  |  |  |
| 7.9B.1 Spurious Emissions for intra-band contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.2 Spurious Emissions for intra-band non-contiguous EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3 Spurious Emissions for inter-band EN-DC within FR1 (2 CCs) |  | Same as 7.9 in TS 38.521-1 [8] |  |  |  |
| 7.9B.3_1.1 Spurious Emissions for EN-DC within FR1 (3 CCs) |  | Same as 7.9A.1 in TS 38.521-1 [8] |  |  |  |
| 7.9B.4 Spurious Emissions for inter-band EN-DC including FR2 (2 CCs) |  | Same as 7.9 in TS 38.521-2 [9] |  |  |  |

## <<< END OF CHANGES >>>