3GPP TSG-RAN5 Meeting #101	R5-236092

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0821 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity and spurious emissions TP analysis for new R16 NR CA comb within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test points of reference sensitivity for CA_n3A-n28A, CA_n3A-n77(2A), CA_n28A-n77(2A), CA_n3A-n28A-n77A need to be indtroduced. - Test point analysis, test cases 6.5A.3.1.1 (General spurious emissions) and 6.5A.3.2 (Spurious emission band UE co-existence) for CA_n3A- n28A need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the TP analysis of reference sensitivity for CA_n3A-n28A, CA_n3A-n77(2A), CA_n28A-n77(2A), CA_n3A-n28A-n77A into Table 4.1.3.1-2 and Table 4.1.3.1-3. - Adding the TP analysis of spurious emissions for CA_n3A-n28A into Table 4.1.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test point analysis of reference sensitivity for CA_n3A-n28A, CA_n3A-n77(2A), CA_n28A-n77(2A), CA_n3A-n28A-n77A will remain incomplete. - Test point analysis of spurious emissions for CA_n3A-n28A will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1, 4.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 2466 TS 38.521-1 CR 2468 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | NOTE: Attached zip-file need to be added. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

4.1.3	Test point analysis per NR CA configuration

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1 through Table 4.1.3.1-5.

This clause contains information on test point analysis, test frequency selection and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1 through Table 4.1.3.1 4. The principles for selection of reference sensitivity test points for NR CA is given in Annex B including a test point analysis check list in B.9.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band)

| CA config |  | Single band refsens exception, victim band (Note 2) |  | Requirement coverage (Note 2) |  | Test point analysis updated |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) |  |  |  | NE |  | RAN5#100 |  |
| CA_n41C |  |  |  | NE |  | RAN5#89-e |  |
| CA_n48B |  |  |  | NE |  | RAN5#94-e |  |
| CA_n66B |  |  |  | NE |  | RAN5#86-e |  |
| CA_n66(2A) |  |  |  | NE |  | RAN5#86-e |  |
| CA_n66(3A) |  |  |  | NE |  | RAN5#100 |  |
| CA_n71(2A) |  |  |  | NE |  | RAN5#94-e |  |
| CA_n77(2A) |  |  |  | NE |  | RAN5#100 |  |
| CA_n78C |  |  |  | NE |  | RAN5#87-e |  |
| NOTE 1: Void. NOTE 2: Notations used: NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |  |  |  |  |

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config |  | Two band refsens exception, victim band (Note 1) |  | Requirement coverage (Note 1) |  | Test point analysis updated |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  | n1 (IMD3) n3(CBI) |  | IMD,CBI |  | RAN5#95-e |  |
| CA_n1A-n8A |  | n1 (IMD4) |  | IMD |  | RAN5#95-e |  |
| CA_n1A-n77A |  | n77 (HD2) |  | HD |  | RAN5#89-e |  |
| CA_n1A-n78A |  | n1 (IMD4) |  | IMD |  | RAN5#94 |  |
| CA_n2A-n48A |  | n48 (HD2), n2(IMD4) |  | HD, IMD |  | RAN5#96-e |  |
| CA_n2A-n5A |  | - |  | NE |  | RAN5#97 |  |
| CA_n2A-n14A |  | - |  | NE |  | RAN5#100 |  |
| CA_n2A-n66A |  | n2(IMD3), n66(IMD5) |  | IMD |  | RAN5#96-e |  |
| CA_n2A-n77A PC3 |  | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) |  | HD, HM, IMD, |  | RAN5#99 |  |
| CA_n2A-n77A PC2 |  | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4) n2(CBI) |  | HD, HM, IMD, CBI |  | RAN5#99 |  |
| CA_n3A-n5A |  | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) |  | IMD |  | RAN5#95-e |  |
| CA_n3A-n8A |  | n8(IMD4), n3(IMD5) |  | IMD |  | RAN5#100 |  |
| CA_n3A-n28A |  | - |  | NE |  | RAN5#101 |  |
| CA_n3A-n41A PC2 |  | n3 (IMD4), n3 (CBI) |  | IMD, CBI |  | RAN5#100 |  |
| CA_n3A-n41A PC3 |  | n3 (IMD4), n3 (CBI), n41 (CBI) |  | IMD, CBI |  | RAN5#100 |  |
| CA_n3A-n77A |  | n77 (HD2), n3 (IMD2), n3 (IMD4) |  | HD, IMD |  | RAN5#99 |  |
| CA_n3A-n77(2A) |  | (NOTE 2) |  | NE |  | RAN5#101 |  |
| CA_n3A-n78A |  | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) |  | HD, IMD |  | RAN5#87-e |  |
| CA_n5A-n66A |  | n5(IMD2) |  | IMD |  | RAN5#96-e |  |
| CA_n5A-n77A PC3 |  | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) |  | HD, HM, IMD |  | RAN5#99 |  |
| CA_n5A-n77A PC2 |  | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4) |  | HD, HM, IMD |  | RAN5#99 |  |
| CA_n5A-n78A |  | n78 (HD2) |  | HD |  | RAN5#94-e |  |
| CA_n7A-n78A |  | n7 and n78 (CBI) |  | CBI |  | RAN5#94-e |  |
| CA_n8A-n78A |  | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) |  | HD, IMD |  | RAN5#87-e |  |
| CA_n14A-n30A |  | - |  | NE |  | RAN5#100 |  |
| CA_n14A-n66A |  | - |  | NE |  | RAN5#100 |  |
| CA_n14A-n77A |  | n77 (HD5), n14 (HM), n14 (IMD5) |  | HD, HM, IMD |  | RAN5#100 |  |
| CA_n20A-n78A |  | n78 (HD4), n20 (IMD4) |  | HD, IMD |  | RAN5#100 |  |
| CA_n26A-n66A |  | n26 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n26A-n70A |  | n26 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n28A-n41A |  |  |  | NE |  | RAN5#91-e |  |
| CA_n28A-n77A |  | n77 (HD5), n28 (IMD5) |  | HD, IMD |  | RAN5#99 |  |
| CA_n28A-n77(2A) |  | (NOTE 2) |  | NE |  | RAN5#101 |  |
| CA_n28A-n78A |  | n78 (HD5) |  | HD |  | RAN5#99 |  |
| CA_n28A-n79A |  |  |  | NE |  | RAN5#91-e |  |
| CA_n29A-n66A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n29A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n29A-n71A |  | n29 (CBI) |  | CBI |  | RAN5#95-e |  |
| CA_n39A-n41A |  | - |  | NE |  | RAN5#100 |  |
| CA_n41A-n66A3 |  | n66 (CBI), n66 (IMD5) |  | CBI |  | RAN#97 |  |
| CA_n41A-n71A3 |  | n41(HD4), n71 (IMD4) |  | HD |  | RAN5#98 |  |
| CA_n41A-n79A PC3 |  |  |  | NE |  | RAN5#83 |  |
| CA_n41A-n79A PC2 |  | n41 (CBI), n79 (CBI) |  | CBI |  | RAN5#100 |  |
| CA_n48A-n66A |  | n48 (HD2), n66 (IMD5) |  | HD, IMD |  | RAN5#94-e |  |
| CA_n48A-n66(2A) |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48A-n70A |  | n70 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n48A-n71A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n48A-n71(2A) |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48A-n77A |  | - |  | NE |  | RAN5#97-e |  |
| CA_n48B-n66A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48B-n70A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48B-n71A |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n66A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n66(2A) |  | (NOTE 2) |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n70A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n71A |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n66A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n66A-n71A |  | n66 (IMD4) |  | IMD |  | RAN5#87-e |  |
| CA_n66A-n77A PC3 |  | n77 (HD2), n66(IMD2), n66(IMD5) |  | HD, IMD |  | RAN5#99 |  |
| CA_n66A-n77A PC2 |  | n77 (HD2), n66(IMD2), n66(IMD5), n66(CBI) |  | HD, IMD, CBI |  | RAN5#99 |  |
| CA_n70A-n71A |  | n70 (HD3), n70 (IMD4) |  | HD, IMD |  | RAN5#85 |  |
| CA_n71A-n77A |  | n71 (IMD5) |  | IMD |  | RAN5#97-e |  |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case. NOTE 3: Only single uplink analysed. IMD exception not yet covered. |  |  |  |  |  |  |  |

Table 4.1.3.1-3: Reference Sensitivity test cases per CA configuration (three bands)

| CA config |  | Three band refsens exception, victim band (Note 1) |  | Requirement coverage (Note 1) |  | Test point analysis updated |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n5A-n48A |  | n2(IMD3), n48(IMD3) |  | IMD |  | RAN5#100 |  |
| CA_n2A-n5A-n77A PC3 |  | n2(IMD3), n5 (IMD5), n77(IMD3) |  | IMD |  | RAN5#98 |  |
| CA_n2A-n5A-n77A PC2 |  | n2(IMD3), n5 (IMD5) |  | IMD |  | RAN5#100 |  |
| CA_n2A-n48A-n66A |  | n48(IMD2), n66 (IMD4), n2(IMD2) |  | IMD |  | RAN5#100 |  |
| CA_n2A-n48A-n77A |  | - |  | NE |  | RAN5#100 |  |
| CA_n2A-n66A-n77A PC3 |  | n2(IMD2, IMD4, IMD5), n66 (IMD2, IMD4, IMD5), n77(IMD2, IMD4) |  | IMD |  | RAN5#98 |  |
| CA_n2A-n66A-n77A PC2 |  | n2(IMD2), n66 (IMD2) |  | IMD |  | RAN5#100 |  |
| CA_n3A-n28A-n77A |  | n3(IMD3), n28(IMD3), n77(IMD3) |  | IMD |  | RAN5#101 |  |
| CA_n5A-n48A-n66A |  | n48(IMD5) |  | IMD |  | RAN5#100 |  |
| CA_n5A-n48A-n77A |  | - |  | NE |  | RAN5#100 |  |
| CA_n5A-n66A-n77A PC3 |  | n66 (IMD3), n77(IMD3, IMD4, IMD5) |  | IMD |  | RAN5#98 |  |
| CA_n5A-n66A-n77A PC3 |  | n66 (IMD3) |  | IMD |  | RAN5#100 |  |
| CA_n26A-n66A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n26A-n66(2A)-n70A |  |  |  | NE |  | RAN5#96-e |  |
| CA_n28A-n41A-n79A |  | n28(IMD3), n41 (IMD4), n79(IMD3) |  | IMD |  | RAN5#100 |  |
| CA_n29A-n66A-n70A |  |  |  | NE |  | RAN5#89-e |  |
| CA_n41A-n66A-n71A |  |  |  | NE |  | RAN5#99 |  |
| CA_n48A-n66A-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n66(2A)-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n66(2A)-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n70A-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48B-n66A-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48B-n66A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n66A-n77A |  | - |  | NE |  | RAN5#100 |  |
| CA_n48B-n70A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n66A-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n66A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n70A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n66A-n70A-n71A |  |  |  | NE |  | RAN5#87-e |  |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |  |  |  |  |

-------- Skip sections not changed --------

4.1.3.2	Spurious emissions test cases for FR1 UL CA

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per CA configuration and are stored as zip-files as defined in annex A.

Table 4.1.3.2-1: Frequency range analysis availability per CA configuration

| CA config | Number of test point | Justification | Comments |
| --- | --- | --- | --- |
| CA_n1A-n3A |  | 38.521-1_TpAnalysisSpur(CA_n1A-n3A).zip | RAN5#99 |
| CA_n1A-n8A | 1 | 38.521-1_TpAnalysisSpur(CA_n1A-n8A).zip | RAN5#98 |
| CA_n1A-n78A | 3 | 38.521-1_TpAnalysisSpur(CA_n1A-n78A) v3.zip | RAN5#97 |
| CA_n1A-n79A | 4 | 38.521-1_TpAnalysisSpur(CA_n1A-n79A).zip | RAN5#97 |
| CA_n2A-n77A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A-n77A) v2.zip | RAN5#97 |
| CA_n2A-n5A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A_n5A).zip | RAN5#97 |
| CA_n2A-n48A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A_n48A).zip | RAN5#97 |
| CA_n3A-n8A | 3 | 38.521-1_TpAnalysisSpur(CA_n3A-n8A).zip | RAN5#100 |
| CA_n3A-n28A | 3 | 38.521-1_TpAnalysisSpur(CA_n3A-n28A).zip | RAN5#101 |
| CA_n3A-n41A | 6 | 38.521-1_TpAnalysisSpur(CA_n3A-n41A).zip | RAN5#97 |
| CA_n3A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n3A-n77A).zip | RAN5#100 |
| CA_n3A-n78A | 4 | 38.521-1_TpAnalysisSpur(CA_n3A-n78A) v1.zip | RAN5#97 |
| CA_n5A-n48A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n48A) v2.zip | RAN5#99 |
| CA_n5A-n66A | 3 | 38.521-1_TpAnalysisSpur(CA_n5A-n66A).zip | RAN5#100 |
| CA_n5A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n77A) v2.zip | RAN5#97 |
| CA_n8A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n8A-n78A) v1.zip | RAN5#97 |
| CA_n20A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n20A-n78A).zip | RAN5#100 |
| CA_n24A-n41A | 5 | 38.521-1_TpAnalysisSpur(CA_n24A-n41A).zip | RAN5#95e |
| CA_n24A-n48A | 2 | 38.521-1_TpAnalysisSpur(CA_n24A-n48A).zip | RAN5#95e |
| CA_n24A-n77A | 4 | 38.521-1_TpAnalysisSpur(CA_n24A-n77A).zip | RAN5#95e |
| CA_n26A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n66A).zip | RAN5#96e |
| CA_n26A-n70A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n70A).zip | RAN5#96e |
| CA_n28A-n41A | 4 | 38.521-1_TpAnalysisSpur(CA_n28A-n41A) v1.zip | RAN5#97 |
| CA_n28A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n28A-n77A).zip | RAN5#100 |
| CA_n28A-n78A | 6 | 38.521-1_TpAnalysisSpur(CA_n28A-n78A).zip | RAN5#99 |
| CA_n28A-n79A | 1 | 38.521-1_TpAnalysisSpur(CA_n28A-n79A).zip | RAN5#100 |
| CA_n39A-n41A | 1 | 38.521-1_TpAnalysisSpur(CA_n39A-n41A).zip | RAN5#97 |
| CA_n41A-n79A | 8 | 38.521-1_TpAnalysisSpur(CA_n41A-n79A) v1.zip | RAN5#97 |
| CA_n48A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n48A-n66A) v1.zip | RAN5#97 |
| CA_n48A-n70A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n70A).zip | RAN5#96e |
| CA_n48A-n71A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n71A) v1.zip | RAN5#97 |
| CA_n66A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n66A-n71A)_v1.zip | RAN5#99 |
| CA_n66A-n77A | 9 | 38.521-1_TpAnalysisSpur(CA_n66A-n77A) v2.zip | RAN5#97 |
| CA_n70A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n70A-n71A).zip | RAN5#96e |
| CA_n71A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n71A-n77A).zip | RAN5#97 |

-------- End of changes --------

Add the attached .zip file to TR 38.905: “38.521-1_TpAnalysisSpur(CA_n3A-n28A).zip”