3GPP TSG WG RAN5 Meeting #101	R5-236126

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test configuration and error correction for 7.6A.2 In-band blocking for category M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test configuration for 7.6A.2 In-band blocking for category M1 is still missing. - There are errors in “Table 7.6A.2.4.2-1: Example for interferer frequencies”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test configuration for 7.6A.2 In-band blocking for category M1 was added. - Errors in “Table 7.6A.2.4.2-1: Example for interferer frequencies” were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test configuration for 7.6A.2 In-band blocking for category M1 will remain missing. - Errors in “Table 7.6A.2.4.2-1: Example for interferer frequencies” will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in R5-236140. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6A.2	In-band blocking for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.6A.2.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.6A.2.3	Minimum Conformance Requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.2.3-1 and 7.6A.2.3-2. For operating bands with an unpaired DL part (as noted in Table 5.2A-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.2.3-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.3-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.6A.2.

7.6A.2.4	Test Description

7.6A.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for category M1 in clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6A.2.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in TS 36.521-1 [14] Annexes A.2 and A.3. The details of the OCNG patterns used are specified in TS 36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1 [14] Annex C.2.

Table 7.6A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  |  | Uplink Configuration |  |  |
|  | Modulation | RB allocation |  | Modulation | RB allocation | Narrowband index (Note 1) |
| 1 | QPSK | 4 |  | QPSK | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211[3], 5.2.4. Note 2: Downlink RB position shall be RBstart = 0 within the narrowband. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6A.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	UE location according to TS 36.508 [12] clause 4.13 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB31 configuration in TS 36.508 [12].

8.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.6A.2.4.3.

7.6A.2.4.2	Test Procedure

1.	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.6A.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The SS sends one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.521-1 [14] Annex A.5.1.1.

2.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 7.6A.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the parameters of the signal generator for an interfering signal below the wanted signal in Case 1 according to Tables 7.6A.2.5-1 and 7.6A.2.5-2.

4.	Set the Downlink signal level to the value as defined in Table 7.6A.2.5-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6A.2.5-1 for at least the duration of the throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101-1 [7], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

5.	Measure the average throughput for duration sufficient to achieve statistical significance according to TS 36.521-1 [14] Annex G.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal in Case 1 at step 3.

7.	Repeat steps from 3 to 6, using interfering signals in Case 2 at step 3 and 6. The ranges of case 2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to Table 7.6A.2.4.2-1.

Table 7.6A.2.4.2-1: Example for interferer frequencies

|  | Lower frequency | Upper frequency |
| --- | --- | --- |
| Band 256 DL | 2170 MHz | 2200 MHz |
| Band 256 Midrange | 2185 MHz |  |
| Receive band wanted signal (BW 1.4MHz) | 2184.3 MHz | 2185.7 MHz |
| Interferer case 1 | 2182.1875 MHz | 2187.8125 MHz |
| Interferer case 2 (inner frequency) | 2180.7925 MHz | 2189.2075 MHz |
| Interferer case 2 (outer frequency) | 2155.5925 MHz | 2214.4075 MHz |
| Outer limit for in band blocking | 2155MHz | 2215MHz |
| Number of test frequencies case 2 | 19 | 19 |

7.6A.2.4.3	Message Contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and the following exception.

Table 7.6A.2.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-UE-PUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

7.6A.2.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [14] Annex A.3.2 with parameters specified in Tables 7.6A.2.5-1 and 7.6A.2.5-2.

Table 7.6A.2.5-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.5-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

<< End of changes >>