3GPP TSG WG RAN5 Meeting #101	R5-236135

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case 7.8B Intermodulation characteristics for category NB1 and NB2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 7.8B Intermodulation characteristics for category NB1 and NB2 is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.8B Intermodulation characteristics for category NB1 and NB2 was introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in R5-236140. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.8B	Intermodulation characteristics for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

7.8B.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.8B.3	Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 with parameters specified in Table 7.8B.3-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8B.3-1: Wide band intermodulation for category NB1 and NB2

| Parameters for wideband intermodulation |  |
| --- | --- |
| Category NB1 or NB2 Signal power | REFSENS + 12 dB |
| CW interferer signal power | - 46 dBm |
| 1.4 MHz E-UTRA interferer signal power | - 46 dBm |
| CW interferer offset | ± 2.2 MHz |
| 1.4 MHz E-UTRA interferer | ± 4.4 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 7.8B.

7.8B.4	Test description

7.8B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for category NB1 and NB2 in clause 5.2B. All of these configurations shall be tested with applicable test parameters, and are shown in Table 7.8B.4.1-1. The details of the downlink reference measurement channels (RMCs) are specified in Annex A.3. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 7.8B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | QPSK | 12 | BPSK | 1@0 | 15 kHz |

1.	Connect the SS to the UE antenna connector as shown in TS 36.508 [12] Annex A, Figure A.6 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channel is set according to Table 7.8B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other pre-configuration methods.

7.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the highest Satellite constellation (SAN class) supported by the UE by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31-NB configuration in TS 36.508 [12]. Test system shall send same SIB31-NB information during the test.

8.	Deactivate UE prediction of satellite trajectory by AT commands or any other pre-configuration methods.

9.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.8B.4.3.

7.8B.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.8B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

2.	Set the Downlink signal level according to the Table 7.8B.5-1.

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.5-1 and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D of the present document.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

7.8B.4.3	Message contents

Message contents are according to TS 36.508 [12] clause 8.1.6

7.8B.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Table 7.8B.5-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8B.5-1: Test parameters for Wide band intermodulation for category NB1 and NB2

| Parameters for wideband intermodulation |  |
| --- | --- |
| Category NB1 or NB2 Signal power | REFSENS + 12 dB |
| CW interferer signal power | - 46 dBm |
| 1.4 MHz E-UTRA interferer signal power | - 46 dBm |
| CW interferer offset [MHz] | -BW/2 – 2.1 / +BW/2 + 2.1 |
| 1.4 MHz E-UTRA interferer offset [MHz] | 2*CW interferer offset |

7.9	Spurious emissions

<< End of changes >>