3GPP TSG WG RAN5 Meeting #101	R5-236149

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | N/A | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of measurement uncertainties and test tolerances for test cases from 7.5 to 7.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Measurement uncertainties and test tolerances for test cases from 7.5 to 7.9 are undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define measurement uncertainties and test tolerances for test cases from 7.5 to 7.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases from 7.5 to 7.9 will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.4 Maximum input level | Same as clause 7.4 in TS 38.521-1 [2] for FDD band with f ≤ 3GHz. |  |
| 7.5 Adjacent channel selectivity7.5 Adjacent channel selectivity | Same as clause 7.5 in TS 38.521-1 [2] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.5 in TS 38.521-1 [2]”. |
| 7.6.2 In-band blocking | Same as clause 7.6.2 in TS 38.521-1 [2] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.6.2 in TS 38.521-1 [2]”. |
| 7.6.3 Out of Band Blocking | Same as clause 7.6.3 in TS 38.521-1 [2] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.6.3 in TS 38.521-1 [2]”. |
| 7.6.4 Narrow band blocking | Same as clause 7.6.4 in TS 38.521-1 [2] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.6.4 in TS 38.521-1 [2]”. |
| 7.7 Spurious response | Same as clause 7.6.3. | Same as clause 7.6.3. |
| 7.8.2 Wide band Intermodulation | Same as clause 7.8.2 in TS 38.521-1 [2] for FDD band with “f ≤ 3.0GHz”. | Same as clause 7.8.2 in TS 38.521-1 [2]”. |
| 7.9 Spurious emissions | Same as clause 7.8.2 in TS 38.521-1 [2] with “f ≤ 12.75 GHz” |  |

F.2	Interpretation of measurement results (normative)

<< Unchanged sections omitted >>

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.4 Maximum input level | Same as clause 7.4 in TS 38.521-1 [2] for FDD band with f ≤ 3GHz. | Maximum input level - TT |
| 7.5 Adjacent channel selectivity7.5 Adjacent channel selectivity | Same as clause 7.5 in TS 38.521-1 [2]”. | Same as clause 7.5 in TS 38.521-1 [2]”. |
| 7.6.2 In-band blocking | Same as clause 7.6.2 in TS 38.521-1 [2]”. | Same as clause 7.6.2 in TS 38.521-1 [2]”. |
| 7.6.3 Out of Band Blocking | Same as clause 7.6.3 in TS 38.521-1 [2]”. | Same as clause 7.6.3 in TS 38.521-1 [2]”. |
| 7.6.4 Narrow band blocking | Same as clause 7.6.4 in TS 38.521-1 [2]”. | Same as clause 7.6.4 in TS 38.521-1 [2]”. |
| 7.7 Spurious response | Same as clause 7.6.3. | Same as clause 7.6.3. |
| 7.8.2 Wide band Intermodulation | Same as clause 7.8.2 in TS 38.521-1 [2]”. | Same as clause 7.8.2 in TS 38.521-1 [2]”. |
| 7.9 Spurious emissions | Same as clause 7.9 in TS 38.521-1 [2]”. | Same as clause 7.9 in TS 38.521-1 [2]”. |

Annex G (informative):
Change history

<< End of changes >>