3GPP TSG-RAN WG5 Meeting #101                                           R5-236153

# Chicago, USA, 13 – 17 November 2023

Agenda Item:	5.4.17

Source: 	CAICT

Title: 	Discussion on test of test cases with no exception requirements

Document for:	Discussion and Endorsement

Introduction

At RAN5#100 meeting, there are different understandings on TS 38.521-3 and TS 38.522 about how to perform the test cases with non-exception requirements.

This contribution is trying to reach an agreement on the understanding and giving a way forward on how to correct TS 38.521-3 and TS 38.522.

Discussion

The following is from TS 38.521-3：

4.5	Applicability and test coverage rules

4.5.0	General

(1)	The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following:

a)	For each NR band in a device; test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the EN-DC or NE-DC FR2 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-2 [9] for each NR band. Test only one EN-DC or NE-DC combination per FR2 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of TS 38.101-3 [4] using LTE anchor agnostic approach.

c)	Test all the EN-DC or NE-DC FR1 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-1 [8] for each NR band. Test only one EN-DC or NE-DC combination per FR1 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of 38.101-3 [4] using LTE anchor agnostic approach.

(2)	The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)	For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. If Standalone FR2 cannot be tested (due to test case not being complete), then test in EN-DC or NE-DC mode following (1) b) above.

c)	Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested. If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), then test in EN-DC or NE-DC mode following (1)c) above.

The different understandings are for the above green highlighted part.

Understanding 1: There is only one scenario that Standalone cannot be tested which is because of reason of DUT (UE does not support the band in SA mode).

Understanding 2: There are two scenarios that Standalone cannot be tested which are because of non-DUT reason or because of DUT reason. Test of Standalone test case not being complete is non-DUT reason. UE does not support the band in SA mode is DUT reason. Test case not being complete is interpreted as test of Standalone test case not being complete. There are many non-DUT reasons that may lead to test of Standalone test case not being complete, e.g., TE for the Standalone test case is not available (not validated by certification organizations, or test lab only has TE of NSA test cases, or ....) In RAN5, test of non-exception requirements with LTE anchor agnostic approach is technically the same as the standalone test case, which means for the same UE, if one is tested, the other can be skipped.

The following is from the draft spec of TS 38.522 after RAN5 #100 meeting (z_38522-i00) for Standalone FR1 and Standalone FR2 test cases, which clearly shows that “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are with revision marks which means they were approved at RAN5 #100 meeting when the different understandings were bringing out. These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. (Not sure if there is still any company submitting TS 38.522 CRs with “Understanding 2” at this RAN5 #101 meeting).

Standalone FR1 test cases approved by RAN5 with “Understanding 2” are as bellow:

Standalone FR2 test cases approved by RAN5 with “Understanding 2” are as bellow:

Observation: “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are approved at RAN5 #100 meeting when the two different understandings were bringing out.

Conclusion: These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. Which means “Understanding 2” is quite a common understanding in RAN5 since the beginning.

Proposal: RAN5 shall endorse either the below proposal 1 or Proposal 2.

Proposal 1: RAN5 reach an agreement on “Understanding 1”. Check if there is still any company submitting TS 38.522 CRs with “Understanding 2” at this RAN5 #101 meeting. Revise section 4.5.0 of 38.521-3 and the 99 test cases with “Understanding 2” in TS 38.522 as per “Understanding 1” at next RAN5 meeting (RAN5 #102).

Proposal 2: RAN5 reach an agreement on “Understanding 2”. Revise section 4.5.0 of 38.521-3 as per “Understanding 2” at next RAN5 meeting (RAN5 #102) and continue submitting TS 38.522 CRs for test cases with non-exception requirements as per “Understanding 2”.

Conclusion

Understanding 1: There is only one scenario that Standalone cannot be tested which is because of reason of DUT (UE does not support the band in SA mode).

Understanding 2: There are two scenarios that Standalone cannot be tested which are because of non-DUT reason or because of DUT reason. Test of Standalone test case not being complete is non-DUT reason. UE does not support the band in SA mode is DUT reason. Test case not being complete is interpreted as test of Standalone test case not being complete. There are many non-DUT reasons that may lead to test of Standalone test case not being complete, e.g., TE for the Standalone test case is not available (not validated by certification organizations, or test lab only has TE of NSA test cases, or ....) In RAN5, test of non-exception requirements with LTE anchor agnostic approach is technically the same as the standalone test case, which means for the same UE, if one is tested, the other can be skipped.

Observation: “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are approved at RAN5 #100 meeting when the two different understandings were bringing out.

Conclusion: These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. Which means “Understanding 2” is quite a common understanding in RAN5 since the beginning.

Proposal: RAN5 shall endorse either the below proposal 1 or Proposal 2.

Proposal 1: RAN5 reach an agreement on “Understanding 1”. Check if there is still any company submitting TS 38.522 CRs with “Understanding 2” at this RAN5 #101 meeting. Revise section 4.5.0 of 38.521-3 and the 99 test cases with “Understanding 2” in TS 38.522 as per “Understanding 1” at next RAN5 meeting (RAN5 #102).

Proposal 2: RAN5 reach an agreement on “Understanding 2”. Revise section 4.5.0 of 38.521-3 as per “Understanding 2” at next RAN5 meeting (RAN5 #102) and continue submitting TS 38.522 CRs for test cases with non-exception requirements as per “Understanding 2”.

# References

- draft spec of TS 38.522 after RAN5 #100 meeting (z_38522-i00): NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases

- TS 38.521-3 v18.0.0: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios