3GPP TSG-RAN5 Meeting #101	R5-236178

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Source:	MediaTek Inc.

Title:	Discussion on handling of legacy cases over 8 mins

# Document for:	Endorsement

Agenda Item:	6.3.18.5

1.	Introduction

As the visibility duration of a NGSO satellite (LEO with orbit height 600 km) is about 8.5 minutes, the detail is described in TS 36.508 Cl. 6.3.5.2. However, there are legacy NB-IoT cases with test duration longer than 8 minutes as listed in below table.

| Clause | TC Title | Guard Time (s) |
| --- | --- | --- |
| 22.1.1.M3 | NB-IoT / Control Plane CIoT EPS optimisation for EPS services - Module 3 | 600 |
| 22.2.4 | NB-IoT / Cell selection / Qrxlevmin and Qqualmin / Serving cell becomes non-suitable (S<0 or barred or Srxlev > 0 and Squal < 0) | 1500 |
| 22.4.1 | NB-IoT / Notification of BCCH modification in idle mode / eDRX cycle longer than the modification period | 30300 |
| 22.4.4 | NB-IoT / RRC connection establishment / Paging / Access Barring for UE with AC 0 to 9 / ab-Category a, b and c | 3000 |
| 22.4.5 | NB-IoT / RRC connection establishment / Paging / Access Barring for UE with AC 11 to 15 / ab-Category a, b and c | 3000 |
| 22.4.8 | NB-IoT / RRC connection establishment / Access Barring for UE with AC 0 to 9 / MO exception data / ab-Category a, b and c | 3000 |
| 22.4.9 | NB-IoT / RRC connection establishment / Access Barring for UE with AC 11 to 15 / MO exception data / ab-Category a, b and c | 3000 |
| 22.4.14 | NB-IoT / RRC Connection Establishment / Multi-Carrier | 3600 |
| 22.5.1 | NB-IoT / Authentication not accepted by the network, GUTI used / Authentication not accepted by the UE, SQN failure / Authentication not accepted by the UE, non-EPS authentication unacceptable / Network failing the authentication check | 2400 |
| 22.5.3 | NB-IoT / NW initiated detach Re-attach required / UE initiated detach Abnormal case EMM common procedure collision / UE initiated detach Abnormal case Local detach after 5 attempts due to no network response | 3000 |
| 22.5.4 | NB-IoT / Attach to new PLMN IMSI / Network reject with Extended Wait Timer / Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off | 2700 |
| 22.5.6 | NB-IoT / Attach Abnormal cases / Unsuccessful attach or Repeated rejects for network failures / Change of cell into a new tracking area / EPS services not allowed / Failure due to non integrity protection /UE initiated detach USIM removed from the UE / Detach procedure collision | 3600 |
| 22.5.9 | NB-IoT / UE in NB-S1 mode supporting CIoT Optimizations / Paging with not matching identity / Control Plane Service request Rejected (IMSI invalid / Illegal ME / EPS services not allowed / UE identity cannot be derived by the network / UE implicitly detached) | 1800 |
| 22.5.17 | NB-IoT / Attach Success /Normal tracking area update accepted / Periodic tracking area update T3412 Extended Value / PSM | 1200 |
| 22.5.18 | NB-IoT / Attach & Normal tracking area update Procedure / Success / without Idle eDRX parameters / With Idle eDRX parameters/ With and without Idle eDRX and PSM parameters | 1200 |
| 22.6.1a | NB-IoT / UE routing of uplinks packets / Control Plane | 1800 |

2.	Discussion

For above cases, when testing under NGSO scenario the satellite (serving cell) will move away and become invisible after 8 mins. This will cause the cases stopped or failed as unexpected.

As it is difficult to determine the UE state and the ongoing procedure (either normal or abnormal) before the satellite fading away due to no specific timing in each step. If the UE is in connected mode, as there is no handover procedure for NB-IoT, UE will enter Idle mode and the case will not be able to continue. If the UE is in Idle mode and there is available neighbour cell, the UE may perform cell reselection. But this needs to update the test configuration and test procedure a lot.

To solve the problems caused by limited visibility window, there are some options as below:

Option 1: set these cases inapplicable to NGSO.

Option 2: using special test mode with same ephemeris information broadcasted in SIB31-NB.

For the 1st option, it may narrow down the test coverage of legacy NB-IoT cases for NGSO.

For the 2nd option, this refers to the specific test conditions as mentioned in RAN4 reply LS [1] and [2]. Below is an example from the LS:

Q2d: Are the zero Doppler or time varying assumptions applicable for conformance testing of demod performance requirements in section 8 in TS 38.101-5 and 36.102?

Q2d: In previous LS response (R4-2314001), it was indicated that zero orbit Doppler conditions related to satellite motion for DL in service link were applicable to demodulation cases in section 8 in TS 38.101-5 and TS 36.102. However, demodulation requirements were specified with TDL channel model which implemented a separate Doppler shift into channel model.

As a conclusion of RAN4#108bis, RAN4 would like to add the following clarifications regarding conformance demod test cases:

- Delay will be constant.

  - Constant delay value to be derived from the ephemeris (i.e., orbit emulation) for the zero orbit Doppler condition.

- The same ephemeris info will be maintained by the TE during each demod measurement.

So, the same ephemeris information is used to maintain the constant Doppler and/or delay. This means that the SS broadcasts same SIB31-NB during the test and UE needs to enter special test mode to deactivate prediction of satellite trajectory in the preamble. See more details in [3].

Considering the lack of test coverage for option 1, option 2 is suggested to be used for any cases longer than 8 minutes and this is also the solution used to NTN RF test.

Proposal #1: RAN5 to approve the usage of special test mode with same ephemeris information to be broadcasted during the test of legacy applicable NB-IoT cases for NGSO scenario.

3.	Proposal/Way Forward

In the contribution, we discuss the handling of legacy NB-IoT cases over 8 mins with following proposal:

Proposal #1: RAN5 to approve the usage of special test mode with same ephemeris information to be broadcasted during the test of legacy applicable NB-IoT cases for NGSO scenario.

4.	References

[1] 	R4-2314001: Reply LS on clarifications for Non-Terrestrial Networks, Keysight

[2] 	R4-2317006: Further Reply LS on clarifications for Non-Terrestrial Networks, Keysight

[3] 	R5-236158: Discussion on addition of AT command for NTN test, MediaTek Inc.

[4] 	R5-237090: Update of legacy NB-IoT TC for NGSO, MediaTek Inc.