3GPP TSG-RAN5 Meeting #101	R5-236236

# Chicago, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4017 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.1.3.6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UDC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the test case 7.1.3.6.4 with non used extension group in test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The non used extension group has been removed. Specific message path has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains inconsistent and includes non used ASN.1 fields. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

##### 7.1.3.6.4	PDCP UDC/ Handover/ Intra-frequency

7.1.3.6.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency handover and drb-ContinueUDC is not configured }

then { UE maintains its previous UDC configuration for the DRB and resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency handover and drb-ContinueUDC is configured }

then { UE maintains its previous UDC configuration for the DRB and continues to use the previous UDC buffer }

}

7.1.3.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.1.2, TS 38.331, clauses 5.3.5.6.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

[TS 38.331, clause 5.3.5.6.5]

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

7.1.3.6.4.3	Test description

7.1.3.6.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured with DRBs in RLC AM mode.

-	The cell power levels are configured as per the Table 7.1.3.6.4.3.1-1.

Table 7.1.3.6.4.3.1-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 |  |

Table 7.1.3.6.4.3.1-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | 91 |  |

7.1.3.6.4.3.2	Test procedure sequence

Table 7.1.3.6.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | UE sends the PDCP Data PDU#0 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | UE sends the PDCP Data PDU#1 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 6 | The SS changes NR Cell 2 parameters according to the row "T1" in table 7.1.3.6.4.3.1-1(FR1) / 7.1.3.6.4.3.1-2(FR2). |  |  |  |  |
| 7 | The SS transmits NR RRCReconfiguration message to perform SpCell change from NR Cell1 to NR Cell 2 | <-- | RRCReconfiguration | - | - |
| 8 | UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 10 | CHECK: Does UE send the PDCP Data PDU#2 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 1, Checksum = 1111? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| 11 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 12 | UE sends the PDCP Data PDU#3 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 3, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #3 after decompression. (Note 1) |  | PDCP PDU | - | - |
| 13 | SS adjusts the cell-specific reference signal level according to row "T2". |  |  |  |  |
| 14 | The SS transmits NR RRCReconfiguration message to perform SpCell change from NR Cell 2 to NR Cell 1 | <-- | RRCReconfiguration | - | - |
| 15 | UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 16 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 17 | CHECK: Does UE send the PDCP Data PDU#4 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 4, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.4.3.3	Specific message contents

Table 7.1.3.6.4.3.3-1: RRCReconfiguration (steps 7)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-2: RadioBearerConfig (Table 7.1.3.6.4.3.3-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-3: Void

Table 7.1.3.6.4.3.3-4: CellGroupConfig (7.1.3.6.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 2 |  |  |
| } |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| Uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-5: RRCReconfiguration (steps 14)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-6: RadioBearerConfig (Table 7.1.3.6.4.3.3-5)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-7: PDCP-Config (Table 7.1.3.6.4.3.3-6)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| drb-ContinueUDC | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-8: CellGroupConfig (7.1.3.6.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| rach-ConfigDedicated | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>