3GPP TSG-RAN WG5 Meeting #101	R5-236257

Chicago, USA, 13th – 17th Nov, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0165 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicabilities for Release-17 PRS-RSRP and PRS-RSRPP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test applicabilities for Release-17 NR PRS-RSRP and PRS-RSRPP positioning test cases need to be introduced - The test applicabilities of TC 14.2.9 and TC15.2.9 are incorrect. The UE supporting any one of the PPW type could conduct the test cases without mwasurement gaps. - The test applicabilitiy condition and comment of TC 16.2.1, TC 16.2.2 and 16.3.1 are in the wrong column |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added test applicabilities for PRS-RSRP reporting delay and accuracy test cases - Added test applicabilities for PRS-RSRPP reporting delay test cases - Updated the test applicabilities of TC14.2.9 and TC15.2.9 - Updated test applicabilitiy condition and comment of TC 16.2.1, TC 16.2.2 and 16.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-17 NR positioning methods performance test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4-11, Table 4-12, Table A.4.3-6E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-1 CR 0438 TS 37.571-1 CR 0439 TS 37.571-1 CR 0440 TS 37.571-1 CR 0441 TS 37.571-1 CR 0442 TS 37.571-1 CR 0443 TS 37.571-1 CR 0444 TS 37.571-1 CR 0445 TS 37.571-1 CR 0446 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 4-11: Applicability of tests and additional information for testing for test cases in TS 37.571-1 [5] for NR

| Clause | TC Title | Release of LPP | Applicability |  | Additional Information |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment | Specific ICS | Specific IXIT | Number of TC Executions |  | Release RAT |
| 14 | NR RSTD measurement requirements |  |  |  |  |  |  |  |  |
| 14.2.1 | NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.2 | NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.3 | NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.4 | NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.2.8 | NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C54nr | All FR2 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.9_1s | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 1 | Rel-17 | C55nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 14.2.9_2s | NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 2 | Rel-17 | C56nr | All FR2 NR UEs. The UEs shall support DL-TDOA and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.2.10 | NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Rel-17 | C57nr | All FR2 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 14.3.1 | NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.2 | NR RSTD measurement accuracy test case for dual positioning frequency layer in FR1 SA | Rel-16 | C48nr | All FR1 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.3 | NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.4 | NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Rel-16 | C49nr | All FR2 NR UEs. The UEs shall support DL-TDOA |  |  |  |  | Rel-16 |
| 14.3.7 | NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C54nr | All FR2 NR UEs. The UEs shall support DL-TDOA and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 14.3.8 | NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Rel-17 | C57nr | All FR2 NR UEs. The UEs shall support UE assisted DL-TDOA and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15 | UE Rx-Tx time difference measurement requirements |  |  |  |  |  |  |  |  |
| 15.2.1 | UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.2 | UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.3 | UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.4 | UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.2.8 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C58nr | All FR2 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.9_1s | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 1 | Rel-17 | C59nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow types |  |  |  |  | Rel-17 |
| 15.2.9_2s | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 2 | Rel-17 | C60nr | All FR2 NR UEs. The UEs shall support Multi-RTT and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type sand measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.2.10 | UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA | Rel-17 | C61nr | All FR2 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 15.3.1 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Rel-16 | C46nr | All FR1 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.3.2 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layers in FR2 SA | Rel-16 | C47nr | All FR2 NR UEs. The UEs shall support Multi-RTT |  |  |  |  | Rel-16 |
| 15.3.5 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C58nr | All FR2 NR UEs. The UEs shall support UE assisted Multi-RTT and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 15.3.6 | UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Rel-17 | C61nr | All FR2 NR UEs. The UEs shall support Multi-RTT and nr-UE-RxTEG-ID-MaxSupport |  |  |  |  | Rel-17 |
| 16 | NR PRS-RSRP measurement requirements |  |  |  |  |  |  |  |  |
| 16.2.1 | PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Rel-16 | C53nr | All FR1 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.2.2 | PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Rel-16 | C53nr | All FR1 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.2.3 | PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Rel-16 | C50nr | All FR2 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.2.4 | PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Rel-16 | C50nr | All FR2 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.2.7 | RS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA | Rel-17 | C62nr | All FR2 NR UEs. The UEs shall support DL-AoD and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 16.2.8_1s | PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 1 | Rel-17 | C63nr | All FR2 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type |  |  |  |  | Rel-17 |
| 16.2.8_2s | PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA: Subtest 2 | Rel-17 | C64nr | All FR2 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 16.3.1 | PRS-RSRP measurement accuracy with PRS in FR1 | Rel-16 | C53nr | All FR1 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.3.2 | PRS-RSRP measurement accuracy with PRS in FR2 | Rel-16 | C50nr | All FR2 NR UEs. The UEs shall support DL-AoD |  |  |  |  | Rel-16 |
| 16.3.4 | PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Rel-17 | C62nr | All FR2 NR UEs. The UEs shall support DL-AoD and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 17 | NR PRS-RSRPP measurement requirements |  |  |  |  |  |  |  |  |
| 17.2.1 | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Rel-17 | C65nr | All FR1 NR UEs. The UEs shall support DL-AoD and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  | Rel-17 |
| 17.2.2 | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Rel-17 | C66nr | All FR1 NR UEs. The UEs shall support DL-AoD and maxDL-PRS-FirstPathRSRP-MeasPerTRP and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 17.2.3_1s | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Sub-test 1 | Rel-17 | C67nr | All FR1 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  | Rel-17 |
| 17.2.3_2s | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA: Sub-test 2 | Rel-17 | C68nr | All FR1 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type and measurements based on reduced number of samples of a DL-PRS Resource Set and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  |  |
| 17.2.4 | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Rel-17 | C69nr | All FR2 NR UEs. The UEs shall support DL-AoD and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  | Rel-17 |
| 17.2.5 | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Rel-17 | C70nr | All FR2 NR UEs. The UEs shall support DL-AoD and maxDL-PRS-FirstPathRSRP-MeasPerTRP and measurements based on reduced number of samples of a DL-PRS Resource Set |  |  |  |  | Rel-17 |
| 17.2.6_1s | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Rel-17 | C71nr | All FR2 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  | Rel-17 |
| 17.2.6_2s | PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Rel-17 | C72nr | All FR2 NR UEs. The UEs shall support DL-AoD and DL-PRS Processing Capability outside MG and one of prs-ProcessingWindow type and measurements based on reduced number of samples of a DL-PRS Resource Set and maxDL-PRS-FirstPathRSRP-MeasPerTRP |  |  |  |  |  |
| Note 1: If the signal type for BDS supported by the UE includes B1C then Rel-16 of LPP is required. If the signal type for BDS supported by the UE includes B2a and/or B3I then Rel-17 of LPP is required. |  |  |  |  |  |  |  |  |  |

Table 4-12: Applicability of tests Conditions for RAT-independent test cases in TS 37.571-1 [5] for NR

| C01nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 THEN R ELSE N/A |
| --- |
| C02nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 THEN R ELSE N/A |
| C03nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 THEN R ELSE N/A |
| C04nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 THEN R ELSE N/A |
| C05nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 THEN R ELSE N/A |
| C06nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND A.4.3-2/3 THEN R ELSE N/A |
| C07nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A |
| C08nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A |
| C09nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND A.4.3-2/3 THEN R ELSE N/A |
| C10nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 AND A.4.3-2/3 THEN R ELSE N/A |
| C11nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 THEN R ELSE N/A |
| C12nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 THEN R ELSE N/A |
| C13nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A |
| C14nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 AND A.4.3-2/3 THEN R ELSE N/A |
| C15nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 THEN R ELSE N/A |
| C16nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 AND A.4.3-2/3 THEN R ELSE N/A |
| C17nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 THEN R ELSE N/A |
| C18nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 AND A.4.3-2/3 THEN R ELSE N/A |
| C19nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/6 THEN R ELSE N/A |
| C20nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/7 THEN R ELSE N/A |
| C21nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/9 THEN R ELSE N/A |
| C22nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/8 THEN R ELSE N/A |
| C23nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/36 THEN R ELSE N/A |
| C24nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/37 THEN R ELSE N/A |
| C25nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/18 THEN R ELSE N/A |
| C26nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/38 THEN R ELSE N/A |
| C27nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 THEN R ELSE N/A |
| C28nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 THEN R ELSE N/A |
| C29nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 AND A.4.3-2/3 THEN R ELSE N/A |
| C30nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/2 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 AND A.4.3-2/3 THEN R ELSE N/A |
| C31nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/40 THEN R ELSE N/A |
| C32nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/41 THEN R ELSE N/A |
| C33nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.1-4/1 OR A.4.1-4/2) AND ((A.4.3-2/1 AND NOT A.4.3-2/24) OR (A.4.3-2/2 AND NOT A.4.3-2/25)) AND A.4.3-2/39 THEN R ELSE N/A |
| C34nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/6 AND A.4.3-2/3 THEN R ELSE N/A |
| C35nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/7 AND A.4.3-2/3 THEN R ELSE N/A |
| C36nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/9 AND A.4.3-2/3 THEN R ELSE N/A |
| C37nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/8 AND A.4.3-2/3 THEN R ELSE N/A |
| C38nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/36 AND A.4.3-2/3 THEN R ELSE N/A |
| C39nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/37 AND A.4.3-2/3 THEN R ELSE N/A |
| C40nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/18 AND A.4.3-2/3 THEN R ELSE N/A |
| C41nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/38 AND A.4.3-2/3 THEN R ELSE N/A |
| C42nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/39 AND A.4.3-2/3 THEN R ELSE N/A |
| C43nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/40 AND A.4.3-2/3 THEN R ELSE N/A |
| C44nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.1-4/1 AND (A.4.3-2/1 OR A.4.3-2/2) AND A.4.3-2/41 AND A.4.3-2/3 THEN R ELSE N/A |
| C45nr IF A.4.1-1/6 AND A.4.3-2/20 THEN R ELSE N/A |
| C46nr IF (A.4.1-1/6 AND A.4.1-5/1) AND A.4.3-2/29 THEN R ELSE N/A |  |
| C47nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 THEN R ELSE N/A |  |
| C48nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A |  |
| C49nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A |  |
| C50nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) THEN R ELSE N/A |  |
| C51nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A |  |
| C52nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) THEN R ELSE N/A |  |
| C53nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) THEN R ELSE N/A |  |
| C54nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 THEN R ELSE N/A |  |
| C55nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C56nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/32 OR A.4.3-2/33) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C57nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/32 AND A.4.3-6G/1 THEN R ELSE N/A |  |
| C58nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6B/23 THEN R ELSE N/A |  |
| C59nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C60nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C61nr IF (A.4.1-1/6 AND A.4.1-5/2) AND A.4.3-2/29 AND A.4.3-6G/1 THEN R ELSE N/A |  |
| C62nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 THEN R ELSE N/A |  |
| C63nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C64nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C65nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 THEN R ELSE N/A |  |
| C66nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 THEN R ELSE N/A |  |
| C67nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C68nr IF (A.4.1-1/6 AND A.4.1-5/1) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C69nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 THEN R ELSE N/A |  |
| C70nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 THEN R ELSE N/A |  |
| C71nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |
| C72nr IF (A.4.1-1/6 AND A.4.1-5/2) AND (A.4.3-2/30 OR A.4.3-2/31) AND A.4.3-6E/10 AND A.4.3-6B/23 AND A.4.3-2/48 AND (A.4.3-2/49 OR A.4.3-2/50 OR A.4.3-2/51) THEN R ELSE N/A |  |

< Unchanged sections omitted >

Table A.4.3-6E: NR DL-AoD Measurements

| Item | NR DL-AoD Measurements | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | maxDL-PRS-RSRP-MeasurementFR1 | 37.355, 6.5.11.6 | Rel-16 | pc_maxDL_PRS_RSRP_MeasurementFR1 |  |
| 2 | maxDL-PRS-RSRP-MeasurementFR2 | 37.355, 6.5.11.6 | Rel-16 | pc_maxDL_PRS_RSRP_MeasurementFR2 |  |
| 3 | Support of simul-NR-DL-AoD-DL-TDOA for at least one NR band | 37.355, 6.5.11.6 | Rel-16 | pc_simul_NR_DL_AoD_DL_TDOA |  |
| 4 | Support of simul-NR-DL-AoD-Multi-RTT for at least one NR band | 37.355, 6.5.11.6 | Rel-16 | pc_simul_NR_DL_AoD_Multi_RTT |  |
| 5 | Support of periodicalReporting in standalone mode | 37.355, 6.5.11.6 | Rel-16 | pc_DL_AoD_periodicalReporting_standalone |  |
| 6 | Support of periodicalReporting in UE-based mode | 37.355, 6.5.11.6 | Rel-16 | pc_DL_AoD_periodicalReporting_UEB |  |
| 7 | Support of periodicalReporting in UE-assisted mode | 37.355, 6.5.11.6 | Rel-16 | pc_DL_AoD_periodicalReporting_UEA |  |
| 8 | Support of mg-ActivationRequest for DL-AoD positioning method | 37.355, 6.5.12.6 | Rel-17 | pc_mg_ActivationRequest_DL_AoD |  |
| 9 | posMeasGapSupport for DL-AoD positioning method | 37.355, 6.5.12.6 | Rel-17 | pc_posMeasGapSupport_DL_AoD |  |
| 10 | maxDL-PRS-FirstPathRSRP-MeasPerTRP | 37.355, 6.5.11.6a | Rel-17 | pc_maxDL_PRS_FirstPathRSRP_MeasPerTRP |  |