3GPP TSG-RAN WG5 Meeting #101	R5-236258

Chicago, USA, 13th – 17th Nov, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0166 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PRS processing window type PICs description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of PPW type PICS is inconsistent with the core specification. The related test applicabilities are not implementable |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the description of PPW type PICS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicability can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table A.4.3-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Unchanged sections omitted >

Table A.4.3-2: E-UTRA and NR UE Positioning Capabilities

| Item | E-UTRA and NR UE Positioning Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of UE based Assisted-GNSS | 36.355 | Rel-9 | pc_UEB_AGNSS | This implies support of LPP A.4.2-1/1 |
| 2 | Support of UE assisted Assisted-GNSS | 36.355 | Rel-9 | pc_UEA_AGNSS | This implies support of LPP A.4.2-1/1 |
| 3 | Support of GNSS Fine Time Assistance | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA | This implies support of LPP A.4.2-1/1 |
| 4 | Support of UE assisted OTDOA | 36.355 | Rel-9 | pc_OTDOA | This implies support of LPP A.4.2-1/1 |
| 5 | Support of UE assisted ECID | 36.355 | Rel-9 (FDD) Rel-13 (TDD) (NOTE 2) | pc_ECID | This implies support of LPP A.4.2-1/1 |
| 6 | Support for A-GPS L1C/A | 36.355 | Rel-9 | pc_A_GPS_L1C_A | This implies support of LPP A.4.2-1/1 |
| 7 | Support for A-GLONASS | 36.355 | Rel-9 | pc_A_GLONASS | This implies support of LPP A.4.2-1/1 |
| 8 | Support for A-GPS L1C/A and Modernized GPS | 36.355 | Rel-9 | pc_A_GPS_L1C_A_MGPS | This implies support of LPP A.4.2-1/1 |
| 9 | Support for A-Galileo | 36.355 | Rel-12 | pc_A_Galileo | This implies support of LPP A.4.2-1/1 (NOTE 1) |
| 10 | Support of UE Fine Time Assistance measurements for UE-based Assisted-GNSS | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA_UEB | This implies support of LPP A.4.2-1/1 |
| 11 | Support of UE Fine Time Assistance measurements for UE-assisted Assisted-GNSS | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA_UEA | This implies support of LPP A.4.2-1/1 |
| 12 | Support of GNSS Acquisition Assistance | 36.355; 37.571-2, 5.4.1 | Rel-9 | pc_GNSS_AA | This implies support of LPP A.4.2-1/1 |
| 13 | Support for A-SBAS | 36.355 | Rel-9 | pc_A_SBAS |  |
| 14 | Support for A-QZSS | 36.355 | Rel-9 | pc_A_QZSS |  |
| 15 | Support of UE assisted OTDOA for Carrier Aggregation | 36.355 | Rel-10 | pc_OTDOA_CA | This implies support of LPP A.4.2-1/1 |
| 16 | Support of inter-frequency RSTD measurements that require measurement gaps | 36.355 | Rel-10 | pc_InterFreq_RSTD_withGaps | This implies support of UE assisted OTDOA A.4.3-2/4 |
| 17 | Support of inter-frequency RSTD measurements | 36.355 | Rel-10 | pc_InterFreq_RSTD | This implies support of UE assisted OTDOA A.4.3-2/4 |
| 18 | Support for A-BDS | 36.355 | Rel-12 | pc_A_BDS | This implies support of LPP A.4.2-1/1 (NOTE 3) |
| 19 | Support of UE assisted OTDOA for 3DL Carrier Aggregation | 36.355 | Rel-12 | pc_OTDOA_3DLCA | This implies support of LPP A.4.2-1/1 |
| 20 | Support for UE-Assisted MBS | 36.355 | Rel-13 | pc_UEA_MBS | This implies support of LPP A.4.2-1/1 |
| 21 | Support for UE-Assisted WLAN | 36.355 | Rel-13 | pc_WLAN | This implies support of LPP A.4.2-1/1 |
| 22 | Support for UE-Assisted Bluetooth | 36.355 | Rel-13 | pc_Bluetooth | This implies support of LPP A.4.2-1/1 |
| 23 | Support for UE-Assisted Sensor | 36.355 | Rel-13 | pc_Sens | This implies support of LPP A.4.2-1/1 |
| 24 | No support of periodical reporting for UE based Assisted-GNSS. | 36.355 | Rel-14 | pc_UEB_Noperiodic | This implies support of LPP A.4.2-1/1 |
| 25 | No support of periodical reporting for UE assisted Assisted-GNSS. | 36.355 | Rel-14 | pc_UEA_Noperiodic | This implies support of LPP A.4.2-1/1 |
| 26 | Support for UE-Based MBS | 36.355 | Rel-14 | pc_UEB_MBS | This implies support of LPP A.4.2-1/1 |
| 27 | Support for UE-Based WLAN | 36.355 | Rel-14 | pc_UEB_WLAN | This implies support of LPP A.4.2-1/1 |
| 28 | Support for UE-Based Sensor | 36.355 | Rel-14 | pc_UEB_Sens | This implies support of LPP A.4.2-1/1 |
| 29 | Support for UE-Assisted Multi-RTT | 37.355 | Rel-16 | pc_Multi_RTT | This implies support of LPP A.4.2-1/1 |
| 30 | Support for UE-Assisted DL-AoD | 37.355 | Rel-16 | pc_UEA_DL_AoD | This implies support of LPP A.4.2-1/1 |
| 31 | Support for UE-Based DL-AoD | 37.355 | Rel-16 | pc_UEB_DL_AoD | This implies support of LPP A.4.2-1/1 |
| 32 | Support for UE-Assisted DL-TDOA | 37.355 | Rel-16 | pc_UEA_DL_TDOA | This implies support of LPP A.4.2-1/1 |
| 33 | Support for UE-Based DL-TDOA | 37.355 | Rel-16 | pc_UEB_DL_TDOA | This implies support of LPP A.4.2-1/1 |
| 34 | Support for UE-Assisted NR E-CID | 37.355 | Rel-16 | pc_NR_ECID | This implies support of LPP A.4.2-1/1 |
| 35 | Support for NR UL-SRS-Capability | 37.355 | Rel-16 | pc_NR_UL_SRS_Capability | This implies support of LPP A.4.2-1/1 |
| 36 | Support for GPS L1 C/A and GLONASS | 36.355 | Rel-9 | pc_A_GPS+GLONASS | This implies support of LPP A.4.2-1/1 |
| 37 | Support for GPS L1 C/A and Galileo | 36.355 | Rel-12 | pc_A_GPS+Galileo | This implies support of LPP A.4.2-1/1. (NOTE 1) |
| 38 | Support for GPS L1 C/A and BDS | 36.355 | Rel-12 | pc_A_GPS+BDS | This implies support of LPP A.4.2-1/1 (NOTE 3) |
| 39 | Support for GPS L1 C/A, GLONASS and BDS | 36.355 | Rel-12 | pc_A_GPS+GLONASS+BDS | This implies support of LPP A.4.2-1/1 (NOTE 3) |
| 40 | Support for GPS L1 C/A, GLONASS and Galileo | 36.355 | Rel-12 | pc_A_GPS+GLONASS+Galileo | This implies support of LPP A.4.2-1/1. (NOTE 1) |
| 41 | Support for GPS L1 C/A, BDS and Galileo | 36.355 | Rel-12 | pc_A_GPS+Galileo+BDS | This implies support of LPP A.4.2-1/1. (NOTE 1) (NOTE 3) |
| 42 | Support of UE assisted OTDOA (based on LTE signals) on NR | 37.355 | Rel-15 | pc_OTDOA_onNR | This implies support of LPP A.4.2-1/1 |
| 43 | Support of UE assisted ECID (based on LTE signals) on NR | 37.355 | Rel-15 (FDD) Rel-13 (TDD) (NOTE 2) | pc_ECID_onNR | This implies support of LPP A.4.2-1/1 |
| 44 | Support for UE-Assisted UL-TDOA | 37.355 | Rel-17 | pc_UEA_UL_TDOA | This implies support of LPP A.4.2-1/1 |  |
| 45 | Support for low latency measurement gap activation request for DL-PRS measurements. | 37.355 | Rel-17 | pc_mg_ActivationRequest | This implies support of LPP A.4.2-1/1. The UE can include this field only if the UE supports pc_mg_ActivationCommPRS_Meas and pc_mg_ActivationRequestPRS_Meas |  |
| 46 | Support for preconfiguration of MGs in RRC signalling for PRS measurements and the use of DL MAC CE from the gNB to activate/deactivate the preconfigured MG for PRS measurements. | 38.306 | Rel-17 | pc_mg_ActivationCommPRS_Meas |  |  |
| 47 | Support for preconfiguration of MGs in RRC signalling for PRS measurements and supports the use of UL MAC CE to request the activation/deactivation of the preconfigured MG for PRS measurements. | 38.306 | Rel-17 | pc_mg_ActivationRequestPRS_Meas | The UE can include this field only if the UE supports pc_mg_ActivationCommPRS_Meas |  |
| 48 | Support for DL-PRS Processing Capability outside MG | 37.355 | Rel-17 | pc_prs_ProcessingCapabilityOutsideMGinPPW | This implies support of LPP A.4.2-1/1 |  |
| 49 | Support one of option1, option2 or option3 priority options of prs-ProcessingWindowType1A | 37.355 | Rel-17 | pc_prs_ProcessingWindowType1A | This implies support of LPP A.4.2-1/1 |  |
| 50 | Support one of option1, option2 or option3 priority options of prs-ProcessingWindowType1B | 37.355 | Rel-17 | pc_prs_ProcessingWindowType1B | This implies support of LPP A.4.2-1/1 |  |
| 51 | Support one of option1, option2 or option3 priority options of prs-ProcessingWindowType2 | 37.355 | Rel-17 | pc_prs_ProcessingWindowType2 | This implies support of LPP A.4.2-1/1 |  |
| NOTE 1: Non-backwards compatible changes were made to the Galileo Assistance Data in LPP Rel-12, therefore testing cannot be done for earlier releases. NOTE 2: For TDD with LPP releases before Rel-13 the UE Rx - Tx time difference measurement report mapping is ambiguous and therefore testing shall not be performed. NOTE 3: If the signal type for BDS supported by the UE includes B1C then Rel-16 of LPP is required. If the signal type for BDS supported by the UE includes B2a and/or B3I then Rel-17 of LPP is required. |  |  |  |  |  |