3GPP TSG-RAN5 Meeting #101	R5-236298

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4770 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN-IoT: NB-IoT Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The general NB-IoT NTN test environment lacks details in the following areas: NTN timing & scheduling offsets. 2. The NB-IoT NTN test environment in NGSO scenario is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Figure 7A.14.2.1-1 has been replaced, description of NTN scheduling offsets clause has been completed, new NTN timing parameters have been defined for the purpose of NTN test case prose specification, NTN impact to the RRC connection release sequence has been documented. 2. The NB-IoT NTN test environment in NGSO scenario has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The NB-IoT NTN test environment remains incomplete. 2. The NB-IoT NTN test cases cannot be executed in NGSO scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.4, 7A.4.3, 7A.4.4, 7A.7, 7A.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7A.4	Timing considerations

Based on SIB scheduling and search space configuration as described in the previous clauses timing considerations are made as below, with the timing reference being the absolute time at the SS Rx/Tx antenna.

### 7A.4.1	Random access procedure

Details regarding random access procedure in NB-IoT are specified in clause 16.3 of TS 36.213 [30] and clause 5.1 of TS 36.321 [16]. The timing of a random access procedure in NB-IoT is illustrated in table 7A.4.1-1.

Table 7A.4.1-1: Timing of random access procedure

| ... | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |  | ... |  |  | ... |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | Type2 CSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |  |  | Type2 CSS |  |  | ... |
| RACH preamble |  |  |  | DCI format N1 |  |  |  |  |  | RAR with UL grant |  |  |  |  |  |  |  |  | Msg3 | ... | DCI format N0/1 |  |  |  |
| ... | tdelay1 |  |  | tDCI |  | tdelay2 |  |  |  | tRAR |  | tdelay3 |  |  |  |  |  |  | tMsg3 | tdelay4 | tDCI |  | tdelay5 |  |
| step 1 |  |  |  | step 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | step 3 |  | step 4 |  |  | ... |

with

| steps 1-4 | Random access procedure according to TS 36.300 [69] clause 10.1.5.1: step 2: RAR is addressed to RA-RNTI step 3: Msg3 contains either a CCCH SDU (a) or the C-RNTI (b) step 4: a) temporary C-RNTI based contention resolution (DL assignment for Msg4) b) C-RNTI based contention resolution (UL grant) NOTE: According to TS 36.213 [30] clause 16.5.1, Table 16.5.1-4 and clause 16.4.1, Table 16.4.1-5 the DCI for step 4 is sent on NPDCCH in the Type2 common search space |  |
| --- | --- | --- |
| tdelay1 | 3ms (but up to 50ms) | depends on Type2-CSS configuration, start of RA response window according to TS 36.321 [16] clause 5.1.4 and when the RACH preamble happens |
| tDCI | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| tdelay2 | 4ms | scheduling delay according to DCI (TS 36.213 [30] clause 16.4.1 and table 16.4.1-1) |
| tRAR | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.4.1) |
| tdelay3 | 12ms | scheduling delay according to RAR grant (TS 36.213 [30] clause 16.3.3) |
| tMsg3 | 1ms | depending on MSC index and number of repetitions according to RAR grant; the TBS is 88 bits in all cases (TS 36.213 [30] clause 16.3.3) |
| tdelay4 | 3ms | delay before the UE monitors NPDCCH again (TS 36.213 [30] clause 16.6) |
| tdelay5 | 13ms | depending on whether UL or DL transmission follows, see clause 7A.4.2 and 7A.4.3 |

NOTE 1:	In addition to the delay in table 7A.4.1-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general a random access procedure needs more than two search space cycles.

### 7A.4.2	Uplink transmissions

Table 7A.4.2-1: Timing of uplink transmissions

| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | DCI format N0 |  |  |  |  |  |  |  |  |  | UL-SCH |  |  |  |  | ACK/NACK |  | ... |
|  | tDCI1 |  | tdelay1 |  |  |  |  |  |  |  | tData |  | tdelay2 |  |  | tDCI2 |  |  |

with

| tDCI1 | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| --- | --- | --- |
| tdelay1 | 8ms | according to TS 36.213 [30] table 16.5.1-1 |
| tData | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.5.1 and TS 36.211 Table 10.1.2.3-1) (NOTE 4) |
| tdelay2 | 3ms | according to TS 36.213 [30] clause 16.5.2 |
| tDCI2 | 2ms | (same as tDCI1) |

NOTE 1:	In addition to the delay in table 7A.4.2-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general an uplink transmission takes one search space cycle.

NOTE 3:	Even when in case of necessary UL re-transmission the grant for the re-transmission is sent immediately using adaptive HARQ NACK, it is hardly predictable how many search space cycles the complete successful transmission takes. Nevertheless one re-transmission may still be finished in the same search space cycle in which the initial UL has been scheduled.

NOTE 4:	For single tone () according to TS 36.213 [30] Table 16.5.1.2-1 the maximum value of ITBS is 10. Therefore according to table 16.5.1.2-2 the maximum grant of 1000 bits requires at least IRU = 5 corresponding to 6 resource units.
Furthermore for single tone according to TS 36.211 [35] Table 10.1.2.3-1 a resource unit takes 16 slots i.e. 8ms.
 tData  48ms (6 * 8) for maximum grant (single-tone).

### 7A.4.3	Downlink transmissions

Table 7A.4.3-1: Timing of downlink transmissions

| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | DCI format N1 |  |  |  |  |  | DL-SCH |  |  |  |  |  |  |  |  |  |  |  |  |  | ACK/NACK |  |  |  | ... |
|  | tDCI |  | tdelay1 |  |  |  | tData |  | tdelay2 |  |  |  |  |  |  |  |  |  |  |  | tACK/NACK |  |  |  |  |

with

| tDCI | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| --- | --- | --- |
| tdelay1 | 4ms | scheduling delay according to DCI (TS 36.213 [30] clause 16.4.1 and table 16.4.1-1) |
| tData | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.4.1) |
| tdelay2 | 12ms | depending on HARQ-ACK resource allocation according to TS 36.213 [30] clause 16.4.2 |
| tACK/NACK | 2ms | multiples of 2ms according to TS 36.211 [35] Table 10.1.2.3-1; depending on number of repetitions configured by RRC signalling (NPUSCH-ConfigCommon-NB-r13, NPUSCH-ConfigDedicated-NB-r13; NRep 1) |

NOTE 1:	In addition to the delay in table 7A.4.3-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general a downlink transmission takes one search space cycle.

NOTE 3:	Assuming a search space configuration as per clause 7A.3 in case of a HARQ NACK from the UE, a re-transmission needs to be scheduled in the next search space cycle (as even when the DL transmission has been scheduled using the first search space candidate, at the point in time the SS detects the NACK there is no search space candidate in the current search space cycle anymore).

### 7A.4.4	Half-duplex mode

According to TS 36.211 [35], for NB-IoT, only type-B half-duplex FDD operation is supported (clause 10.2.2.3) in which according to clause 4.1 the UE cannot transmit and receive at the same time.

In addition, in clause 16.6 of TS 36.213 [30] there are further rules further restricting the timing of UL/DL transmissions, which are summarised in table 7A.4.4-1 and table 7A.4.4-2, if the UE is not configured with higher layer parameter twoHARQ-ProcessesConfig.

Table 7A.4.4-1: Uplink transmissions

|  | n | n+1 | ... | n+k-1 | n+k |  |  | n1 | n1+1 | n1+2 | n1+3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI Format N0 |  |  |  |  | NPUSCH transmission |  |  |  |  |  |  | ACK/NACK (NPDCCH) |  |
|  |  | UL-SCH transmission: NPDCCH not monitored by the UE |  |  |  |  |  |  |  |  |  |  |  |

Table 7A.4.4-2: Downlink transmissions

|  | n | n+1 |  |  |  |  |  |  |  |  |  |  |  |  | n+k-1 | n+k |  |  | n1 | n1+1 | n1+2 | n1+3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI Format N1 |  |  |  |  |  |  | NPDSCH transmission |  |  |  |  |  |  |  |  | NPUSCH Format 2: ACK/NACK |  |  |  |  |  |  |
|  |  | DL-SCH transmission: NPDCCH not monitored by the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If the UE is configured with higher layer parameter twoHARQ-ProcessesConfig (Rel-14 and later), the UE can detect another PDCCH candidate before NPDSCH or NPUSCH transmission starts (see TS 36.213 [30] clause 16.6 for further details). Nevertheless there needs to be an appropriate timing for the transmissions being associated with two consecutive PDCCH candidates.

NOTE:	So far there are no test requirements in any test case to schedule more than one PDCCH candidate in the same search space.

### 7A.4.5	Conclusions

Based on the system information scheduling as in clause 7A.2 and search space configuration as in clause 7A.3 the following conclusions are made:

-	Random access procedure:
After detection of the random access preamble steps 2-4 of the random access procedure take at least two search space cycles.

-	Uplink and downlink transmissions:
In general an UL or DL transmission takes one search space cycle and UL/DL transmissions are mutually exclusive, i.e. a search space cycle can be either for UL or for DL.

<SECTIONS SKIPPED>

## 7A.7	RRC connection release sequence

According to TS 36.331 [19] clause 5.3.8.3, after reception of the RRCConnectionRelease the UE may either wait 10s or for indication of acknowledgement from lower layer. Furthermore according to TS 36.331 [19] clause 5.3.12, upon leaving RRC_CONNECTED state the UE resets layer 2.

Assuming search space cycles of 64ms as described in clause 7A.3 and with a given search space cycle N starting at time T being at least 100ms in advance, TTCN schedules the RRC connection release sequence as:

1.	At T:	Start of search space cycle N:
	Send RRCConnectionRelease, stop UL grants.

2.	At T + 64ms:	Start of search space cycle N+1:
	Release security (optional; only when security has been configured).
	Scheduling of UL grants for search space cycle N+1 and four subsequent cycles
	(NOTE 1).

3.	At T + 80ms:	Release SRBs and DRBs (if configured).

4.	At T + 96ms:	(Re-)configure SRBs.

5.	Delay of 1100ms from T onward (NOTE 2)

NOTE 1:	The UL grants are provided to allow the UE to send an RLC ACK to acknowledge the RRCConnectionRelease; as RLC is not configured anymore or reset at the SS already, the ACK is discarded by the SS. It is assumed that once the UE has sent the RLC ACK it stops the 10s timer.

NOTE 2:	The delay of 1100ms is to ensure the UE is re-camping on the cell and has read relevant system information, MIB-NB, SIB1-NB, SIB2-NB and all other SIs.

<SECTIONS SKIPPED>

## 7A.14	Non-Terrestrial Network (NTN)

### 7A.14.1	NTN cell configuration

An NB-IoT cell to be used in a Non-Terrestrial Network (NTN) test environment is an NB-IoT cell that is broadcasting a system information block combination, defined in TS 36.508 [3] clause 8.1.4.3.1.1, that contains SystemInformationBlockType31-NB (SIB31-NB). It is called an NB-IoT NTN cell.

In addition to that, an NB-IoT NTN cell shall be configured in SIB1-NB as "barred" in cellAccessRelatedInfo-r13 IE and as "notBarred" in cellAccessRelatedInfo-NTN-r17 IE.

Typically an NTN test case can be executed under one of the following two sub-NTN test environments:

-	the Geosynchronous Orbit (GSO) scenario, or

-	the Non-Geosynchronous Orbit (NGSO) scenario.

When a GSO scenario is simulated, TTCN implementation is responsible for maintaining the system information up-to-date (including SIB31-NB) and to ensure that the updated system information is being broadcast in the NB-IoT NTN cell before the timer T317 expires at UE.

When an NGSO scenario is simulated, TTCN provides to the SS only initial contents of SIB31-NB for a given NB-IoT NTN cell. As the test case proceeds, the SS shall periodically update SIB31-NB content with the values defined in the satellite ephemeris data specified in TS 36.508 [3] Annex B.1 according to the current timing and broadcast it afterwards. The updates and calculations shall be done by the SS with a periodicity equal to the si-WindowLength-r13 value.

In a multi-cell NGSO test case, TTCN initialises all the NB-IoT NTN cells in the SS with an identical SIB31-NB content as per the first cell specified in TS 36.508 [3] Table 6.3.5.2-1. After all the NB-IoT NTN cells are initialised in the SS, TTCN reconfigures and synchronises in the SS the contents of each SIB31-NB with the values corresponding to the cell number in the test case.

### 7A.14.2	NTN timing considerations

#### 7A.14.2.1	General

DL and UL are frame aligned at the UL time synchronization reference point (RP). To accommodate the long propagation delays in NTN, the timing relationships are defined using a Common Timing Advance (Common TA) and two offsets: Koffset and Kmac, with Koffset ≥ TTA.

The various NTN timing relationships are illustrated in Figure 7A.14.2.1-1.

Figure 7A.14.2.1-1: NTN timing relationship parameters

#### 7A.14.2.2	NTN timing advance

##### 7A.14.2.2.1	General

The Timing Advance (TA) formula, specified in TS 36.211 [35] clause 8.1, can be summarized as follows:

TTA = TTA,legacy + TTA,NTN

TTA,NTN = TTA,common + TTA,UE-specific

with:

-	TTA,legacy = (NTA + NTA,offset) * Ts: the (legacy) part of the TA formula that applies also to Terrestrial Networks,

-	TTA,common =  * Ts: the Common TA between the RP and the NTN payload,

-	TTA,UE-specific =  * Ts: the TA corresponding to the service link delay.

##### 7A.14.2.2.2 TA in GSO scenario

The GSO test environment in TS 36.508 [3] specifies that both the serving satellite (NTN payload) and the UE under test have a static position during the lifetime of a test case execution. Therefore the corresponding TTA is also static for a given NTN cell. TTCN provides to the SS the following inputs to enable the SS to calculate TTA:

Table 7A.14.2.2-1: NTN timing advance - input parameters

| TTA element | Associated parameter(s) |  | Relationship |
| --- | --- | --- | --- |
| (in sec) | Protocol parameter | ASP field | (TS 36.213 [30] clause 16.1.2) |
| TTA,legacy |  | InitialValue integer (0..2047) | NTA = InitialValue * 16 |
| TTA,common |  | NtnCommonTA integer (0..8316827) | See below. |
| TTA,UE-specific |  | NtnUeSpecificTA integer (0..8316827) | NtnUeSpecificTA See below. |
| NOTE: The time unit TS is defined in TS 36.211 [35] clause 4. |  |  |  |

From the nta-CommonParameters-17 specified in TS 36.331 [19], only the value of nta-Common-r17 is defined in TS 36.508 [3] SIB31-NB for the GSO scenario. TTCN will forward this value to the SS as NtnCommonTA for the calculation by the SS of  according to the formula given in TS 36.213 [30] clause 16.1.2.

NOTE:	For the GSO scenario it is assumed that the SS uses the calculation:  = NtnCommonTA.

is calculated using the GSO serving satellite static position specified in TS 36.508 [3] SIB31-NB's ephemeris data and the UE static position specified in TS 36.508 [3] clause 4.13. The end result is provided to the SS by TTCN. The calculation is provided hereafter:

The two-way propagation delay in sec between the GSO serving satellite and the non-moving UE is assumed to be constant at any time and is calculated according to the formula:

where c is the speed of light (299792458 m/sec) and d is the distance in meters between the serving satellite and the UE.

For two points in Euclidian geometry d in meters is derived by:

with

and

, ,  being serving satellite coordinates, in m

, ,  being UE coordinates, in m

To use above formulas the positioning data provided in TS 36.508 [3] is first converted to the coordinates accordingly.

Example of the calculation of :

given UE latitude 25.08439333 and positionX-r17 of -16976014:

-16976014* 1.3 m = -22068818.2 m

-3025296.94 m

Following the above calculations, the final value t results in 0.24338972709427653 sec that yields  to have a value of 7476932.416336175, that is rounded to 7476932.

##### 7A.14.2.2.3 TA in NGSO scenario

The NGSO test environment in TS 36.508 [3] specifies that the position of the serving satellite (NTN payload) varies with the time. This implies that TA also changes during the lifetime of a test case execution. To reduce the complexity, the reference point RP is defined at the satellite itself (TS 36.508 [3] clause 6.3.5.2) in NGSO scenario. This eliminates necessity of calculating that resolves to zero. However, the SS shall regularly (every si-WindowLength-r13) re-calculate the value of  for a particular NB-IoT NTN cell based on:

-	the actual timing,

-	the UE static position given in TS 36.508 [3] clause 4.13, and

-	the satellite ephemeris data specified in TS 36.508 [3] Annex B.1.

#### 7A.14.2.3	NTN scheduling offsets

The NTN timing parameters listed in Table 7A.14.2.3-1 come into play when scheduling data in an NTN cell.

Table 7A.14.2.3-1: NTN timing parameters in an NTN cell

| NTN timing parameter | Definition | Reference |
| --- | --- | --- |
| DLoffset | Koffset + Kmac | TS 36.321 [16] clause 7.7 |
| UE-eNB RTT | TTA + Kmac | TS 36.321 [16] clause 3.1 |
| Kmac | k-Mac or 0 | TS 36.213 [30] clause 16.6 |
| Koffset | k-Offset - Differential Koffset | TS 36.213 [30] clause 16 |
| RTToffset | UE-eNB RTT | TS 36.321 [16] clause 7.7 |

TTCN configures the NTN offsets Koffset and Kmac in the SS as part of the NTN cell configuration. The SS shall use those parameters to adjust the timing & scheduling of DL & UL transmissions specified in clauses 7A.4, 7A.5 and 7A.6 as follows:

-	Table 7A.4.1-1: Timing of random access procedure

| tdelay1 | 3ms + UE-eNB RTT | according to TS 36.321 [16] clause 5.1.4. |
| --- | --- | --- |
| tdelay3 | 12ms + Koffset | according to TS 36.213 [30] clause 16.5.1. |
| tdelay4 | 3ms + UE-eNB RTT | according to TS 36.321 [16] clause 5.1.5. |
| tdelay5 | DL: 13ms UL: 13ms + Koffset | depending on whether UL or DL transmission follows, see below. |

-	Table 7A.4.2-1: Timing of uplink transmissions

| tdelay1 | 8ms + Koffset | according to TS 36.213 [30] clause 16.5.1. |
| --- | --- | --- |
| tdelay2 | 3ms + RTToffset | according to TS 36.321 [16] clause 7.7. |

-	Table 7A.4.3-1: Timing of downlink transmissions

| tdelay2 | 12ms + Koffset | according to TS 36.213 [30] clause 16.4.2. |
| --- | --- | --- |

-	Table 7A.4.4-1: Uplink transmissions

-	The end of the period during which NPDCCH is not monitored by the UE is n1+Kmac+3: according to TS 36.213 [30] clause 16.6.

In addition to the NTN timing parameters referenced in Table 7A.14.2.3-1, the following NTN timing parameters are defined in the present specification for the purpose of conformance testing:

-	TNTN,offset = DLoffset

-	NPDCCH period bundle:

-	in a Terrestrial Networks (TN) test environment:

-	NPDCCH period bundle = NPDCCH period

-	in a Non Terrestrial Networks (NTN) test environment:

-	NPDCCH period bundle = CEIL(TNTN,offset / NPDCCH period) * NPDCCH period + NPDCCH period

#### 7A.14.2.4	RRC connection release sequence

The sequence specified in clause 7A.7 applies, with the following differences:

TTCN schedules the RRC connection release sequence at time T being at least TNTN,offset in advance:

2.	At T + 1 NPDCCH period bundle:	
		Scheduling of UL grants for search space cycle starting at (T + 1 NPDCCH period bundle) and four subsequent cycles

3.	At T + 1 NPDCCH period bundle + 16ms

4.	At T + 1 NPDCCH period bundle + 32ms