3GPP TSG-RAN5 Meeting #101	R5-236306

# Chicago, USA, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0538 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the UE implementation related PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case of pc_noOf_PDUsSameConnection = 1 AND pc_noOf_PDUsNewConnection =1, connected without release is not maintained at step 1b1 in Table 4.5.2.2-4 If pc_noOf_PDUsNewConnection =1, then, RRCRelease is transmitted at step 20a1 in Table 4.5.2.2-2 even though connected without release is present. The reason for RRCRelease transmission is to allow optional new PDU establishment in Table 4.5.2.2-4. Since pc_noOf_PDUsSameConnection =1, AT command for new PDU will not be triggered at step 1a1 in Table 4.5.2.2-4 if UE does not establish new PDU autonomously. This leads to step 1b1 in Table 4.5.2.2-4 and UE stays in RRC IDLE state even though RRC CONNECTED is required at the end procedure (i.e. connected without release is present). The definition of pc_noOf_PDUsNewConnection and pc_noOf_PDUsSameConnection is “Number of UE-requested PDU session establishments...”, therefore if the UE does not automatically request to establish a PDU session both PICS shall be set to 0, Similarly, the definition of pc_noOf_PDNsSameConnection and pc_noOf_PDNsNewConnection is “Number of UE-requested PDN connection establishments after ATTACH...”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the value of PICS pc_noOf_PDUsSameConnection, pc_noOf_PDUsNewConnection, pc_noOf_PDNsSameConnection and pc_noOf_PDNsNewConnection shall only include that the UE triggers without the aid of an external trigger. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different values of PICS may cause unexpected behaviour, causing test cases to fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table A.4.4-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is related to RAN5 SIG AP#100.02. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table A.4.4-2: Definition of UE implementation capabilities

| Item |  | Definition of UE implementation capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Void |  |  |  |  |  |  |  |  |  |
| 2 |  | Void |  |  |  |  |  |  |  |  |  |
| 3 |  | Number of UE-requested PDU session establishments after REGISTRATION during the same signalling connection |  | 24.501 |  | Rel-15 |  | pc_noOf_PDUsSameConnection |  | If the UE requires an external trigger to establish a PDU session, this value shall be set to 0 |  |
| 4 |  | Number of UE-requested PDU session establishments after REGISTRATION in a new signalling connection |  | 24.501 |  | Rel-15 |  | pc_noOf_PDUsNewConnection |  | If the UE requires an external trigger to establish a PDU session, this value shall be set to 0 |  |
| 5 |  | Number of UE-requested PDN connection establishments after ATTACH during the same signalling connection |  | 24.301 |  | Rel-15 |  | pc_noOf_PDNsSameConnection |  | If the UE requires an external trigger to establish a PDN connection, this value shall be set to 0 |  |
| 6 |  | Number of UE-requested PDN connection establishments after ATTACH in a new signalling connection |  | 24.301 |  | Rel-15 |  | pc_noOf_PDNsNewConnection |  | If the UE requires an external trigger to establish a PDN connection, this value shall be set to 0 |  |
| 7 |  | Void |  |  |  |  |  |  |  |  |  |
| 8 |  | Support of Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface |  | TS 24.501, 6.1.4.2 |  | Rel-15 |  | pc_TransferEmergencyPDUN1toS1noN26 |  | Will the UE attempt to transfer an existing Emergency PDU session upon inter-system change from N1 mode to S1 mode in EMM-IDLE mode if the network does not support N26 interface |  |
| 9 |  | Support of Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface |  | TS 24.501, 6.1.4.2 |  | Rel-15 |  | pc_TransferEmergencyPDUS1toN1noN26 |  | Will the UE attempt to transfer an existing Emergency PDN connection upon inter-system change from S1 mode to N1 mode in EMM-IDLE mode if the network does not support N26 interface |  |
| 10 |  | Support of UE's usage setting as data centric |  | TS 24.501, 4.3.1 |  | Rel-15 |  | pc_data_centric |  | UE supports to be configured to consistently behave as a Data centric UE. |  |
| 11 |  | Support of join in MBS multicast session by sending a PDU Session Modification Request |  | TS 23.247 7.2.1 |  | Rel-17 |  | pc_Join_MBS_by_PDU_Modification |  | If pc_Join_MBS_by_PDU_Modification, UE join in MBS multicast session by sending a PDU Session Modification Request, else UE join in MBS multicast session by sending a PDU Session Establishment Request |  |