3GPP TSG-RAN5 Meeting #101	R5-236316

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4771 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. If the UE does not support switch off, a valid alternative path, for example during the postamble, is to remove the USIM. MUSIM UEs have multiple USIMs, therefore an extra parameter is required to specify which USIM shall be removed. 2. There are typos in clause 4.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduced an optional parameter to the existing MMI to remove the USIM, which specifies which USIM is to be removed. 2. Editorial issues have been resolved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. An MUSIM UE not supporting switch off cannot be turned off. 2. Editorial typos will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Generic WLAN Test Model

The generic WLAN test model is applicable for IMS over WLAN testing, as well as for E-UTRAN/UTRAN-WLAN interworking testing. The TTCN and SS implementations emulate an untrusted Non-3GPP IP Access as specified in TS 23.402 [63]: There is the WLAN access point (WLAN AP) and the emulation of ePDG and AAA-server.

### 4.5.1	WLAN Access Point

The WLAN access point emulation is done at the SS with a few configuration parameters provided by TTCN. The following layers need to be supported:

-	Station Management Entity

-	Physical layer:

-	Physical Sublayer Management Entity

-	Physical layer Convergence procedure

-	Physical Medium Dependent

-	MAC layer:

-	MAC Management Entity

-	MAC sublayer

-	Also contains security services

-	ANQP element and HS 2.0 ANQP Network elements

WLAN security protocol using WPA/WPA2 algorithms shall be supported.

### 4.5.2	ePDG/AAA-Server Emulation

An IPsec tunnel is established between the UE being in the untrusted network and the ePDG: Establishment, maintenance and release of the IPsec tunnel requires IP signalling between the tunnel end-points (UE, ePDG) according to RFC 5996 [64] and TS 33.402 [25]. IP data between the UE and the emulated 3GPP network is transferred through the IPsec tunnel by using UDP Encapsulation of IPsec ESP Packets according to RFC 3948 [65].

Figure 4.5.2-1 shows the principle of the IPsec tunnel. Note that the IP addresses of 3GPP and non-3GPP network shall be different.

Figure 4.5.2-1: IPsec tunnel for untrusted Non-3GPP IP Access to 3GPP network

Table 4.5.2-1 and table 4.5.2-2 summarise the network entities of the 3GPP network and the non-3GPP network for the different scenarios of the WLAN test model.

Table 4.5.2-1: Entities of a 3GPP network (PDN) in the WLAN test model (NOTE 1)

| Network entity | Purpose | Network emulation by TTCN | Address assignment to the UE |
| --- | --- | --- | --- |
| DHCP server | IPv4 address allocation | no DHCP server emulation in TTCN but IPv4 address allocation is done by the SS via IKE signalling during IPsec tunnel establishment | DHCP discovery procedure |
| ICMPv6 server | IPv6 address allocation | no ICMPv6 server emulation in TTCN but IPv6 address allocation is done by the SS via IKE signalling during IPsec tunnel establishment | Router discovery procedure |
| DNS server | IP address resolution | In general no DNS server emulation in TTCN (NOTE 2) | IKE signalling during IPsec tunnel establishment |
| P-CSCF | IMS server | emulation started for the PDN | IKE signalling during IPsec tunnel establishment |
| XCAP server | XCAP | Emulation started for 3GPP network in IMS XCAP scenarios with 3GPP-XCAP-Server | DNS query at the PDN’s DNS server (NOTE 2) |
| BSF server | GBA authentication | Emulation started for 3GPP network in IMS XCAP scenarios with 3GPP-XCAP-Server and BSF server used for GBA authentication | DNS query at the PDN’s DNS server (NOTE 2) |
| NOTE 1: Access to the 3GPP network happens through IPsec tunnel between UE and ePDG. NOTE 2: DNS server emulation is needed for the 3GPP network in IMS XCAP test scenarios with 3GPP-XCAP server, i.e. when the XCAP server is emulated to be in the 3GPP network; this requires PDN connection to the 3GPP network via IPsec tunnel. |  |  |  |

Table 4.5.2-2: Entities of a non-3GPP network (WLAN AP) in the WLAN test model

| Network entity | Purpose | Network emulation by TTCN | Address assignment to the UE |
| --- | --- | --- | --- |
| DHCP server | IPv4 address allocation | no DHCP server emulation in TTCN but IPv4 address allocation is done by the SS when UE associates with the WLAN AP | DHCP discovery procedure |
| ICMPv6 server | IPv6 address allocation | no ICMPv6 server emulation in TTCN but IPv6 address allocation is done by the SS when UE associates with the WLAN AP | Router discovery procedure |
| DNS server | IP address resolution | DNS server emulation started to handle queries for the ePDG and the XCAP server (NOTE 1) | lower layer signalling with WLAN AP when UE associates with the WLAN AP |
| ePDG | Gateway to PDN | no IP-handling for ePDG in TTCN | DNS query at non-3GPP DNS server |
| XCAP server | XCAP | Emulation started for non-3GPP network in IMS XCAP scenarios with 3GPP-XCAP-Server | DNS query at non-3GPP DNS server (NOTE 1) |
| BSF server | GBA authentication | n.a. (NOTE 2) |  |
| NOTE 1: DNS server emulation of non-3GPP network shall handle queries for the XCAP server only if the XCAP server is emulated to be in the non-3GPP network. NOTE 2 As working assumption there is no GBA authentication in case of non-3GPP XCAP server. |  |  |  |

During establishment of the IPsec tunnel, parameters for authentication and authorization (EAP AKA) are exchanged between the ePDG and the AAA-server (see TS 33.402 [25] clause 8).

The means for establishment, maintenance and release of the IPsec tunnel shall be provided by the SS.  The TTCN does not deal with IKE messages as such. In addition to the IP test model of clause 4.2.4 there are the following requirements for SS implementation:

-	IPsec tunnelling according to RFC 5996 [64] (IKEv2) and RFC 3948 [65]

-	Emulation of EAP AKA according to RFC 4187 [66] i.e. interworking between IPsec (ePDG) and emulated AAA-server

The enhancement of the common IP test model of clause 4.2.4 is shown in figure 4.5.2-2.

Figure 4.5.2-2: Generic WLAN Test Model

Further implementation requirements:

IPsec tunnel handling is controlled by the TTCN with static parameters:

-	DNS for dynamic selection of the ePDG is handled in the TTCN according to the common IP test model

-	DHCP and ICMPv6 can be handled in the TTCN when needed (as for E-UTRA or UTRAN)

-	IMS SIP/SDP signalling is handled in the TTCN as for E-UTRA or UTRAN

-	RTP/RTCP loopback model can be configured in the same way as for E-UTRA

-	Routing of IP packets from and to WLAN is controlled by the DRB-Mux in the same way as for E-UTRA or UTRAN

In the context of the IPsec tunnel, logically two instances of the DRB-Mux can be considered:

-	Uplink

-	lower DRB-Mux to distinguish whether a UDP packet is IKE or ESP data

-	an IKE or ESP packet needs to be routed to the IPsec tunnel

-	upper DRB-Mux to handle IP packets coming out of the IPsec tunnel

-	Downlink

-	upper DRB-Mux to distinguish whether an IP packet shall be sent into the IPsec tunnel

-	lower DRB-Mux to determine routing of IKE/ESP packets (i.e. the packets inside the tunnel)

<SECTIONS SKIPPED>

## 5.2	Upper Tester ASPs

There is one primitive defined carrying either an MMI or an AT command to be sent to the system adaptor and one common confirmation primitive to be sent by the system adaptor.

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_SYSTEM_REQ |  |  |
| TTCN-3 Type | Record |  |  |
| Cmd |  | TTCN-3 Type | union |
| AT |  | charstring carrying the AT command as defined in TS 27.007 [32], TS 27.005 [31] and TS 27.060 [33] |  |
| MMI |  | - Cmd (charstring) - List of parameters: - Name (charstring) - Value (charstring) |  |
| CnfRequired |  | TTCN-3 Type | Ut_CnfReq_Type |
|  |  | CNF_REQUIRED: SS shall reply with one confirmation (NOTE) NO_CNF_REQUIRED: SS shall swallow any confirmation generated LOCAL_CNF_REQUIRED: SS shall immediately send one confirmation when the command is submitted to the UE i.e. in case of operator interaction when the operator has confirmed the command, but SS shall not wait for the UE responding(NOTE) |  |
| NOTE: In the TTCN, a confirmation shall only be requested in cases when there is no signalling from the UE being triggered by the MMI/AT command |  |  |  |

| TTCN-3 ASP Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name | UT_COMMON_CNF |  |  |
| TTCN-3 Type | Record |  |  |
| Result |  | TTCN-3 Type | boolean |
|  |  | true: success false: failure NOTE: The return value false will result in the test immediately finishing with a fatal error. In case of AT commands, the SS shall return false if and only if the UE comes back with "ERROR" as result code according to ITU-T Rec V.250 clause 5.7 [60]. In case of e.g. "+CME ERROR" the SS shall return true and put the UE's response into the ResultString |  |
| ResultString |  | TTCN-3 Type | charstring |
|  |  | response by the UE for commands which request the UE to return a result, optional In case of AT commands, ResultString shall contain the complete line according to the specification of the AT command. Any carriage return and linefeed characters, defined in TS 27.007 [32] clause 3.1, contained in the response shall be included as the textual representations "<CR>" and "<LF>" respectively. |  |

The following MMI commands are defined.

Table 5.1: MMI commands

| Command | Parameters |  |  |  |
| --- | --- | --- | --- | --- |
|  | Name |  | Value |  |
| "SWITCH_ON" | (none) |  |  |  |
| "SWITCH_OFF" | (none) |  |  |  |
| "POWER_ON" | (none) |  |  |  |
| "POWER_OFF" | (none) |  |  |  |
| "INSERT USIM" | "USIM" |  | <USIM> |  |
| "REMOVE_USIM" | "USIM" (optional) | <USIM> |  |  |
| "CHECK_PLMN" | "PLMN" |  | <PLMN ID> |  |
| "CHECK_ETWS_INDICATION" | “WARNING1” |  |  | <WARNING1> |
|  | “WARNING2” |  |  | <WARNING2> |
| “CHECK_ETWS_ALERT” | (none) |  |  |  |
| “CHECK_ETWS_NO_ALERT” | (none) |  |  |  |
| "CHECK_CMAS_INDICATION" | “WARNING1” |  |  | <WARNING1> |
|  | “WARNING2” |  |  | <WARNING2> |
| “CHECK_CMAS_ALERT” | (none) |  |  |  |
| “CHECK_CMAS_NO_ALERT” | (none) |  |  |  |
| "HRPD_PDN_CONNECTION” | (none) |  |  |  |
| “CHECK_SMS_LENGTH_CONTENTS" | "Length" |  |  | <Length> |
|  | "Msg" |  |  | <Msg> |
| "CONFIGURE_SMS_ONLY" | (none) |  |  |  |
| "DISABLE EPS CAPABILITY" | (none) |  |  |  |
| "DETACH_NON_EPS" | (none) |  |  |  |
| "CLEAR_STORED_ASSISTANCE_DATA" | (none) |  |  |  |
| "CHECK_DTCH_THROUGHCONNECTED" | (none) |  |  |  |
| "GERAN_UPLINK_DATA" | (none) |  |  |  |
| "SELECT_CSG" | "PLMN" |  |  | <PLMN ID> |
|  | "CSG" |  |  | <CSG ID> |
| “TRIGGER_USER_RESELECTION” | (none) |  |  |  |
| "REQUEST_NON_CALL_RELATED_SS" | (none) |  |  |  |
| "MBMS_SERVICE_INTEREST" | "PLMN" |  |  | <PLMN ID> |
|  | "Service" |  |  | <MBMS Service ID> |
|  | "Interest" |  |  | "ON" / "OFF" |
|  | "SAI" |  |  | <MBMS SAI> |
| “MBMS_SERVICE_ACTIVE” | "Service" |  |  | <MBMS Service ID> |
|  | "SAI" |  |  | <MBMS SAI> |
|  | “Active” |  |  | "ON" / "OFF" |
| “MBMS_PRIORITY_OVER_UNICAST” | "Priority Over Unicast" |  |  | "FALSE" / "TRUE" |
| "CHECK_ETWS_NO_INDICATION" | “WARNING1” |  |  | <WARNING1> |
|  | “WARNING2” |  |  | <WARNING2> |
| "CHECK_CMAS_NO_INDICATION" | “WARNING1” |  |  | <WARNING1> |
|  | “WARNING2” |  |  | <WARNING2> |
| "CHECK_CALL_DISPLAY" | "ToDisplay" |  |  | <NAME>/<NUMBER> |
| "CHECK_CSG" | "CSG" |  |  | < CSG ID > |
|  | "Included" |  |  | "FALSE" / "TRUE" |
| "CHECK_NITZ_DST" | "DaylightSavingTime:" |  |  | DST |
| "CHECK_RESPONSE_DISPLAY" | "ToDisplay" |  |  | <STRING> |
| "CHECK_SS_USER_IND" | "Supplementary Service" |  |  | <SUPPLEMENTARY SERVICE> |
|  | "ToDisplay" |  |  | <STRING> |
| “CHECK_SS_USER_IND_FAILURE” | (none) |  |  |  |
| "CONFIGURE_OPERATIONMODE" | "OPERATION MODE" |  |  | <OPERATION MODE> |
| “CONFIGURE_ANDSF_RULE” | “ANDSF Rule” |  |  | <ANDSF RULE> |
| “WLAN_ANDSF_CONFIGURED” | (none) |  |  |  |
| “WLAN_ANDSF_NOT_CONFIGURED” | (none) |  |  |  |
| "CMTLR_DUMMY" (see Note) | ”Notification Type” |  |  | <NOTIFICATION TYPE> |
| "CCFC_DUMMY" (see Note) | ”Status” |  |  | <STATUS> |
| "CNAP_DUMMY" (see Note) | ”Testname” |  |  | <TESTNAME> |
| "CTZE_DUMMY" (see Note) | "Year |  |  | <YEAR> |
|  | "Month" |  |  | <MONTH> |
|  | "Day" |  |  | <DAY> |
|  | "Hour" |  |  | <HOUR> |
|  | "Minutes" |  |  | <MINUTES> |
|  | "Seconds" |  |  | <SECONDS> |
|  | “Daylight SavingTime” |  |  | <DAYLIGHT SAVING TIME> |
|  | "TimeZone" |  |  | <TIMEZONE> |
| "CMWN_DUMMY" (see Note) | (none) |  |  |  |
| “D2D_DISCOVERY_CLEAR” | (none) |  |  |  |
| “D2D_DISCOVERY_CONFIGURATION” | “ProSeApplicationID” |  |  | <charstring> |
|  | "PLMN_List" |  |  | <PLMN ID_List> |
|  | T4005 |  |  | <T4005> |
| “D2D_DISCOVERY” | “ProSeApplicationID” |  |  | <charstring> |
|  | “Announce” |  |  | "ON" / "OFF" |
|  | “Monitor” |  |  | "ON" / "OFF" |
| “D2D_COMMUNICATION” | “ProSe Layer-2 Group ID” |  |  | <charstring> |
|  | “Transmit” |  |  | "ON" / "OFF" |
|  | “Receive” |  |  | "ON" / "OFF" |
|  | “Max size of UE transmitted Direct Communication data” |  |  | <bytes> |
| “D2D_COMMUNICATION_CONFIGURATION” | “ProSe Layer-2 Group ID” |  |  | <charstring> |
|  | "PLMN_List" |  |  | <PLMN ID_List> |
|  | T4005_Minutes |  |  | <T4005> |
|  | “Security Life time timer in minutes” (Default: 0) |  |  | <SecurityTimer> |
| "V2X_SIDELINK" | "Data Type" |  |  | "IP" / "NON-IP" |
| “INSERT_USIM_OR_PRECONFIGURE” | "USIM" |  |  | <USIM> |
| “DELETE_NTN_PLMN_NA” | (none) |  |  |  |
| “CONFIGURE_NTN_UE_POSITION” | "SpecReference" |  |  | <charstring> |
| Note: These MMI commands are used to indicate to the SS that a UT response is expected by the TTCN in the case when the AT command is not supported. They can be ignored if the UE supports the AT command. The prefix of these commands matches the start of the AT response expected by the TTCN |  |  |  |  |

<End of modified section>