3GPP TSG-RAN5 Meeting #101	 R5-236377

# Chicago, US, November 13rd – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0828 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test point analysis clause for additional NR CA band configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Anritsu, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update test point analysis clause for additional NR CA band configurations of CA_n2A-n77C, CA_n5A-n77C, CA_n66A-n77C, and CA_n2A-n48A, and CA_n2A-n48B |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the following tables for additional NR CA band configurations: - Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band) - Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands) Include the missing information of CA_n77C into Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The updated information is missing in the spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR 2487 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 4.1.3	Test point analysis per NR CA configuration

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1 through Table 4.1.3.1-5.

This clause contains information on test point analysis, test frequency selection and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1 through Table 4.1.3.1 4. The principles for selection of reference sensitivity test points for NR CA is given in Annex B including a test point analysis check list in B.9.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band)

| CA config | Single band refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n2(2A) |  |  |  | NE |  | RAN5#100 |  |  |
| CA_n41C |  | NE | RAN5#89-e |
| CA_n48B |  | NE | RAN5#94-e |
| CA_n66B |  | NE | RAN5#86-e |
| CA_n66(2A) |  | NE | RAN5#86-e |
| CA_n66(3A) |  |  |  | NE |  | RAN5#100 |  |  |
| CA_n71(2A) |  | NE | RAN5#94-e |
| CA_n77(2A) |  |  |  | NE |  | RAN5#100 |  |  |
| CA_n77C |  | NE | RAN5#96e |
| CA_n78C |  | NE | RAN5#87-e |
| NOTE 1: Void. NOTE 2: Notations used: NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |  |  |  |  |

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config |  | Two band refsens exception, victim band (Note 1) |  |  | Requirement coverage (Note 1) |  |  | Test point analysis updated |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  | n1 (IMD3) n3(CBI) |  |  | IMD,CBI |  |  | RAN5#95-e |  |  |
| CA_n1A-n8A |  |  | n1 (IMD4) |  |  | IMD |  |  | RAN5#95-e |  |  |
| CA_n1A-n77A |  | n77 (HD2) |  |  | HD |  |  | RAN5#89-e |  |  |
| CA_n1A-n78A |  | n1 (IMD4) |  |  | IMD |  |  | RAN5#94 |  |  |
| CA_n2A-n48A |  |  | n48 (HD2), n2(IMD4) |  |  | HD, IMD |  |  | RAN5#96-e |  |  |
| CA_n2A-n5A |  |  | - |  |  | NE |  |  | RAN5#97 |  |  |
| CA_n2A-n14A |  |  | - |  |  | NE |  |  | RAN5#100 |  |  |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#100 |
| CA_n2A-n48B | (NOTE 2) | NE | RAN5#101 |
| CA_n2A-n66A |  |  | n2(IMD3), n66(IMD5) |  |  | IMD |  |  | RAN5#96-e |  |  |
| CA_n2A-n77A PC3 |  |  | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) |  |  | HD, HM, IMD, |  |  | RAN5#99 |  |  |
| CA_n2A-n77A PC2 |  |  | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4) n2(CBI) |  |  | HD, HM, IMD, CBI |  |  | RAN5#99 |  |  |
| CA_n2A-n77C | (NOTE 2) | NE | RAN5#101 |
| CA_n3A-n5A |  | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) |  |  | IMD |  |  | RAN5#95-e |  |  |
| CA_n3A-n8A |  | n8(IMD4), n3(IMD5) |  |  | IMD |  |  | RAN5#100 |  |  |
| CA_n3A-n41A PC2 |  | n3 (IMD4), n3 (CBI) |  |  | IMD, CBI |  |  | RAN5#100 |  |  |
| CA_n3A-n41A PC3 |  | n3 (IMD4), n3 (CBI), n41 (CBI) |  |  | IMD, CBI |  |  | RAN5#100 |  |  |
| CA_n3A-n77A |  | n77 (HD2), n3 (IMD2), n3 (IMD4) |  |  | HD, IMD |  |  | RAN5#99 |  |  |
| CA_n3A-n78A |  | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) |  |  | HD, IMD |  |  | RAN5#87-e |  |  |
| CA_n5A-n66A |  |  | n5(IMD2) |  |  | IMD |  |  | RAN5#96-e |  |  |
| CA_n5A-n77A PC3 |  |  | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) |  |  | HD, HM, IMD |  |  | RAN5#99 |  |  |
| CA_n5A-n77A PC2 |  |  | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4) |  |  | HD, HM, IMD |  |  | RAN5#99 |  |  |
| CA_n5A-n77C | (NOTE 2) | NE | RAN5#101 |
| CA_n5A-n78A |  | n78 (HD2) |  |  | HD |  |  | RAN5#94-e |  |  |
| CA_n7A-n78A |  | n7 and n78 (CBI) |  |  | CBI |  |  | RAN5#94-e |  |  |
| CA_n8A-n78A |  | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) |  |  | HD, IMD |  |  | RAN5#87-e |  |  |
| CA_n14A-n30A |  | - |  |  | NE |  |  | RAN5#100 |  |  |
| CA_n14A-n66A |  | - |  |  | NE |  |  | RAN5#100 |  |  |
| CA_n14A-n77A |  | n77 (HD5), n14 (HM), n14 (IMD5) |  |  | HD, HM, IMD |  |  | RAN5#100 |  |  |
| CA_n20A-n78A |  | n78 (HD4), n20 (IMD4) |  |  | HD, IMD |  |  | RAN5#100 |  |  |
| CA_n26A-n66A |  | n26 (IMD2) |  |  | IMD |  |  | RAN5#94-e |  |  |
| CA_n26A-n70A |  | n26 (IMD2) |  |  | IMD |  |  | RAN5#94-e |  |  |
| CA_n28A-n41A |  |  |  |  | NE |  |  | RAN5#91-e |  |  |
| CA_n28A-n77A |  | n77 (HD5), n28 (IMD5) |  |  | HD, IMD |  |  | RAN5#99 |  |  |
| CA_n28A-n78A |  | n78 (HD5) |  |  | HD |  |  | RAN5#99 |  |  |
| CA_n28A-n79A |  |  |  |  | NE |  |  | RAN5#91-e |  |  |
| CA_n29A-n66A |  |  |  |  | NE |  |  | RAN5#94-e |  |  |
| CA_n29A-n70A |  |  |  |  | NE |  |  | RAN5#94-e |  |  |
| CA_n29A-n71A |  | n29 (CBI) |  |  | CBI |  |  | RAN5#95-e |  |  |
| CA_n39A-n41A |  | - |  |  | NE |  |  | RAN5#100 |  |  |
| CA_n41A-n66A3 |  |  | n66 (CBI), n66 (IMD5) |  |  | CBI |  |  | RAN#97 |  |  |
| CA_n41A-n71A3 |  |  | n41(HD4), n71 (IMD4) |  |  | HD |  |  | RAN5#98 |  |  |
| CA_n41A-n79A PC3 |  |  |  |  | NE |  |  | RAN5#83 |  |  |
| CA_n41A-n79A PC2 |  | n41 (CBI), n79 (CBI) |  |  | CBI |  |  | RAN5#100 |  |  |
| CA_n48A-n66A |  | n48 (HD2), n66 (IMD5) |  |  | HD, IMD |  |  | RAN5#94-e |  |  |
| CA_n48A-n66(2A) |  | (NOTE 2) |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48A-n70A |  | n70 (IMD2) |  |  | IMD |  |  | RAN5#94-e |  |  |
| CA_n48A-n71A |  |  |  |  | NE |  |  | RAN5#94-e |  |  |
| CA_n48A-n71(2A) |  | - |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48A-n77A |  |  | - |  |  | NE |  |  | RAN5#97-e |  |  |
| CA_n48B-n66A |  | (NOTE 2) |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48B-n70A |  | (NOTE 2) |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48B-n71A |  | - |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48(2A)-n66A |  | (NOTE 2) |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48(2A)-n66(2A) |  |  | (NOTE 2) |  |  | NE |  |  | RAN5#96-e |  |  |
| CA_n48(2A)-n70A |  | (NOTE 2) |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48(2A)-n71A |  | - |  |  | NE |  |  | RAN5#95-e |  |  |
| CA_n48(2A)-n71(2A) |  |  | - |  |  | NE |  |  | RAN5#96-e |  |  |
| CA_n66A-n70A |  |  |  |  | NE |  |  | RAN5#94-e |  |  |
| CA_n66A-n71A |  | n66 (IMD4) |  |  | IMD |  |  | RAN5#87-e |  |  |
| CA_n66A-n77A PC3 |  |  | n77 (HD2), n66(IMD2), n66(IMD5) |  |  | HD, IMD |  |  | RAN5#99 |  |  |
| CA_n66A-n77A PC2 |  |  | n77 (HD2), n66(IMD2), n66(IMD5), n66(CBI) |  |  | HD, IMD, CBI |  |  | RAN5#99 |  |  |
| CA_n66A-n77C | (NOTE 2) | NE | RAN5#101 |
| CA_n70A-n71A |  | n70 (HD3), n70 (IMD4) |  |  | HD, IMD |  |  | RAN5#85 |  |  |
| CA_n71A-n77A |  |  | n71 (IMD5) |  |  | IMD |  |  | RAN5#97-e |  |  |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case. NOTE 3: Only single uplink analysed. IMD exception not yet covered. |  |  |  |  |  |  |  |  |  |  |

<End of changes>