3GPP TSG-RAN5 Meeting #101	 R5-236379

# Chicago, US, November 13rd – November 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2489 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test configuration table modification due to core specification alignments and CR implementation mismatch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test configuration table modifications due to core specification alignments and CR (R5-234199) implementation mismatch |  |  |  |  |  |  |  |  |
| Summary of change: |  | Due to specification alignments made for Table 7.3A.0.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations, the corresponding test configuration table needs to be modified. In the meantime, fix the CR R5-234199 implementation mismatch issues in the following same table: - Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core requirements and CR R5-234199 is not implemented correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 7.3A.2_1		Reference sensitivity power level for 3DL CA exceptions

7.3A.2_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band NR 3DL CA with UL CA support

7.3A.2_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2_1.4	Test description

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1770/ DL 2170 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | 2600 | n28 | UL 725/ DL 780 | n79 | 4600 | 10MHz | 5 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4480 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n41 | 2645 | n79 | 4850 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<End of changes>