3GPP TSG-RAN5 Meeting #101	R5-236385

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2697 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum requirements for 7.6.16.0 - FR2 NCSG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add minimum requirements for FR2 NCSG TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minimum requirements for FR2 NCSG TCs are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.16.0 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.6.16.0	Minimum conformance requirements

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 7.6.16.0.1-1 and 7.6.16.0.1-2 (deactivated Scell).

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 7.6.16.0.1-3 and 7.6.16.0.1-4 (deactivated Scell).

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in TS 38.133 [6] clause 9.1.5.3 for measurement conducted within NCSG.

Mpss/sss_sync_with_gaps : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting power class 4, Mpss/sss_sync with_gaps =24

Mmeas_period_ with_gaps: For a UE supporting power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting power class 2, Mmeas_period_ with_gaps =24. For a UE supporting power class 3, Mmeas_period_ with_gaps =24. For a UE supporting power class 4, Mmeas_period with_gaps =24.

Table 7.6.16.0.1-1: Time period for PSS/SSS detection with NCSG (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_gaps x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_gaps) x max(VIRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps x max(VIRP, DRX cycle) x CSSFintra |

Table 7.6.16.0.1-2: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 7.6.16.0.1-3: Measurement period for intra-frequency measurements with NCSG (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period with_gaps) x max(VIRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320ms | Mmeas_period with_gaps x max(VIRP, DRX cycle) x CSSFintra |

Table 7.6.16.0.1-4: Measurement period for intra-frequency measurements with NCSG (deactivated SCell) (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.3 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.8 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.13 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.3.1.1 and 10.1.3.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.8.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.13.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.3.1.1, 10.1.3.1.2, 10.1.8.1.1 and 10.1.13.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.7.1. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.7.1, and 9.2.7.2.

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured. Otherwise, UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 7.6.16.0.2-1.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 7.6.16.0.2-2.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync_inter = 40 samples.

MSSB_index_inter: For a UE supporting FR2 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2 power class 4, MSSB_index_inter = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1 or 5, Mmeas_period_inter =64 samples. For a UE supporting FR2 power class 2, Mmeas_period_inter=40 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 40 samples.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in TS 38.133[6] clause 9.1.5.x for measurement conducted within NCSG.

Table 7.6.16.0.2-1: Time period for PSS/SSS detection with NCSG (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_inter Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_inter) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Mpss/sss_sync_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 7.6.16.0.2-2: Measurement period for inter-frequency measurements with NCSG (FR2)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_inter Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_inter) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Mmeas_period_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

When NCSG are provided for inter frequency measurements, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 7.6.16.0.2-2.

For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement,

-	An inter-frequency SSB measurement is defined as measurement with NCSG if

-	the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and

-	the SSB is not completely contained in the active BWP of the UE

When network configures NCSG, the delay requirements are specified in TS 38.133 [6] clause 9.3.10.

-	For inter-frequency SSB based measurements with NCSG, UE may cause scheduling restriction as specified in TS 38.133 [6] clause 9.3.10.3.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.3 for a corresponding Band.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.6.3, and 9.3.10.

7.6.16.0.3	Minimum conformance requirements for interruptions

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s),

-	when mgta is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the last each of the interrupted slots after VIL1 and VIL2.

where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if  for the UL transmission is less than the length of one slot; L=2 otherwise.

Note: Network is supposed to take into account the possible difference between the estimated TA at network and actual TA at UE when scheduling UE in the above slot(s).

The interruptions of NCSG in number of slots are listed in Table 7.6.16.0.3-1 on all serving cells when per-UE NCSG is configured or on FR1 serving cells when per-FR FR1 NCSG is configured to ncsg-MeasGapPerFR-r17 capable UE. There are two interruptions in each NCSG occasion, VIL1 before ML and VIL2 after ML, in NR standalone (with single carrier or NR CA). Each of them has number of interrupted slots captured in Table 7.6.16.0.3-1.

Table 7.6.16.0.3-1: Number of interrupted slots on FR2 serving cells for FR2 NCSG during each VIL in NR standalone operation (with single carrier, NR CA)

| NR | Number of interrupted slots on serving cells |  |
| --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied | When MG timing advance of 0.75ms is applied |
| (kHz) | VIL=0.75ms | VIL=0.75ms |
| 60 | 3 | 3 |
| 120 | 6 | 6 |
| NOTE 1: Void |  |  |

<< End of changes >>