3GPP TSG-RAN5 Meeting #101	R5-236387

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2717 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of serving satellite config for NB-IOT NTN test cases into Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Serving satellite configs are missing for NB-IOT NTN test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to core spec TS 36.133 A.3.28, add serving satellite config into Annex for NB-IOT NTN test cases reference. And update based on R4-2318072. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.10.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2318072 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## A.10.3	Reference NPRACH Configurations

Table A.10.3-1 and A.10.3-2 defines the reference NB-IoT PRACH configurations for a NB-IoT RRM test case where the UE is required to transmit NPRACH during the testing procedure, but the testing purpose of the RRM test case does not include testing NPRACH performance.

Table A.10.3-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Prameters not per NPRACH coverage level |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |
| Parameters per NPRACH coverage Level |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Valid values as defined in 3GPP TS 36.331 [5] |
| nprach-Periodicity | ms40 | ms40 | ms40 | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |
| nprach-StartTime | ms8 | ms8 | ms8 | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |
| nprach-SubcarrierOffset | n0 | n0 | n0 | {n0, n12, n24, n36, n2, n18, n34} |
| nprach-NumSubcarriers | n12 | n12 | n12 | {n12, n24, n36, n48} |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} | {zero, oneThird, twoThird, one} |
| maxNumPreambleAttemptCE | n3 | n5 | n7 | {n3, n4, n5, n6, n7, n8, n10} |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 | {n1, n2, n4, n8, n16, n32, n64, n128} |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |
| npdcch-StartSF-CSS-RA | V8 | v2 | v2 | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |
| npdcch-Offset-RA | zero | Zero | zero | {zero, oneEighth, oneFourth, threeEighth} |
| NOTE: See Clause 6.3.2 in 3GPP TS 36.331 [5] for further information on the parameters in this table. |  |  |  |  |

Table A.10.3-2: NPRACH.R-2: TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Prameters not per NPRACH coverage level |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |
| nprach-PreambleFormat | fmt-0 |  |  | See 3GPP TS 36.211 [9] clause 10.1.6 |
| Parameters per NPRACH coverage Level |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 | Valid values as defined in 3GPP TS 36.331 [5] |
| nprach-Periodicity | ms80 | ms80 | ms80 | {ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240} |
| nprach-StartTime | ms10 | ms10 | ms10 | {ms10, ms20, ms40, ms80, ms160, ms320, ms640, ms1280, ms 2560, ms5120} |
| nprach-SubcarrierOffset | n0 | n0 | n0 | {n0, n12, n24, n36, n2, n18, n34} |
| nprach-NumSubcarriers | n12 | n12 | n12 | {n12, n24, n36, n48} |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} | {zero, oneThird, twoThird, one} |
| maxNumPreambleAttemptCE | n3 | n5 | n7 | {n3, n4, n5, n6, n7, n8, n10} |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 | {n1, n2, n4, n8, n16, n32, n64, n128, n256, n512, n1024} |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |
| npdcch-StartSF-CSS-RA | v8 | v4 | v4 | {v4, v8, v16, v32, v48, v64, v96, v128} |
| npdcch-Offset-RA | zero | zero | zero | {zero, oneEighth, oneFourth, threeEighth} |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |
| NOTE: See Clause 6.7.3.2 in 3GPP TS 36.331 [5] for further information on the parameters in this table. |  |  |  |  |

## A.10.4	Reference Configurations for Satellites

The general parameters for SIB31 setup for serving satellite is specified in Table A.10.4-1.

Table A.10.4-1: SIB31 parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

# A.11	V2V Sidelink Communication on Dedicated V2V Carrier

## A.11.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2V sidelink communication on dedicated V2V carrier.

<< End of changes >>