3GPP TSG-RAN5 Meeting #101	R5-236410

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2946 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new table for BWP without Coreset0 in 6.2.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RedCap TC 7.1.1.8.4 use BWP without CORESET#0, new configuration table is needed to add in 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the NCD-SSB and BWP configuration table from 7.1.1.8.4 Table 7.1.1.8.4.3.2-3/4 to 38.508-1 6.2.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No common configuration for BWP without CORESET#0 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2.3.1	Test frequencies for NR standalone signalling testing

The default channel bandwidth for signalling test is specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR test frequencies are as follows:

-	for band with only one test frequency (e.g. n51): Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

-	for band with up to four test frequencies: Low Range (NRf1), Mid Low Range (NRf2), Mid High Range (NRf3) and High Range (NRf4);

The signalling test frequencies NRf5, NRf6, NRf7 are mapped respectively as NRf1, NRf2, NRf3 on the operating band for inter-band.

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling is specified in Table 6.2.3.1-1 (FDD FR1 BW 5MHz), Table 6.2.3.1-2 (FDD FR1 BW 10MHz), Table 6.2.3.1-3 (TDD FR1 BW 5MHz), Table 6.2.3.1-4 (TDD FR1 BW 10MHz), Table 6.2.3.1-4A (TDD FR1 BW 60MHz), Table 6.2.3.1-4B (TDD FR1 BW 20MHz for RedCap UE), Table 6.2.3.1-5 (TDD FR1 BW 100MHz), Table 6.2.3.1-5A (TDD FR1 BW 20MHz for RedCap UE),Table 6.2.3.1-6 (TDD FR2 BW 100MHz) and Table 6.2.3.1-7 (NR FDD FR1 SUL bands).

For signalling test cases requiring an additional BWP without CORESET#0, the test frequencies for Low Range (NRf1), subcarrier spacing, highest mandatory UL and DL channel bandwidth, SS/PBCH block for CD-SSB and NCD-SSB, and BWP without CORESET#0 parameters is specified in Table 6.2.3.1-X (FR1), Table 6.2.3.1-Y (FR2)

<End of modified section 1>

<Start of modified section 2>

Table 6.2.3.1-X: Test frequencies for NR FR1 bands configuring a BWP without CORESET#0

| NR Band | SCS [kHz] Note2 | CBW [MHz] Note3 | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | point A [MHz] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] Note 4 | absoluteFrequencySSB [ARFCN] Note 5 |  | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of BWP | LRB of BWP | locationAndBandwidth of BWP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 177312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.5 for bandwidth=20 MHz and SCS=15 kHz |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n8 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 188512 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.8 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n12 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 148362 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.12 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n13 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 151668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.13 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n14 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 153068 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.14 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n20 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 161712 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.20 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n24 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 306668 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.24 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n26 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 175312 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.26 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n29 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n30 | 15 | 10 | 52 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 471468 | 24 | 25 | 6624 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.30 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 24 | 25 | 6624 |
| n70 | 15 | 15 | 79 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 401562 | 54 | 25 | 6654 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.70 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 25 | 6654 |
| n71 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 126912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.71 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n76 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n91 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n92 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289912 | 80 | 25 | 6680 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.92 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 80 | 25 | 6680 |
| n93 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n100 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n1 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 431500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.1 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n2 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.2 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n3 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 370500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.3 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n7 | 15 | 50 | 270 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 533500 | 218 | 52 | 14243 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.7 for bandwidth=50 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 218 | 52 | 14243 |
| n25 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 393456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.25 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n28 | 15 | 30 | 160 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 157120 | 108 | 52 | 14133 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.28 for bandwidth=30 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 108 | 52 | 14133 |
| n65 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 425548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.65 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n66 | 15 | 40 | 216 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 429456 | 162 | 52 | 14187 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.66 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 162 | 52 | 14187 |
| n74 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 298548 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.74 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n75 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n94 | 15 | 20 | 106 | Downlink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 289948 | 54 | 52 | 14079 |
|  |  |  |  | Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.94 for bandwidth=20 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | - | 54 | 52 | 14079 |
| n34 | 15 | 15 | 79 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.34 for bandwidth=15 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 404562 | 54 | 25 | 6654 |
| n51 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n53 | 15 | 10 | 52 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.53 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 498168 | 24 | 25 | 6624 |
| n101 | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |  | NA | NA | NA | NA |
| n38 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.38 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 520952 | 80 | 24 | 6405 |
| n39 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.39 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 382952 | 80 | 24 | 6405 |
| n40 | 30 | 80 | 217 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.40 for bandwidth=80 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 475020 | 192 | 24 | 6517 |
| n48 | 30 | 40 | 106 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.48 for bandwidth=40 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 638984 | 80 | 24 | 6405 |
| n50 | 30 | 60 | 162 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.50 for bandwidth=60 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 297512 | 138 | 24 | 6463 |
| n41 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.41 for bandwidth=100 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 518310 | 222 | 51 | 13972 |
| n77 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.77 for bandwidth=100 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n78 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 626370 | 222 | 51 | 13972 |
| n79 | 30 | 100 | 273 | Downlink& Uplink | Low | Same values as for Low range in clause 4.3.1.1.1.79 for bandwidth=100 MHz and SCS=15 kHz. |  |  |  |  |  |  |  | 699704 | 222 | 51 | 13972 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Subcarrier spacing is configured according to table 6.2.3.1-1 to 6.2.3.1-5. Note 3: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in clause 4.3.1. Note 4: SS block SCS for both CD-SSB and NCD-SSB. Note 5: AbsoluteFrequencySSB for CD-SSB Note 6: n29,n75,n76 are SDL band Note 7: The maximum carrierBandwidth for n91,n93,n100,n51 and n101 with signalling test SCS in 6.2.3.1-1 to 6.2.3.1-5 are no more than 25RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-Y: Test frequencies for NR FR2 bands configuring a BWP without CORESET#0

| NR Band | SCS [kHz] Note 2 | CBW [MHz] Note 3 | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | point A [MHz] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] Note 4 | absoluteFrequencySSB [ARFCN] Note 5 |  | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of BWP | LRB of BWP | locationAndBandwidth of BWP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 120 | 200 | 264 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.2.1.1 for bandwidth=200 MHz and SCS=120 kHz. |  |  |  |  |  |  |  | 2178712 | 66 | 66 | 17941 |
| n258 | 120 | 200 | 264 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.2.1.2 for bandwidth=200 MHz and SCS=120 kHz. |  |  |  |  |  |  |  | 2028712 | 66 | 66 | 17941 |
| n259 | 120 | 200 | 264 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.2.1.3 for bandwidth=200 MHz and SCS=120 kHz. |  |  |  |  |  |  |  | 3045376 | 66 | 66 | 17941 |
| n260 | 120 | 200 | 264 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.2.1.4 for bandwidth=200 MHz and SCS=120 kHz. |  |  |  |  |  |  |  | 2878712 | 66 | 66 | 17941 |
| n261 | 120 | 200 | 264 | Downlink & Uplink | Low | Same values as for Low range in clause 4.3.1.2.1.5 for bandwidth=200 MHz and SCS=120 kHz. |  |  |  |  |  |  |  | 2245376 | 66 | 66 | 17941 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Subcarrier spacing is configured according to table 6.2.3.1-6. Note 3: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in clause 4.3.1. Note 4: SS block SCS for both CD-SSB and NCD-SSB. Note 5: AbsoluteFrequencySSB for CD-SSB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 2>