3GPP TSG-RAN5 Meeting #101	R5-236417

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4036 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UL CG TYPE1 TC 7.1.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.TC does not align with the TTCN implementation. (1)In step3, SS send 2 RLC SDUs not 8 RLC SDUs, and the other 5 RLC SDUs are sent in Step7A. (2)In step3 and Step 7A, each RLC SDU is 99 bytes. (3)In step6/step17, RRCReconfiguration does not need a limitation for Timing. (4)In step19, RLC SDU transmission by SS does not need a limitation for Timing. (5)In step20, RLC SDU transmission by UE does not need a limitation for Timing. UE cannot loop back RLC SDU on any CG type1 PUSCH occasion (N>0)because the CG type1 has been released in step17. (6) In step19 RLC SDU size if 99 bytes not 96 bytes 2.When SCS=15Khz is undertest, SS send RLC SDUs in SFN1022 in step7A and timeDomainOffset for CG TYPE1 is changed to 39. The next the CG type1 PUSCH occasion is SFN 1023, subframe9, slot 0. TC will fail because the SFN does not wrap around in Step9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update TC description to align with TTCN implementation. - Update timeDomainOffset in step6 to 19, therefore after UE receive the RLC SDUs in SFN 1022 in step7A, the next CG type1 PUSCH occasion is SFN 1, subframe9, slot 0. The SFN wrap around in Step9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - TC does not align with the TTCN implementation. - Conformant UE may fail in step9 when testing SCS 15Khz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.6.2	Correct handling of UL grant / configured grant Type 1

7.1.1.6.2.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled with Configured grant type 1 }

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

(timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)is satisfied }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(2)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a new UL grant type 1 in an RRC message }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(3)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a RRC message including sps-Configuration with rrcConfiguredUplinkGrant set as ‘release’ }

then { UE deletes the stored configured UL Grant type 1 parameters and stops transmitting UL MAC PDU’s as per configured UL grant type 1 }

}

(4)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU as per the new grant }

}

(5)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a UL grant addressed to its C-RNTI resulting in UL transmission overlap in time domain as configured grante type 1 }

then { UE transmits MAC PDU as per grant addressed to its C-RNTI }

}

7.1.1.6.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clause 10.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.4.1]

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in subclause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if the configuredGrantTimer for the corresponding HARQ process is not running:

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol=(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a repetition bundle that takes place.

NOTE 2:	A HARQ process is configured for a configured uplink grant if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes.

NOTE 3:	If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI.

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC per Serving Cell and per BWP. Multiple configurations can be active simultaneously only on different Serving Cells. For Type 2, activation and deactivation are independent among the Serving Cells. For the same Serving Cell, the MAC entity is configured with either Type 1 or Type 2.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN=0 in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

Upon configuration of a configured grant Type 1 for a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset and S (derived from SLIV as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

When a configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if the configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate an Configured Grant Confirmation MAC CE as defined in subclause 6.1.3.7;

2>	cancel the triggered configured uplink grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured uplink grant deactivation.

Retransmissions except for repetition of configured uplink grants use uplink grants addressed to CS-RNTI.

[TS 38.300, clause 10.3]

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

The dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s).

When CA is configured, at most one configured uplink grant can be signalled per serving cell. When BA is configured, at most one configured uplink grant can be signalled per BWP. On each serving cell, there can be only one configured uplink grant active at a time. A configured uplink grant for one serving cell can either be of Type 1 or Type 2. For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When SUL is configured, a configured uplink grant can only be signalled for one of the 2 ULs of the cell.

7.1.1.6.2.3	Test description

7.1.1.6.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1.

7.1.1.6.2.3.2	Test procedure sequence

Table 7.1.1.6.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration messageto configure UL configured grant type 1 in SFN 900, timeDomainOffset is set to 9. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a DL MAC PDU containing 2 RLC SDUs of size 99 bytes in SFN 1022 on UM DRB. (Note 3) | <-- | MAC PDU (two RLC SDUs) | - | - |
| 4 | Check: Does the UE transmit a MAC PDU containing first RLC SDU in Symbol ‘x0’, Slot y0’, SFN ‘z0’ after the SFN in step 3 wraps around? Where [(z0 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y0 × numberOfSymbolsPerSlot) + x0] = (9 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). (Note 4) | --> | MAC PDU (one RLC SDU) | 1 | P |
| 5 | Check: Does the UE transmit a MAC PDU containing second RLC SDU in Symbol ‘x1’, Slot y1’, SFN ‘z1’? Where [(z1 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y1 × numberOfSymbolsPerSlot) + x1] = (9 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 6 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 1, timeDomainOffset is set to 19. | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 7 | The UE transmits NR RRCReconfigurationComplete. message | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 7A | SS transmits a DL MAC PDU containing 5 RLC SDUs of size 99 bytes in SFN 1022 on UM DRB. (Note 3) | <-- | MAC PDU (five RLC SDUs) | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing first RLC SDU received in step 7A in Symbol ‘x2’, Slot y2’, SFN ‘z2’? Where [(z2 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y2 × numberOfSymbolsPerSlot) + x2] = (9 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), | --> | MAC PDU (one RLC SDU) | 2 | F |
| 9 | Check: Does the UE transmit a MAC PDU containing first RLC SDU in Symbol ‘x3’, Slot y3’, SFN ‘z3’ after the SFN in step 7A wraps around? Where [(z3 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y3 × numberOfSymbolsPerSlot) + x3] = (19 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 10 | Check: Does the UE transmit a MAC PDU containing second RLC SDU in Symbol ‘x4’, Slot y4’, SFN ‘z4’? Where [(z4 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y4 × numberOfSymbolsPerSlot) + x4] = (19 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 11 | SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 in Slot ‘p0’of PDCCH (p0 = floor ((y4 +2) * (PDCCHSCS / PUSCHSCS))), allowing the UE to retransmit one loop back SDU. | <-- | (UL Grant) | - | - |
| 12 | Check: Does the UE retransmit the MAC PDU containing the same second RLC SDU as in step 10 in Symbol ‘S’ of Slot ‘q’ of PUSCH? i.e., in the PUSCH slot q = floor (p0 * (PUSCHSCS / PDCCHSCS)) + K2. (Note 5) | --> | MAC PDU (one RLC SDU) | 4 | P |
| 13 | Check: Does the UE transmit a MAC PDU containing third RLC SDU in Symbol ‘x5’, Slot y5’, SFN ‘z5’? Where [(z5 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y5 × numberOfSymbolsPerSlot) + x5] = (19 × numberOfSymbolsPerSlot + S + 2 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 14 | SS transmits a UL Grant using UE’s C-RNTI in in Slot ‘p1’ of PDCCH allowing UE to transmit a MAC PDU containing one RLC SDU, where p1 = floor ((z6 × numberOfSlotsPerFrame - K2) * (PDCCHSCS / PUSCHSCS)). (Note 6) Where [(z6 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y6 × numberOfSymbolsPerSlot) + x6] = (19 × numberOfSymbolsPerSlot + S + 3 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | <-- | (UL Grant) | - | - |
| 15 | Check: Does the UE transmit a MAC PDU containing fourth RLC SDU in Symbol ‘x6’, Slot y6’, SFN ‘z6’? | --> | MAC PDU (one RLC SDU) | 5 | P |
| 16 | Check: Does the UE transmit a MAC PDU containing fifth RLC SDU in Symbol ‘x7’, Slot y7’, SFN ‘z7’? Where [(z7 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y7 × numberOfSymbolsPerSlot) + x7] = (19 × numberOfSymbolsPerSlot + S + 4 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 17 | After step 16, SS transmits NR RRCReconfiguration message to release UL configured grant type 1. | <-- | (NR RRC: RRCReconfiguration | - | - |
| 18 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| 19 | SS transmits a DL MAC PDU containing one RLC SDU of size 99 bytes. | <-- | MAC PDU (one RLC SDU) |  |  |
| 20 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘x8’, Slot y8’, SFN ‘z8’? Where [(z8 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y8 × numberOfSymbolsPerSlot) + x8] = (19 × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 3 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: According to the setting parameters in Table 7.1.1.6.2.3.3-2, TB size for configured grant type 1 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 4: S is the starting symbol relative to the slot of the first PUSCH transmission for new configured grant type 1. The value of S can be obtained from TS 38.508-1 [4], Table 4.6.3-122. Note 5: q is the slot where the UE shall transmit the PUSCH and is determined by as where is the slot with the scheduling DCI, is based on the numerology of PUSCH. S is the starting symbol relatived to the start of the slot q according to TS 38.214 clause 6.1.2.1. Note 6: The UL grant addressed to C-RNTI should result in UL transmission overlap in time domain as configured grant type 1. |  |  |  |  |  |

7.1.1.6.2.3.3	Specific message contents

Table 7.1.1.6.2.3.3-1: RRCReconfiguration (step 1 and step 6, Table 7.1.1.6.2.3.2-1)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |  |
|  |  | Not Present |  |  | NR |  |
| nonCriticalExtension := SEQUENCE {} |  | Not present |  |  |  | EN-DC |
| nonCriticalExtension := SEQUENCE{ |  |  |  |  | NR |  |
| masterCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.1.1.6.2.3.3-2: CellGroupConfig (Table 7.1.1.6.2.3.3-2: RRCReconfiguration)

| Derivation path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE{ |  |  |  |
| servCellIndex | Not present |  | NR |
|  | 1 |  | EN-DC |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE{ |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplink SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList { |  |  |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| timeDomainOffset | 9 |  | For Step 1 |
|  | 19 |  | For Step 6 |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 2 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=9 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 18 |  | FR1_FDD, FR1_TDD |
|  | 25 |  | FR2_TDD |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| PUSCH-TimeDomainResourceAllocationList SEQUENCE { |  |  |  |
| k2 | n4 |  | FR1 |
|  | n8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.2.3.3-3: RRCReconfiguration (step 11, Table 7.1.1.6.2.3.2-1)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |
| radioBearerConfig | Not present |  |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |  |
|  | Not present |  |  | NR |  |
| nonCriticalExtension := SEQUENCE {} |  | Not present |  |  | EN-DC |
| nonCriticalExtension := SEQUENCE{ |  |  |  | NR |  |
| masterCellGroup | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | Not present |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 7.1.1.6.2.3.3-4: CellGroupConfig (Table 7.1.1.6.2.3.3-3: RRCReconfiguration)

| Derivation path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE{ |  |  |  |
| spCellConfigDedicated SEQUENCE{ |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplink SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>