3GPP TSG-RAN5 Meeting #101	R5-236422

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4041 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS Broadcast TC 14.1.3.2-CDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Sending BroadCast MBS Packets will repeat 5 times in step6/11/… because it is not mandatory to receive every BroadCast message. The ondurationTimer for PTM is 20ms, if we assume that SS transmit 2 PDSCH in every SFN(10ms) according the data scheduldling model in 38.523-1 7.1.2.4, 20ms is not enough for 5 MBS Packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the ondurationTimer for PTM to 40ms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.3.2	MBS Broadcast/ MAC/ DRX operation

14.1.3.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and is receiving data via broadcast MRB }

ensure that {

when { Long DRX cycle for MBS Broadcast is configured for a G-RNTI and  [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM }

then { UE starts the drx-onDurationTimerPTM and monitors the PDCCH for this G-RNTI }

}

(2)

with { UE in NR RRC_INACTIVE state and  is receiving data via broadcast MRB }

ensure that {

when { Long DRX cycle for MBS Broadcast is configured for a G-RNTI and  [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM }

then { UE starts the drx-onDurationTimerPTM and monitors the PDCCH for this G-RNTI }

}

(3)

with { UE in NR RRC_CONNECTED state and is receiving data via broadcast MRB }

ensure that {

when { Long DRX cycle for MBS Broadcast is configured for a G-RNTI and  [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM }

then { UE starts the drx-onDurationTimerPTM and monitors the PDCCH for this G-RNTI }

}

14.1.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7a. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 5.7a]

For MBS broadcast, the MAC entity may be configured by RRC with a DRX functionality per G-RNTI that controls the UE's PDCCH monitoring activity for the MAC entity's G-RNTI(s) as specified in TS 38.331 [5]. When in RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED, if broadcast DRX is configured for a G-RNTI, the MAC entity is allowed to monitor the PDCCH for this G-RNTI discontinuously using the broadcast DRX operation specified in this clause; otherwise the MAC entity monitors each PDCCH for this G-RNTI as specified in TS 38.213 [6]. The broadcast DRX operation specified in this clause is performed independently for each G-RNTI and independently from the DRX operation specified in clauses 5.7 and 5.7b.

RRC controls broadcast DRX operation by configuring the following parameters:

-	drx-onDurationTimerPTM: the duration at the beginning of a DRX cycle;

-	drx-SlotOffsetPTM: the delay before starting the drx-onDurationTimerPTM;

-	drx-InactivityTimerPTM: the duration after the PDCCH occasion in which a PDCCH indicates a new DL broadcast transmission for the MAC entity;

-	drx-LongCycleStartOffsetPTM: the long DRX cycle drx-LongCycle-PTM and drx-StartOffset-PTM which defines the subframe where the DRX cycle starts.

When broadcast DRX is configured for a G-RNTI, the Active Time includes the time while:

-	drx-onDurationTimerPTM or drx-InactivityTimerPTM for this G-RNTI is running.

When broadcast DRX is configured for a G-RNTI, the MAC entity shall for this G-RNTI:

1>	if [(SFN × 10) + subframe number] modulo (drx-LongCycle-PTM) = drx-StartOffset-PTM:

2>	start drx-onDurationTimerPTM after drx-SlotOffsetPTM from the beginning of the subframe.

1>	if the MAC entity is in Active Time for this G-RNTI:

2>	monitor the PDCCH for this G-RNTI as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission for MBS broadcast:

3>	start or restart drx-InactivityTimerPTM in the first symbol after the end of the PDCCH reception.

NOTE:	If a cell is configured for MBS broadcast reception, the SFN of this cell is used to calculate the DRX duration of MBS broadcast on this cell.

14.1.3.2.3	Test description

14.1.3.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving a MBS Broadcast service with MBS Service ID '000001'H.

14.1.3.2.3.2	Test procedure sequence

Table 14.1.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 2 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1 indicating the DRX parameter for Broadcast MBS. | - | - | - | - |
| 3a1-3a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| 4 | SS transmits RRCReconfiguration to configure specific DRX parameters for unicast. | <-- | RRCReconfiguration | - | - |
| 5 | The UE transmits RRCReconfigurationComplete. | --> | RRCReconfigurationComplete | - | - |
| - | Exception: Step 6 is repeated 5 times | - | - | - | - |
| 6 | The SS transmits a MBS Packet on the MTCH with LCID=1 when the drx-onDurationTimerPTM is running and DRX for unicast is in inactive time. | <-- | MBS Packet. | - | - |
| 7 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 8 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC:UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 9 | Check: Is the number of reported MBS Packets received on the MTCH in step 8 greater than zero? | - | - | 3 | P |
| 10 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | Exception: Step 11 is repeated 5 times | - | - | - | - |
| 11 | The SS transmits a MBS Packet on the MTCH with LCID=1 when the drx-onDurationTimerPTM is running. | <-- | MBS Packet. | - | - |
| 12-19 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 2. | - | - | - | - |
| 20 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 21 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC:UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 22 | Check: Is the number of reported MBS Packets received on the MTCH in step 21 greater than the number of reported in step 8? | - | - | 1 | P |
| - | EXCEPTION: Steps 23a1-23a9 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if inactiveState is configured | - | - | - | - |
| 23a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| - | Exception: Step 23a2 is repeated 5 times | - | - | - | - |
| 23a2 | The SS transmits a MBS Packet on the MTCH with LCID=1 when the drx-onDurationTimerPTM is running. | <-- | MBS Packet. | - | - |
| 23a3 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 23a4 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 23a5 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 23a6 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 23a7 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 23a8 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC:UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 23a9 | Check: Is the number of reported MBS Packets received on the MTCH in step 23a8 greater than the number of reported in step 21? | - | - | 2 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

14.1.3.2.3.3	Specific message contents

Table 14.1.3.2.3.3-1: SIB1 of NR Cell 1 (preamble and all steps, Table 14.1.3.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.3.2.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  | 1 entry |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.3.2.3.3-2: ServingCellConfigCommonSIB (Table 14.1.3.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.3.2.3.3-3 |  |
| } |  |  |  |

Table 14.1.3.2.3.3-3: DownlinkConfigCommonSIB (Table 14.1.3.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.3.2.3.3-4 |  |
| } |  |  |  |

Table 14.1.3.2.3.3-4: BWP-DownlinkCommon (Table 14.1.3.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditioni MBS_Broadcast | Table 14.1.3.2.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-5: ACTIVATE TEST MODE (preamble, Table 14.1.3.2.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.3.2.3.3-6: MBSBroadcastConfiguration (preamble and all steps, Table 14.1.3.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA with condition DRX_MBS_Broadcast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 := SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList with condition DRX_MBS_Broadcast |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| drx-ConfigPTM-List-r17 SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17 { | 1 entry |  |  |
| DRX-ConfigPTM-r17[1] | DRX-ConfigPTM | entry 1 Table 14.1.3.2.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-7: DRX-ConfigPTM (Table 14.1.3.2.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-ConfigPTM-r17 ::= SEQUENCE { |  |  |  |
| drx-onDurationTimerPTM-r17 CHOICE { |  |  |  |
| milliSeconds | ms40 |  |  |
| } |  |  |  |
| drx-InactivityTimerPTM-r17 | ms1 |  |  |
| drx-LongCycleStartOffsetPTM-r17 CHOICE { |  |  |  |
| ms640 | 320 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-8: CLOSE UE TEST LOOP (step 3a1, Table 14.1.3.2.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C

Table 14.1.3.2.3.3-9: RRCReconfiguration (step 4, Table 14.1.3.2.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| nonCriticalExtension::= SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 14.1.3.2.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-10: CellGroupConfig (Table 14.1.3.2.3.3-10)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup | DRX-Config | Table 14.1.3.2.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-11: DRX-Config (Table 14.1.3.2.3.3-11)

| Derivation Path: 38.508-1 [4], Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms20 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.2.3.3-12: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 7, step 20 and step 23a7, Table 14.1.3.2.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.1.3.2.3.3-13: RRCRelease (step 23a1, Table 14.1.3.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 14.1.3.2.3.3-14: Paging (step 23a3, Table 14.1.3.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

<End of modified section 1>