3GPP TSG-RAN5 Meeting #101	R5-236427

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4046 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.2.X-RLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.2.1 and 14.2.2.2 needs to be added to test UE using correct set initial value for UM receive state variable in RLC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.2.1 MBS Multicast/ UM RLC / 6bit SN / Correct set initial value for UM receive state variable/ PTM and 14.2.2.2 MBS Multicast/ UM RLC / 12bit SN / Correct set initial value for UM receive state variable/ PTM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.1, 14.2.2.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.2	MBS Multicast/ RLC

14.2.2.1	MBS Multicast/ UM RLC / 6bit SN / Correct set initial value for UM receive state variable/ PTM

14.2.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission }

ensure that {

when { UE receives first UMD PDU including SN and '2^[sn-FieldLength - 1] <= SN <=  2^[sn-FieldLength] -1' }

then { The UE sets the initial value of RX_Next_Highest to the SN of the first UMD PDU including SN and initial value of RX_Next_Reassembly to a value before RX_Next_Highest and stores the UMD PDU in receive buffer }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission }

ensure that {

when { UE receives a UMD PDU, which is not the first UMD PDU, including SN and 'SN = SN of first UMD PDU - 1' before t-reassembly expiry }

then { The UE stores the UMD PDU in receive buffer }

}

(3)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission }

ensure that {

when { UE receives a UMD PDU, which is not the first UMD PDU, including SN and '(RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly'  }

then {  The UE discards the received UMD PDU }

}

(4)

with { UE in RRC_Connected state and Multicast MRB established with DL only RLC-UM entity for PTM transmission }

ensure that {

when { UE places a UMD PDU including SN into the reception buffer and all byte segments with that SN are received }

then {  UE delivers the reassembled SDU to upper layers }

}

(5)

with { UE in RRC_Connected state and using Multicast MRB with DL only RLC-UM entity for PTM transmission }

ensure that {

when { RLC re-establishment is performed upon request by RRC }

then { The UE resets initial value of RX_Next_Highest to the SN of the first received UMD PDU containing an SN and initial value of RX_Next_Reassembly to a value before RX_Next_Highest and stores the UMD PDU in receive buffer }

}

14.2.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clause 5.1.2, 5.2.2.2.2, 7.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38. 322, clause 5.1.2]

When upper layers request an RLC entity re-establishment, the UE shall:

-	discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any;

-	stop and reset all timers;

-	reset all state variables to their initial values.

[TS 38. 322, clause 5.2.2.2.2]

When an UMD PDU is received from lower layer, the receiving UM RLC entity shall:

-	if the UMD PDU header does not contain an SN:

-	remove the RLC header and deliver the RLC SDU to upper layer.

-	else if (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly:

-	discard the received UMD PDU.

-	else:

-	place the received UMD PDU in the reception buffer.

[TS 38.322, clause 7.1]

Each receiving UM RLC entity shall maintain the following state variables:

a) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0. For groupcast and broadcast of NR sidelink communication or for SL-SRB4 of NR sidelink discovery, it is initially set to the SN of the first received UMD PDU containing an SN. For the receiving UM RLC entity configured for MCCH or MTCH, it is up to UE implementation to set the initial value of RX_Next_Reassembly to a value before RX_Next_Highest.

b) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

c) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0. For groupcast and broadcast of NR sidelink communication or for SL-SRB4 of NR sidelink discovery, it is initially set to the SN of the first received UMD PDU containing an SN. For the receiving UM RLC entity configured for MCCH or MTCH, it is initially set to the SN of the first received UMD PDU containing an SN.

14.2.2.1.3	Test description

14.2.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.2.1.3.2	Test procedure sequence

Table 14.2.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2a1-2a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 3 | The SS transmits UMD PDU#3 containing first segment of RLC SDU#2 via RLC-UM of MRB, RLC SN=K.(Note 1) | <-- | MBS Packet (UMD PDU#3) | - | - |
| 4 | 20ms after step 3 the SS transmits UMD PDU#1 containing first segment of RLC SDU#1 via RLC-UM of MRB, RLC SN=K-1.(Note 1) | <-- | MBS Packet (UMD PDU#1) | - | - |
| 5 | 40 ms after step 3 the SS transmits UMD PDU#6 containing last segment of RLC SDU#3 via RLC-UM of MRB, RLC SN=K. (Note 1) | <-- | MBS Packet (UMD PDU#4) | - | - |
| 6 | 60 ms after step 3 the SS transmits UMD PDU#4 containing last segment of RLC SDU#2 via RLC-UM of MRB, RLC SN=K-1. (Note 1) | <-- | MBS Packet (UMD PDU#2) | - | - |
| 7 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 8 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 9 | Check: Is the number of reported MBS Packets received on the MRB in step 8 equal to 2? | - | - | 1,2,4 | P |
| 10 | Wait 140ms to ensure t-Reassembly expire.(Note 2) | - | - | - | - |
| 11 | The SS transmits UMD PDU#6 containing last segment of RLC SDU#3 via RLC-UM of MRB, RLC SN=0. | <-- | MBS Packet (UMD PDU#6) | - | - |
| 12 | 20ms after step 11 the SS transmits UMD PDU#5 containing first segment of RLC SDU#3 via RLC-UM of MRB, RLC SN=0. | <-- | MBS Packet (UMD PDU#5) | - | - |
| 13 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 14 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 15 | Check: Is the number of reported MBS Packets received on the MRB in step 14 equal to 2? | - | - | 3 | P |
| 16 | The SS transmits NR RRCReconfiguration message to trigger RLC re-establishment on DRB using Reconfig with sync procedure. (Note 1) | <-- | RRCReconfiguration | - | - |
| 17 | The UE transmits a NR RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 18 | The SS transmits UMD PDU#9 containing first segment of RLC SDU#5 via RLC-UM of MRB, RLC SN=L.(Note 3) | <-- | MBS Packet (UMD PDU#9) | - | - |
| 19 | 20ms after step 3 the SS transmits UMD PDU#7 containing first segment of RLC SDU#4 via RLC-UM of MRB, RLC SN=L-1.(Note 3) | <-- | MBS Packet (UMD PDU#7) | - | - |
| 20 | 40 ms after step 3 the SS transmits UMD PDU#10 containing last segment of RLC SDU#5 via RLC-UM of MRB, RLC SN=L. (Note 3) | <-- | MBS Packet (UMD PDU#10) | - | - |
| 21 | 60 ms after step 3 the SS transmits UMD PDU#8 containing last segment of RLC SDU#4 via RLC-UM of MRB, RLC SN=L-1. (Note 3) | <-- | MBS Packet (UMD PDU#8) | - | - |
| 22 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 23 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 24 | Check: Is the number of reported MBS Packets received on the MRB in step 23 equal to 4? | - | - | 5,2,4 | P |
| Note 1: K = (2^[sn-FieldLength])-1. If UM RLC sn-FieldLength =6, K=63. If UM RLC sn-FieldLength =12, K= 4095. Note 2: T-Reassembly (200ms) starts in step3. Note 3: L = 2^[sn-FieldLength-1]. If UM RLC sn-FieldLength =6, L=32. If UM RLC sn-FieldLength =12, L= 2048. |  |  |  |  |  |

14.2.2.1.3.3	Specific message contents

Table 14.2.2.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.2.1.3.3-2: RRCReconfiguration (step 1a15, Table 14.2.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and UM_PTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.2.1.3.3-4 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.2.1.3.3-3: RRCReconfiguration (step 1b10, Table 14.2.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn and MRBm and UM_PTM | n is chosen as the next available number higher or equal to 2 m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.2.1.3.3-5 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.2.1.3.3-4: CellGroupConfig (Table 14.2.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 Table 14.2.2.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.2.1.3.3-5: CellGroupConfig (Table 14.2.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm_DRBn and UM_PTM (Note 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 Table 14.2.2.1.3.3-6 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBn | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: n is set to the same value as for the radioBearerConfig IE in Table 14.2.2.1.3.3-3 and m=1. |  |  |  |

Table 14.2.2.1.3.3-6: RLC-BearerConfig (Table 14.2.2.1.3.3-4, Table 14.2.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148, condition UM_DLonly and PTM and MRBm (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| rlc-Config | RLC-Config | Table 14.2.2.1.3.3-7 |  |
| } |  |  |  |

Table 14.2.2.1.3.3-7: RLC-Config (Table 14.2.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-149, condition UM_DLonly |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| um-Uni-Directional-DL SEQUENCE { |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size6 |  |  |
| t-Reassembly | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.2.1.3.3-8: CLOSE UE TEST LOOP (step 2a1, Table 14.2.2.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.2.1.3.3-9: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 7, step13, step 22 and step 27, Table 14.2.2.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.2.1.3.3-10: RRCReconfiguration (step 16, Table 14.2.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.2.1.3.3-10 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.2.1.3.3-12 |  |
| masterKeyUpdate ::= SEQUENCE { |  |  |  |  |
| keySetChangeIndicator |  | false |  |  |
| nextHopChainingCount |  | 0 |  |  |
| nas-Container |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.2.1.3.3-11: RadioBearerConfig (Table 14.2.2.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | true |  |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | true |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs |  |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| reestablishPDCP-r17 | true |  |  |
| pdcp-Config-r17 | PDCP-Config with condition UM_MRB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.2.1.3.3-12: CellGroupConfig (Table 14.2.2.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3+n entries | n is the number of DRBs established before RRC re-establishement |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| RLC-BearerConfig[n+1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm and Re-establish_RLC | entry n+1 m=1 Table 14.2.2.1.3.3-13 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.2.1.3.3-13: RLC-BearerConfig (Table 14.2.2.1.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148, condition UM_DLonly and PTM and MRBm and Re-establish_RLC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| rlc-Config | RLC-Config | Table 14.2.2.1.3.3-7 |  |
| } |  |  |  |

14.2.2.2	MBS Multicast/ UM RLC / 12bit SN / Correct set initial value for UM receive state variable/ PTM

14.2.2.2.1	Test Purpose (TP)

Same as test purpose in clause 14.2.2.1.1

14.2.2.2.2	Conformance requirements

Same as conformance requirements in clause 14.2.2.1.2

14.2.2.2.3	Test description

14.2.2.2.3.1	Pre-test conditions

Same as pre-test conditions in clause 14.2.2.1.3.1

14.2.2.2.3.2	Test procedure sequence

Same as test procedure sequence in clause 14.2.2.1.3.2.

14.2.2.2.3.3	Specific message contents

Same as specific message contents in clause 14.2.2.1.3.3 with exception of Table 14.2.2.1.3.3-7.

Instead the table below applies:

Table 14.2.2.2.3.3-7: RLC-Config (Table 14.2.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-149, condition UM_DLonly |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| um-Uni-Directional-DL SEQUENCE { |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  |  |
| t-Reassembly | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>