3GPP TSG-RAN5 Meeting #101	R5-236432

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4051 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS Multicast TC 14.2.5.2.1-14.2.5.2.3-14.2.5.2.4-Session Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.TC 14.2.5.2.3 have two Step31. 2.Table 14.2.5.2.1.3.3-4/Table 14.2.5.2.3.3.3-11/Table 14.2.5.2.3.3.3-12/Table 14.2.5.2.3.3.3-13/Table 14.2.5.2.4.3.3-4 include wrong Tables in the comment column. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Correct the second Step31 to Step31A in TC 14.2.5.2.3. 2.Correct the comment in Table 14.2.5.2.1.3.3-4/Table 14.2.5.2.3.3.3-11/Table 14.2.5.2.3.3.3-12/Table 14.2.5.2.3.3.3-13/Table 14.2.5.2.4.3.3-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.5.2.1,14.2.5.2.3,14.2.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.5.2.1	MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Accepted

14.2.5.2.1.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state}

ensure that {

when { UE requests to establish a new PDU session associated with MBS multicast sessions and the UE at the same time intends to join two MBS multicast sessions }

then { UE sends PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" }

}

(2)

with { UE is in 5GMM-REGISTERED state and has sent PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session"}

ensure that {

when { UE receives PDU SESSION ESTABLISHMENT ACCEPT or PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with MBS decision setting to "MBS join is accepted" }

then { UE shall consider that it has successfully joined the MBS session and shall store the received TMGI}

}

14.2.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 6.4.1.2, 6.4.1.3, 6.4.2.2 and 6.3.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 6.4.1.2]

If the UE requests to:

a)	establish a new PDU session;

b)	perform handover of an existing PDU session from non-3GPP access to 3GPP access;

c)	transfer an existing PDN connection in the EPS to the 5GS according to subclause 4.8.2.3.1;

d)	transfer an existing PDN connection in untrusted non-3GPP access connected to the EPC to the 5GS; or

e)	establish user plane resources over 3GPP access of an MA PDU session established over non-3GPP access only;

and the UE at the same time intends to join one or more multicast MBS sessions that is associated to the PDU session, the UE should include the Requested MBS container IE in the PDU SESSION ESTABLISHMENT REQUEST message. In that case, the UE shall set the MBS operation to "Join multicast MBS session" and include the multicast MBS session information(s) and shall set the Type of multicast MBS session ID for each of the multicast MBS session information to either "Temporary Mobile Group Identity (TMGI)" or "Source specific IP multicast address" depending on the type of the multicast MBS session ID available in the UE. Then the remaining values of each of the multicast MBS session information shall be set as following:

a)	if the Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the UE shall set the multicast MBS session ID to the TMGI; or

b)	if the Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the UE shall set the Source IP address information and the Destination IP address information to the corresponding values.

[TS 24.501, clause 6.4.1.3]

If the PDU SESSION ESTABLISHMENT REQUEST included the Requested MBS container IE with the MBS operation set to "Join multicast MBS session", the SMF:

a)	shall include the TMGI for the multicast MBS session IDs that the UE is allowed to join, if any, in the Received MBS container IE, shall set the MBS decision to "MBS join is accepted" for each of those Received MBS information, may include the MBS start time to indicate the time when the multicast MBS session starts and shall include the MBS security container in each of those Received MBS information if security protection is applied for that multicast MBS session and the control plane security procedure is used as specified in annex W.4.1.2 in 3GPP TS 33.501 [24], and shall use separate QoS flows dedicated for multicast by including the Authorized QoS flow descriptions IE if no separate QoS flows dedicated for multicast exist or if the SMF wants to establish new QoS flows dedicated for multicast;

[TS 24.501, clause 6.4.2.2]

If the UE requests to join or leave one or more multicast MBS sessions associated with a PDU session, the UE shall include the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message and shall set the MBS operation to "Join multicast MBS session" for the join case or to "Leave multicast MBS session" for the leave case. The UE shall include the multicast MBS session information(s) and shall set the Type of multicast MBS session ID for each of the multicast MBS session information to either "Temporary Mobile Group Identity (TMGI)" or "Source specific IP multicast address" depending on the type of the multicast MBS session ID available in the UE. Then the remaining values of each of the multicast MBS session information shall be set as following:

a)	if the Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the UE shall set the multicast MBS session ID to the TMGI; or

b)	if the Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the UE shall set the Source IP address information and the Destination IP address information to the corresponding values.

[TS 24.501, clause 6.3.2.2]

If the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure and the UE has included the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message with the MBS operation set to "Join multicast MBS session", the SMF:

a)	shall include the TMGI for the multicast MBS session IDs that the UE is allowed to join, if any, in the Received MBS container IE, shall set the MBS decision to "MBS join is accepted" for each of those Received MBS information, may include the MBS start time to indicate the time when the multicast MBS session starts, and shall include the MBS security container in each of those Received MBS information if security protection is applied for that multicast MBS session and the control plane security procedure is used as specified in annex W.4.1.2 in 3GPP TS 33.501 [24], and shall use separate QoS flows dedicated for multicast by including the Authorized QoS flow descriptions IE if no separate QoS flows dedicated for multicast exist or if the SMF wants to establish new QoS flows dedicated for multicast;

14.2.5.2.1.3	Test description

14.2.5.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	UE is made interested in receiving MBS Multicast service with MBS Service ID ‘000101’H and ‘000102’H.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.2.5.2.1.3.2	Test procedure sequence

Table 14.2.5.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Check: Does steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 perform on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in two MBS Multicast session? Note: One MBS session Id is TMGI-1, and another MBS session Id is TMGI-2. | - | - | 1 | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3-17 | Check: Does UE respond to paging with TMGI-1 and receive the MRB associated with TMGI-1 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.38.2.2-1? | - | - | 2 | - |
| 18 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 19 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 20 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 21-35 | Check: Does UE respond to paging with TMGI-2 and receive the MRB associated with TMGI-2 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.38.2.2-1? | - | - | 2 | - |

14.2.5.2.1.3.3	Specific message contents

Table 14.2.5.2.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.5.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.5.2.1.3.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.1.3.3-3: PDU SESSION MODIFICATION COMMAND (step 1a15, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.1.3.3-4: RRCReconfiguration (step 1a15, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | m=1 Table 14.2.5.2.1.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 Table 14.2.5.2.1.3.3-6 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-5: RadioBearerConfig (Table 14.2.5.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 | Table 14.2.5.2.1.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.1.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-6: CellGroupConfig (Table 14.2.5.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-7: TMGI (Table 14.2.5.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  |  |
| } |  |  |  |
| serviceId-r17 | ‘000101’H | OCTET STRING (SIZE (3)) | TMGI-1 |
|  | ‘000102’H | OCTET STRING (SIZE (3)) | TMGI-2 |
| } |  |  |  |

Table 14.2.5.2.1.3.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 1b9, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  |  |  |
| MBMS Service ID |  | ‘000101’H | TMGI-2 |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.1.3.3-9: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.1.3.3-10: RRCReconfiguration (step 1b10, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.5.2.1.3.3-11 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.5.2.1.3.3-12 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-11: RadioBearerConfig (Table 14.2.5.2.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-12: CellGroupConfig (Table 14.2.5.2.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 n is set to the same value as for the radioBearerConfig IE in Table 14.2.5.2.1.3.3-11 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-13: Paging (step 3, step 21, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | TMGI with condition TMGI-1 | entry 1 Table 14.2.5.2.1.3.3-7 | Step 3 |
|  | TMGI with condition TMGI-2 | entry 1 Table 14.2.5.2.1.3.3-7 | Step 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-14: RRCReconfiguration (step 11 and step 29, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm | m=1 | Step 11 |
|  |  | RadioBearerConfig with condition MRBm | m=2 | Step 29 |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 | Step 11 |
|  |  | CellGroupConfig with condition MRBm and UM_PTM | m=2 | Step 29 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-15: CLOSE UE TEST LOOP (step 13 and step 31, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode C LB setup |  |  |  |
| MRB ID | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 ’B | MRB-Identity is 1 | Step 13 |
|  | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 ’B | MRB-Identity is 2 | Step 31 |

<End of modified section 1>

<Start of modified section 2>

14.2.5.2.3	MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Rejected / MBS session has not started or will not start soon

14.2.5.2.3.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state and has sent PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" to join two MBS multicast sessions. One MBS session Id is TMGI-1 and the other MBS session Id is TMGI-2 }

ensure that {

when { UE receives PDU SESSION ESTABLISHMENT ACCEPT or PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with two Received MBS information. One Received MBS information includes TMGI-1 and MBS decision setting to "MBS join is rejected" and Rejection cause setting to "MBS session has not started or will not start soon" and MBS back-off timer setting to neither zero nor deactivated, while another Received MBS information includes TMGI-2 and MBS decision setting to "MBS join is accepted" }

then { UE shall start a back-off timer T3587 with the value provided in the MBS back-off timer value for the received TMGI-1, and shall not attempt to join the MBS session with the same TMGI-1 until the expiry of T3587, and shall consider that it has successfully joined the MBS session with TMGI-2 and shall store the received TMGI-2. }

}

(2)

with { UE is in 5GMM-REGISTERED state and UE has received the "MBS join is rejected" with non-zero back-off timer }

ensure that {

when { upon expiry of T3587 and join this MBS session is still needed }

then { UE sends PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" }

}

(3)

with { UE is in 5GMM-REGISTERED state and has sent PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" }

ensure that {

when { UE receives PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with MBS decision  setting to "MBS join is rejected" and Rejection cause setting to "MBS session has not started or will not start soon" and MBS back-off timer setting to deactivated }

then { UE shall transport the PDU SESSION MODIFICATION COMPLETE message and shall not attempt to join the MBS session with the same TMGI until the UE is switched off, the USIM is removed }

}

(4)

with { UE is in 5GMM-REGISTERED state and UE has received the "MBS join is rejected" with MBS back-off timer setting to deactivated and UE is switched off or the USIM is removed }

ensure that {

when { UE switches on or inserts USIM again and join this MBS session is still needed }

then { UE sends PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" }

}

14.2.5.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 6.4.1.3, 6.3.2.3 and 10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 6.4.1.3]

If the PDU SESSION ESTABLISHMENT ACCEPT message includes the Received MBS container IE, for each of the Received MBS information:

…

b)	if MBS decision is set to "MBS join is rejected", the UE shall consider the requested join as rejected. The UE shall store the received MBS service area associated with the received TMGI, if any. If the received Rejection cause is set to "User is outside of local MBS service area", the UE shall not request to join the same multicast MBS session if neither current TAI nor CGI of the current cell is part of the received MBS service area. If the received Rejection cause is set to "multicast MBS session has not started or will not start soon" and an MBS back-off timer value is included with value that indicates neither zero nor deactivated, the UE shall start a back-off timer T3587 with the value provided in the MBS back-off timer value for the received TMGI, and shall not attempt to join the multicast MBS session with the same TMGI until the expiry of T3587. If the MBS back-off timer value indicates that this timer is deactivated, the UE shall not attempt to join the multicast MBS session with the same TMGI, the Source IP address information of the TMGI, or the Destination IP address information of the TMGI until the UE is switched off, the USIM is removed, or the entry in the "list of subscriber data" for the current SNPN is updated. If the MBS back-off timer value indicates zero, the UE may attempt to join the multicast MBS session with the same TMGI.

[TS 24.501, clause 6.3.2.3]

If the PDU SESSION MODIFICATION COMMAND message includes the Received MBS container IE, for each of the Received MBS informations:

…

b)	if MBS decision is set to "MBS join is rejected", the UE shall consider the requested join as rejected. The UE shall store the received MBS service area associated with the received TMGI, if any. If the received Rejection cause is set to "User is outside of local MBS service area", the UE shall not request to join the same multicast MBS session if neither current TAI nor CGI of the current cell is part of the received MBS service area. If the received Rejection cause is set to "multicast MBS session has not started or will not start soon" and an MBS back-off timer value is included with value that indicates neither zero nor deactivated, the UE shall start a back-off timer T3587 with the value provided in the MBS back-off timer value for the received TMGI, and shall not attempt to join the multicast MBS session with the same TMGI, the Source IP address information of the TMGI, or the Destination IP address information of the TMGI until the expiry of T3587. If the MBS back-off timer value indicates that this timer is deactivated, the UE shall not attempt to join the multicast MBS session with the same TMGI until the UE is switched off, the USIM is removed, or the entry in the "list of subscriber data" for the current SNPN is updated. If the MBS back-off timer value indicates zero, the UE may attempt to join the multicast MBS session with the same TMGI;

…

[TS 24.501, clause 10.3]

Table 10.3.1: Timers of 5GS session management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| T3587 | NOTE 8 | PDU SESSION ACTIVE | PDU SESSION MODIFICATION COMMAND message or PDU SESSION ESTABLISHMENT ACCEPT message received with Received MBS information that includes MBS decision set to "MBS join is rejected" and Rejection cause set to "multicast MBS session has not started or will not start soon" and an MBS back-off timer value | None | Initiating a request to join the multicast MBS session associated with the PDU session if still needed |
| … NOTE 8: The value of this timer is provided by the network in the Received MBS container IE (see subclause 6.3.2.3, subclause 6.4.1.3 and subclause 9.11.4.31). |  |  |  |  |  |

14.2.5.2.3.3	Test description

14.2.5.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving Cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	UE is made interested in receiving MBS Multicast service with MBS Service ID ‘000101’H and ‘000102H’.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.2.5.2.3.3.2	Test procedure sequence

Table 14.2.5.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1b1 to 1b12a1 of the generic procedures described in TS 38.508-1[4] subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and request to join two MBS Multicast session. One MBS Multicast session id is TMGI-1, the other MBS Multicast session id is TMGI-2. MBS Multicast session with TMGI-1 is rejected with cause "MBS session has not started or will not start soon" and back off timer setting to 1min. MBS Multicast session with TMGI-2 is accepted. Start Timer T3587= 1min at step 1a15 or step 1b10. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3 | Cause the UE to join MBS session with TMGI-1 (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest message with 20s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 5-19 | Check: Does UE respond to paging with TMGI-2 and receive the MRB associated with TMGI-2 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.38.2.2-1? | - | - | 1 | - |
| 20 | Timer T3587= 1min expires. | - | - | - | - |
| 21 | Cause the UE to join MBS session with TMGI-1 (NOTE 1) | - | - | - | - |
| 22-28 | Steps 2 to 8 of the procedure in TS 38.508-1[4] Table 4.5.4.2-3 to complete service procedure. | - | - | - | - |
| 29 | The UE transmits a PDU SESSION MODIFICATION REQUEST message to join MBS Multicast session with TMGI-1. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION REQUEST | 2 | P |
| 30 | The SS transmits a PDU SESSION MODIFICATION COMMAND to reject MBS session join request with cause "MBS session has not started or will not start soon" and back off timer setting to deactivated. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 31 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | 3 | P |
| 31A | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 32 | Cause the UE to join MBS session with TMGI-1 (NOTE 1) | - | - | - | - |
| 33 | Check: Does the UE transmit an RRCSetupRequest message within 20s? | --> | NR RRC: RRCSetupRequest | 3 | F |
| 34 | Switch off procedure in TS 38.508-1[4] Table 4.9.6.1-1 to perform in RRC_IDLE. | - | - | - | - |
| 35 | The UE is switched on or the USIM is inserted. | - | - | - | - |
| 36-55a1 | Steps 1-20a1 of the generic procedure for UE registration specified in TS 38.508-1 [4] table 4.5.2.2-2 are performed with Test Mode = Off AND Test Loop Function = Off. | - | - | - | - |
| 56a1-56b12a1 | Check: Does the UE establish an associated PDU Session to the MBS DNN and join a MBS Multicast session with TMGI-1 and a MBS Multicast session with TMGI-2 according to steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34? | - | - | 4 | - |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

14.2.5.2.3.3.3	Specific message contents

Table 14.2.5.2.3.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.5.2.3.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.5.2.3.3.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14 and step 56a14, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |

Table 14.2.5.2.3.3.3-3: PDU SESSION MODIFICATION COMMAND (step 1a15 and step 56a15, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘011’B | multicast MBS session has not started or will not start soon | Step 1a15 |
|  |  | ‘000’B | No additional information provided | Step 56a15 |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘011’B | MBS join is rejected | Step 1a15 |
|  |  | ‘010’B | MBS join is accepted | Step 56a15 |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘10’B | MBS back-off timer included | Step 1a15 |
|  |  | ‘00’B | No MBS timers included | Step 56a15 |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  | Step 56a15 |
| MBS timers |  |  |  | Step 1a15 |
| Length of GPRS Timer 3 contents |  |  |  |  |
| Unit |  | '101'B | value is incremented in multiples of 1 minute |  |
| Timer value |  | '00001'B |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.3.3.3-4: RRCReconfiguration (step 1a15, step 13 and step 56a15, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.5.2.3.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=2 | Step 1a15,Step13 |
|  |  | CellGroupConfig | Table 14.2.5.2.3.3.3-6 | Step 56a15 |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | Not present |  | Step 13 |
|  |  | DedicatedNAS-Message |  | Step 1a15, Step 56a15 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.3.3.3-5: RadioBearerConfig (Table 14.2.5.2.3.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  | Step 1a15,Step 13 |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  | Step 56a15 |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.3.3.3-6: CellGroupConfig (Table 14.2.5.2.3.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.3.3.3-7: TMGI (Table 14.2.5.2.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  |  |
| } |  |  |  |
| serviceId-r17 | ‘000101’H | OCTET STRING (SIZE (3)) | TMGI-1 |
|  | ‘000102’H | OCTET STRING (SIZE (3)) | TMGI-2 |
| } |  |  |  |

Table 14.2.5.2.3.3.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 1b9 and step 56b9, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |

Table 14.2.5.2.3.3.3-9: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10 and step 56b10, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘011’B | multicast MBS session has not started or will not start soon | Step 1b10 |
|  |  | ‘000’B | No additional information provided | Step 56b10 |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘011’B | MBS join is rejected | Step 1b10 |
|  |  | ‘010’B | MBS join is accepted | Step 56b10 |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘10’B | MBS back-off timer included | Step 1b10 |
|  |  | ‘00’B | No MBS timers included | Step 56b10 |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  | Step 56b10 |
| MBS timers |  |  |  | Step 1b10 |
| Length of GPRS Timer 3 contents |  |  |  |  |
| Unit |  | '101'B | value is incremented in multiples of 1 minute |  |
| Timer value |  | '00001'B |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.3.3.3-10: RRCReconfiguration (step 1b10 and step 56b10, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.5.2.3.3.3-11 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm_DRBn and UM_PTM | n is set to the same value as for the radioBearerConfig IE above m=2 | Step 1b10 |
|  |  | CellGroupConfig | Table 14.2.5.2.3.3.3-12 | Step 56b10 |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.3.3.3-11: RadioBearerConfig (Table 14.2.5.2.3.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  | Step 1b10 |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  | Step 56b10 |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.3.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.3.3.3-12: CellGroupConfig (Table 14.2.5.2.3.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 n is set to the same value as for the radioBearerConfig IE in Table 14.2.5.2.3.3.3-11 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.3.3.3-13: Paging (step 5, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | TMGI with condition TMGI-2 | entry 1 Table 14.2.5.2.3.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.3.3.3-14: PDU SESSION MODIFICATION REQUEST (step 29, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.3.3.3-15: PDU SESSION MODIFICATION COMMAND (step 30, Table 14.2.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘011’B | multicast MBS session has not started or will not start soon |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘011’B | MBS join is rejected |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘10’B | MBS back-off timer included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  |  |  |  |
| Length of GPRS Timer 3 contents |  |  |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated |  |
| Timer value |  | '00001'B |  |  |
| MBS security container |  | Not present |  |  |

14.2.5.2.4	MBS Multicast/ Session management / UE-requested PDU session modification / Leave MBS multicast session / Accepted

14.2.5.2.4.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state and UE has joined two MBS multicast session associated with a PDU session. One MBS session Id is TMGI-1, and another MBS session Id is TMGI-2. }

ensure that {

when { UE requests to leave MBS multicast session associated with TMGI-1 }

then { UE sends PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Leave MBS session" and multicast MBS session ID is TMGI-1 }

}

(2)

with { UE is in 5GMM-REGISTERED state and has sent PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Leave MBS session" and multicast MBS session ID is TMGI-1 }

ensure that {

when { UE receives PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with MBS decision setting to "Remove UE from MBS session" and TMGI setting to TMGI-1 }

then { UE considers that multicast MBS session associated with TMGI-1 has successfully left the MBS session and multicast MBS session associated with TMGI-2 has not successfully left the MBS session, and deletes the stored TMGI-1, and sends PDU SESSION MODIFICATION COMPLETE }

}

14.2.5.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 6.4.2.2, 6.3.2.2 and 6.3.2.3. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 6.4.2.2]

If the UE requests to join or leave one or more multicast MBS sessions associated with a PDU session, the UE shall include the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message and shall set the MBS operation to "Join multicast MBS session" for the join case or to "Leave multicast MBS session" for the leave case. The UE shall include the multicast MBS session information(s) and shall set the Type of multicast MBS session ID for each of the multicast MBS session information to either "Temporary Mobile Group Identity (TMGI)" or "Source specific IP multicast address" depending on the type of the multicast MBS session ID available in the UE. Then the remaining values of each of the multicast MBS session informations shall be set as following:

a)	if the Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the UE shall set the multicast MBS session ID to the TMGI; or

b)	if the Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the UE shall set the Source IP address information and the Destination IP address information to the corresponding values.

[TS 24.501, clause 6.3.2.2]

If:

…

b)	the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure and the UE has included the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message with the MBS operation set to "Leave multicast MBS session",

the SMF shall include the multicast MBS session IDs that the UE is removed from, if any, in the Received MBS container IE in the PDU SESSION MODIFICATION COMMAND message and shall set the MBS decision to "Remove UE from multicast MBS session" for each of those Received MBS information. The SMF may include the updated MBS service area in each of the Received MBS information, if any. The SMF may delete the QoS flows associated for the multicast by including the Authorized QoS flow descriptions IE in the PDU SESSION MODIFICATION COMMAND message.

…

[TS 24.501, clause 6.3.2.3]

If the PDU SESSION MODIFICATION COMMAND message includes the Received MBS container IE, for each of the Received MBS informations:

…

c)	if the MBS decision is set to "Remove UE from multicast MBS session", the UE shall consider that it has successfully left the multicast MBS session, and if the received Rejection cause is set to "multicast MBS session is released", the UE shall consider the multicast MBS session as released. Then the UE shall indicate to lower layers to delete the stored TMGI;

14.2.5.2.4.3	Test description

14.2.5.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	UE is made interested in receiving MBS Multicast service with MBS Service ID ‘000101’H and ‘000102’H.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.2.5.2.4.3.2	Test procedure sequence

Table 14.2.5.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in two MBS Multicast session. One MBS session Id is TMGI-1, and another MBS session Id is TMGI-2. | - | - | - | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3 | Cause the UE to leave MBS session with TMGI-1. (NOTE 1) | - | - | - | - |
| 4-10 | Steps 2 to 8 of the procedure in TS 38.508-1[4] Table 4.5.4.2-3 to complete service procedure. | - | - | - | - |
| 11 | The UE transmits a PDU SESSION MODIFICATION REQUEST message to leave MBS Multicast session associated with TMGI-1. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION REQUEST | 1 | P |
| 12 | The SS transmits a PDU SESSION MODIFICATION COMMAND to release MBS Session associated with TMGI-1. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 13 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | 2 | P |
| 14 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS transmits a Paging message including TMGI-1. | <-- | NR RRC: Paging | - | - |
| 16 | Check: Does the UE transmit an RRCSetupRequest message within 10s? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 17-31 | Check: Does UE respond to paging with TMGI-2 and receive the MRB associated with TMGI-2 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.38.2.2-1? | - | - | 2 | - |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

14.2.5.2.4.3.3	Specific message contents

Table 14.2.5.2.4.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.5.2.4.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.5.2.4.3.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.4.3.3-3: PDU SESSION MODIFICATION COMMAND (step 1a15, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.4.3.3-4: RRCReconfiguration (step 1a15, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and UM_PTM | m=1 Table 14.2.5.2.4.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 Table 14.2.5.2.4.3.3-6 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.4.3.3-5: RadioBearerConfig (Table 14.2.5.2.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 | Table 14.2.5.2.4.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.4.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.4.3.3-6: CellGroupConfig (Table 14.2.5.2.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.4.3.3-7: TMGI (Table 14.2.5.2.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  |  |
| } |  |  |  |
| serviceId-r17 | ‘000101’H | OCTET STRING (SIZE (3)) | TMGI-1 |
|  | ‘000102’H | OCTET STRING (SIZE (3)) | TMGI-2 |
| } |  |  |  |

Table 14.2.5.2.4.3.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 1b9, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  |  |  |
| MBMS Service ID |  | ‘000101’H | TMGI-2 |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.4.3.3-9: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.4.3.3-10: RRCReconfiguration (step 1b10, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.5.2.4.3.3-11 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.5.2.4.3.3-12 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.4.3.3-11: RadioBearerConfig (Table 14.2.5.2.4.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.4.3.3-12: CellGroupConfig (Table 14.2.5.2.4.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 n is set to the same value as for the radioBearerConfig IE in Table 14.2.5.2.4.3.3-11 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.4.3.3-13: PDU SESSION MODIFICATION REQUEST (step 11, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘10’B | Leave multicast MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |

Table 14.2.5.2.4.3.3-14: PDU SESSION MODIFICATION COMMAND (step 12, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘100’B | Remove UE from multicast MBS session |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.4.3.3-15: Paging (step 15 and step 17, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | TMGI with condition TMGI-1 | entry 1 | Step 15 |
|  | TMGI with condition TMGI-2 | entry 1 | Step 17 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.4.3.3-16: RRCReconfiguration (step 25, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm | m=2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.4.3.3-17: CLOSE UE TEST LOOP (step 27, Table 14.2.5.2.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode C LB setup |  |  |  |
| MRB ID | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 ’B | MRB-Identity is 2 |  |

<End of modified section 2>