Title: 	Discussion on test point selection for MPR, ACLR, and SEM for NR-U in FR1

Source: 	Ericsson

# Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR shared spectrum channel access, standalone NR-U configuration, for Maximum Power Reduction (MPR), Spectrum Emission mask (SEM) and Adjacent Channel Leakage Ratio (ACLR) tests in FR1.

# Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## Test Environment

The test environment for basic NR FR1 test cases is selected as in table 2.1-1. The same environment selection can be adopted for NR-U test cases.

Table 2.1-1 Test environment configuration summary

| Test Case | Test Environment |
| --- | --- |
| MPR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| ACLR | Normal,TL/VL, TL/VH, TH/VL and TH/VH |
| SEM | NC |

Proposal 1: Define test environment for NR-U test cases as table 2.1-1.

## Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest and Highest for MPR, ACLR and SEM for basic test cases in FR1. The same approach can be followed for NR-U test cases.

Proposal 2: Select Lowest and Highest SCS supported for UE for MPR, ACLR and SEM for NR-U in FR1.

  - Test Frequencies

Test frequencies are selected as Low and High range for MPR, ACLR and SEM for basic test cases in FR1. The same approach can be followed for NR-U test cases.

Proposal 3: Test Low and High range frequencies for MPR, ACLR and SEM for NR-U in FR1.

## Test Channel Bandwidth

Test channel bandwidth is selected as Lowest and Highest range for MPR, ACLR and SEM for basic test cases in FR1. Band n46 supports 10 MHz bandwidth but, as per Table 5.3.5-1 in [2], this bandwidth is restricted to operation when carrier is configured as a downlink SCell part of CA configuration. Therefore, for NR-U standalone configuration select 20 MHz + Highest range as test channel bandwidth.

Proposal 4: Test 20 MHz and Highest channel bandwidth for MPR, ACLR and SEM for NR-U in FR1.

## Modulations and RB allocation

The MPR requirements are defined for full and partial RB allocation, and each applicable modulation type and pre-coding

Table 6.2F.2-1 Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-OFDM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 6.2F.2-2 MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

For MPR, ACLR and SEM for basic test cases in FR1, Edge_1RB was selected over other RB allocations due to high PSD. Due to resource allocation type 2, 1RB allocation is not possible. Instead, lowest possible partial allocation of one interlace could be selected such as the selected interlace index “hits” the lowest/highest RB within the BW/active sub-band.

Proposed allocation “Partial_Left” and “partial_Right for” SCS 30 kHz, BW 20 MHz (same approach for higher BWs):

Proposed allocation “Partial_Left” for SCS 15 kHz, BW 20 MHz (same approach for higher BWs):

Proposed allocation “Partial_Right” for SCS 15 kHz, BW 20 MHz (same approach for higher BWs):

In order to reduce the testing workload, ”Partial_left” is tested only on Low range frequency and ”Partial_Right” is tested only on High range frequency. The following bitmaps could apply for wideband operation:

”Partial_left” with bitmap 10 for 40MHz wideband BW is tested only on Low range frequency.

”Partial_right” with bitmap 01 for 40MHz wideband BW is tested only on High range frequency.

”Partial_left” with bitmap 100 for 60MHz wideband BW is tested only on Low range frequency.

”Partial_right” with bitmap 001 for 60MHz wideband BW is tested only on High range frequency.

”Partial_left” with bitmap 1000 for 80MHz wideband BW is tested only on Low range frequency.

”Partial_right” with bitmap 0001 for 80MHz wideband BW is tested only on High range frequency.

Full allocation is also selected to cover all MPR requirements. This would correspond to bitmap 11 for 40 MHz wideband BW, bitmap 111 for 60 MHz wideband BW, bitmap 1111 for 80 MHz wideband bandwidth. Full allocation would correspond to all interlace indexes (0-9 for SCS 15 kHz and 0-5 for SCS 30 kHz) being scheduled.

Contiguous RB allocation is not possible for partial allocation due to the interlace structure. Since DFT-s-OFDM is only applicable for contiguous RB allocation, DFT-s-OFDM is not selected in conjunction with partial RB allocation.

Proposal 5: The combination of modulation and RB allocation is selected as Table 2.5-1.

Table 2.5-1 Combinations of modulation and RB allocation for UE power class 5

| Modulation | RB allocation |
| --- | --- |
| DFT-s-OFDM PI/2 BPSK1 | Full |
| DFT-s-OFDM QPSK | Full |
| DFT-s-OFDM 16 QAM | Full |
| DFT-s-OFDM 64 QAM | Full |
| DFT-s-OFDM 256 QAM | Full |
| CP-OFDM QPSK | “Partial_Left” |
| CP-OFDM QPSK | “Partial_Right” |
| CP-OFDM QPSK | Full |
| CP-OFDM 16 QAM | “Partial_Left” |
| CP-OFDM 16 QAM | “Partial_Right” |
| CP-OFDM 16 QAM | Full |
| CP-OFDM 64 QAM | “Partial_Left” |
| CP-OFDM 64 QAM | “Partial_Right” |
| CP-OFDM 64 QAM | Full |
| CP-OFDM 256 QAM | “Partial_Left” |
| CP-OFDM 256 QAM | “Partial_Right” |
| CP-OFDM 256 QAM | Full |
| DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3 | Full |
| NOTE 1: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 2: “Partial_Left” is only for Low range test frequency, and “Partial_Right” is only for High range test frequency. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |

Based on above proposal, the total test points are tabulated below:

Table 2.6-1 Total test points of MPR, SEM and ACLR

| Test case | Environment conditions | Test SCS | Test Frequencies | Test ChBW | Number of steps (mod and RB) | Total Test Points |
| --- | --- | --- | --- | --- | --- | --- |
| MPR | 5 | 2 | 2 for Full allocation 1 for Partial allocation | 2 | 10 for Full allocation, 10 for Partial allcoation | 600 |
| SEM | 1 | 2 | 2 for Full allocation 1 for Partial allocation | 2 | 10 for Full allocation, 10 for Partial allcoation | 120 |
| ACLR | 5 | 2 | 2 for Full allocation 1 for Partial allocation | 2 | 10 for Full allocation, 10 for Partial allcoation | 600 |

# Conclusion

It is proposed that RAN5 agrees below proposals on test points of MPR, SEM and ACLR for NR-U test cases:

Proposal 1: Define test environment for NR-U test cases as table 3-1.

Table 3-1 Test environment configuration summary

| Test Case | Test Environment |
| --- | --- |
| MPR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| ACLR | Normal, TL/VL, TL/VH, TH/VL and TH/VH |
| SEM | NC |

Proposal 2: Select Lowest and Highest SCS supported for UE for MPR, ACLR and SEM for NR-U in FR1.

Proposal 3: Test Low and High range frequencies for MPR, ACLR and SEM for NR-U in FR1.

Proposal 4: Test 20 MHz and Highest channel bandwidth for MPR, ACLR and SEM for NR-U in FR1

Proposal 5: The combination of modulation and RB allocation is selected as Table 2.5-1

# References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v16.16.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[3] 3GPP TS 38.521-1 v17.9.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[4] R5-197586 Discussion on test point selection for MPR, NR ACLR and SEM in FR1 in TR 38.905