3GPP TSG RAN WG5#101	R5-236494

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2494 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aligning test configuration for SEM, ACLR and MPR test cases with TP analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align test configuration with TP analysis |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning test configurtion tables for SEM, ACLR and MPR test cases as per TP analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuartion will not be aligned with test configuration table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.2.4.1, 6.5F.2.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | TP analysis in R5-236482 |  |  |  |  |  |  |  |  |

#### << Start of changes >>

Table 6.2F.2.4.1-1: Test Configuration Table for power class 5

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | 20 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 12 | Default |  | DFT-s-OFDM Pi/2 BPSK | Full |
| 2 | Default |  | DFT-s-OFDM QPSK | Full |
| 3 | Default |  | DFT-s-OFDM 16 QAM | Full |
| 4 | Default |  | DFT-s-OFDM 64 QAM | Full |
| 5 | Default |  | DFT-s-OFDM 256 QAM | Full |
| 6 | Low |  | CP-OFDM QPSK | Partial_Left |
| 7 | High |  | CP-OFDM QPSK | Partial_Right |
| 8 | Default |  | CP-OFDM QPSK | Full |
| 9 | Low |  | CP-OFDM 16 QAM | Partial_Left |
| 10 | High |  | CP-OFDM 16 QAM | Partial_Right |
| 11 | Default |  | CP-OFDM 16 QAM | Full |
| 12 | Low |  | CP-OFDM 64 QAM | Partial_Left |
| 13 | High |  | CP-OFDM 64 QAM | Partial_Right |
| 14 | Default |  | CP-OFDM 64 QAM | Full |
| 15 | Low |  | CP-OFDM 256 QAM | Partial_Left |
| 16 | High |  | CP-OFDM 256 QAM | Partial_Right |
| 17 | Default |  | CP-OFDM 256 QAM | Full |
| 18 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3 | Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1F-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 and when IE powerBoostPi2BPSK is set to 0. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

#### << Clauses omitted here >>

Table 6.5F.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | 20 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 12 | Default |  | DFT-s-OFDM Pi/2 BPSK | Full |
| 2 | Default |  | DFT-s-OFDM QPSK | Full |
| 3 | Default |  | DFT-s-OFDM 16 QAM | Full |
| 4 | Default |  | DFT-s-OFDM 64 QAM | Full |
| 5 | Default |  | DFT-s-OFDM 256 QAM | Full |
| 6 | Low |  | CP-OFDM QPSK | Partial_Left |
| 7 | High |  | CP-OFDM QPSK | Partial_Right |
| 8 | Default |  | CP-OFDM QPSK | Full |
| 9 | Low |  | CP-OFDM 16 QAM | Partial_Left |
| 10 | High |  | CP-OFDM 16 QAM | Partial_Right |
| 11 | Default |  | CP-OFDM 16 QAM | Full |
| 12 | Low |  | CP-OFDM 64 QAM | Partial_Left |
| 13 | High |  | CP-OFDM 64 QAM | Partial_Right |
| 14 | Default |  | CP-OFDM 64 QAM | Full |
| 15 | Low |  | CP-OFDM 256 QAM | Partial_Left |
| 16 | High |  | CP-OFDM 256 QAM | Partial_Right |
| 17 | Default |  | CP-OFDM 256 QAM | Full |
| 18 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3 | Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1F-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 and when IE powerBoostPi2BPSK is set to 0. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

#### << End of changes >>