3GPP TSG-RAN5 Meeting #101	R5-236508

# Chicago, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0558 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align with the outcome of the Tdoc R5-225274 to update IMS TCs with NG.114 default EVS configuration A2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated to use the generic procedure A.4.1a and A.4.2a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aligment to use default EVS configuration cannot be completed. The test case applicability cannot be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0333 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 7.2	MTSI MO Voice Call / 504 Server Time-out / 5GS

7.2.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and UE having sent an INVITE request }

ensure that {

when { UE receives 504 Server Time-out response }

then { UE performs initial registration to IMS }

}

7.2.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.2A.1.6]

In the event the UE receives a 504 (Server Time-out) response containing:

1)	a P-Asserted-Identity header field set to a value equal to a URI:

a)	from the Service-Route header field value received during registration; or

b)	from the Path header field value received during registration; and

NOTE 1:	If there are multiple registration flows associated with the registration, then the UE has received from the P-CSCF during registration multiple sets of Path header field and Service-Route header field values. The Path header field value and Service-Route header field value corresponding to the flow on which the 504 (Server Time-out) response was received are checked.

2)	a Content-Type header field set according to subclause 7.6 (i.e. "application/3gpp-ims+xml"), independent of the value or presence of the Content-Disposition header field, independent of the value or presence of Content-Disposition parameters,

then the following treatment is applied:

a)	if the 504 (Server Time-out) response includes an IM CN subsystem XML body as described in subclause 7.6 with the <ims-3gpp> element, including a version attribute, with the <alternative-service> child element:

A)	with the <type> child element set to "restoration" (see table 7.6.2); and

B)	with the <action> child element set to "initial-registration" (see table 7.6.3);

then the UE:

-	shall initiate S-CSCF restoration procedures by performing an initial registration as specified in subclause 5.1.1.2; and

-	may provide an indication to the user based on the text string contained in the <reason> child element of the <alternative-service> child element of the <ims-3gpp> element.

NOTE 2:	If the UE has discovered multiple P-CSCF addresses and has information that the P-CSCF was unable to forward the request resulting in sending back the 504 (Server Time-out) response, when starting the initial registration it is appropriate for the UE to select a P-CSCF address different from the one used for the registration binding on which the 504 (Server Time-out) response was received.

7.2.3	Test description

7.2.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The UE is either configured to use preconditions or to not use preconditions.

Preamble:

-	UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.2.3.2	Test procedure sequence

Table 7.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to INVITE at Steps 8 a or 8b, step described in Table 7.2.3.2-2: Parallel behaviour takes place. | --> | - | - | - |
| - | EXCEPTION: Steps 8a to 8b describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 8a | IF the UE is configured to use preconditions THEN step 1 of Annex A.4.1a takes place. | --> | INVITE | - | - |
| 8b | ELSE step 1 of Annex A.4.2a takes place. | --> | INVITE | - | - |
| 9 | SS sends 504 Server Time-out | <-- | 504 Server Time-out | - | - |
| 9A | UE acknowledges the reception of 504 Server Time-out. | --> | ACK | - | - |
| 10 | Check: Does the UE send an initial registration request? | --> | REGISTER | 1 | P |
| 11-17 | Continue with Annex A.2 steps 2-8 in order to get the UE in a stable registered state. | - | - | - | - |

Table 7.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

Table 7.2.3.3-3: ACK (step 9A, Table 7.2.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.7 Conditions A1 and A4

7.2.3.3	Specific message contents

Table 7.2.3.3-1: 504 Server Time-out (step 3, Table 7.2.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.4.6

Table 7.2.3.3-2: REGISTER (step 4, Table 7.2.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.1.1 conditions A1 and A32

<End of modified section>