Title:	Discussion on test point selection for NS_28 and NS_29 A-MPR in FR1 for 6.2F.3 shared spectrum access

Document for:	Discussion and approval

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being stated in 4.1.1.6 [7]. This is also valid for test point selection for A-MPR in FR1.

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_28 and NS_29. NS_28 and NS_29 is intended for band n46 (5150 MHz – 5925 MHz).

# Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. There are no RB allocation specific or carrier center frequency specific A-MPR requirements for NS_28 and NS_29, see Table 1. Only modulation and waveform dependence.

Table 1: A-MPR for NS_28 power class 5(from [4])

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK4 | ≤ 4.0 | ≤ 6.0 | See Table 6.2F.2-1 |
|  | QPSK | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 4.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 5.5 | ≤ 6.5 |  |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 |  |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

Table 2: A-MPR for NS_29 power class 5(from [5])

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz |  | 60 MHz, 80 MHz |  |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | See Table 6.2F.2-1 | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | QPSK |  | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 2.5 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: Larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

## Test Environment

Based on the baseline agreed in [7], the test environment for legacy test case 6.2.3 can be used.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_28 and NS_29.

## Test Frequencies

The selection of test frequencies can be based on being stated in 4.1.1.6 [7], i.e. Low and High frequency. No additional test frequencies are needed for NS_28 and NS_29, see Table 1 and Table 2.

PROPOSAL 2: Select Low range and High range test frequencies.

## Test Channel Bandwidth

Minimum requirements for NS_28 and NS_29 apply for 20, 40, 60 and 80 MHz channel bandwidth.

PROPOSAL 3: Select for 20, 40, 60 and 80 MHz test channel bandwidth for NS_28 and NS_29 A-MPR in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on being stated in 4.1.1.6 [7] and legacy test case 6.2.3, i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_28 and NS_29 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on being stated in 4.1.1.6 [7] and legacy test case 6.2.3, i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_28 and NS_29 A-MPR in FR1.

## Uplink RB allocation

The UL RB allocation are defined for full and partial RB allocation according to Table 1 and Table 2. For requirement full allocation full will, be used. However, for the partial RB allocation requirement basic test cases in FR1, Edge_1RB right or left was selected. Due to resource allocation type 2, 1RB allocation is not possible. Instead, lowest possible partial allocation of one interlace could be selected. Bellow example for BW 20MHz:

Proposed allocation “Partial_Left” and “partial_Right for” SCS 30 kHz, BW 20 MHz:

Proposed allocation “Partial_Left” for SCS 15 kHz, BW 20 MHz:

Proposed allocation “Partial_Right” for SCS 15 kHz, BW 20 MHz:

PROPOSAL 6: For NS_28 and NS_29 base the uplink RB allocation on Partial left right and full.

2.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_28 and NS_29:

| Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Frequencies | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 (NC) | 2 (L/H) | 4 | 9 | 2 (L/H) | 144 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_28 and NS_29 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies.

PROPOSAL 3: Select for 20, 40, 60 and 80 MHz test channel bandwidth for NS_28 and NS_29 A-MPR in FR1.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_28 and NS_29 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_28 and NS_29 A-MPR in FR1.

PROPOSAL 6: For NS_28 and NS_29 base the uplink RB allocation on Partial left right and full.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.3.0 (2023-09), Table 6.2F.3.2-1

- 3GPP TS 38.101-1, V18.3.0 (2023-09), Table 6.2F.3.3-1

- 3GPP TS 38.521-1, V18.0.0 (2023-09), Table 6.1-1

- 3GPP TS 38.521-1, V18.0.0 (2023-09, Clause 4.1.1.6