3GPP TSG-RAN5 Meeting #101	R5-236550

# Chicago, US, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5480 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to RefSens test cases 7.3A.10 for 5DL CA combo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, Sporton International |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Some CA configurations are incorrectly indicated value in test requirement; - Table format is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated test requirement value to CA_2A-2A-12A-66A-66A and CA_2A-5A-30A-66A-66A based on TP analysis. - Editorial correction to format of Test Requirement table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA band combinations will missing/incorrect in test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Changes covered closed WIC “LTE_CA_Rel13-UEConTest”, “LTE_CA_R14-UEConTest” and “LTE_CA_R15-UEConTest” TP analysis in R5-236397 (36.905 CR0263) and R5-236396 (36.905 CR0262) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

### 7.3A.10	Reference sensitivity level for 5DL CA

Editor’s note: Following aspects missing or not yet determined:

-	Test configuration table is under investigating

-	Test requirement for some CA configuration is TBD

-	Test applicability with related information in Table 7.1-2a needs to add to TS36.521-2

#### 7.3A.10.1	Test purpose

Same as in clause 7.3A.1.1.

#### 7.3A.10.2	Test applicability

This test case applies to all types of E-UTRA UE and forward that support 5DL with CA configurations in Table 7.1-2c.

#### 7.3A.10.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3A.0.

#### 7.3A.10.4	Test description

##### 7.3A.10.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1 through table 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.3A.10.4.1-1 through table 7.3A.10.4.1-2 as appropriate. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.10.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  | SCC3 |  |  |  |  |  |  | SCC4 |  |  |  |  |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range |  | NRB |  |  |  | Band |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD | ULalloc (Note 2,3,4) | DL alloc | UL MOD | DL MOD | UL alloc | DL alloc | UL MOD | UL MOD | DL MOD | UL alloc | UL MOD | DL MOD |  | UL alloc |  | DL alloc |  | UL MOD |  | DL MOD |  | UL alloc |  | DL alloc |  |
| Default Test Settings for a CA_XF Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | X | Low/CC3 | N/A | ALL RBs | X | Low/CC4 |  | N/A |  | ALL RBs |  | X |  | Low/CC5 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |
| 2 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | X | Low/CC3 | N/A | ALL RBs | X | Low/CC4 |  | N/A |  | ALL RBs |  | X |  | Low/CC5 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 3 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | X | High/CC3 | N/A | ALL RBs | X | High/CC4 |  | N/A |  | ALL RBs |  | X |  | High/CC5 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |
| 4 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | X | High/CC3 | N/A | ALL RBs | X | High/CC4 |  | N/A |  | ALL RBs |  | X |  | High/CC5 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| Default Test Settings for a CA_XE-YA, YA-XE Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB | N/A | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 |  | N/A |  | ALL RBs |  | X |  | Mid/CC4 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 4 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 |  | N/A |  | ALL RBs |  | X |  | Mid/CC4 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |
| Default Test Settings for a CA_XC-YD, YD-XC, YD-XB, and CA_XB-YD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A |  | ALL RBs |  | Y |  | Mid/CC3 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A |  | ALL RBs |  | Y |  | Mid/CC3 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 3 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A |  | ALL RBs |  | Y |  | Mid/CC3 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A |  | ALL RBs |  | Y |  | Mid/CC3 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 5 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | ALL RBs | Y | Low/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 6 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | ALL RBs | Y | Low/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 7 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | ALL RBs | Y | High/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 8 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | ALL RBs | Y | High/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| Default Test Settings for a CA_XA-YC-ZC, CA_YC-XA-ZC and CA_YC-ZC-XA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A |  |  |  | N/A |  |  |  |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | ALL RBs | Z | Mid/CC1 |  | ALL RBs | Z | Mid/CC2 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | Highest NRB_agg | N/A | QPSK | N/A | Highest NRB | N/A | QPSK |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | ALL RBs | Z | Mid/CC1 |  | ALL RBs | Z | Mid/CC2 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 3 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | Lowest NRB_agg | N/A | QPSK | N/A | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | ALL RBs | Z | Mid/CC1 |  | ALL RBs | Z | Mid/CC2 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | Highest NRB_agg | N/A | QPSK | N/A | Highest NRB | N/A | QPSK |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | ALL RBs | Z | Mid/CC1 |  | ALL RBs | Z | Mid/CC2 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 5 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | Lowest NRB_agg | N/A | QPSK | N/A | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | ALL RBs | X | Mid |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 6 | Y | Low/CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | Highest NRB_agg | N/A | QPSK | N/A | Highest NRB | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | ALL RBs | X | Mid |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 7 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | Lowest NRB_agg | N/A | QPSK | N/A | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | ALL RBs | X | Mid |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 8 | Y | High/CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | Highest NRB_agg | N/A | QPSK | N/A | Highest NRB | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | ALL RBs | X | Mid |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 9 | X | Mid | N/A | ALL RBs | Y | Low/CC1 | REFSENS | Lowest NRB_agg | N/A | QPSK | REFSENS | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB | QPSK | QPSK |  | ALL RBs | Y | Low/CC2 |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 10 | X | Mid | N/A | ALL RBs | Y | Low/CC1 | REFSENS | Highest NRB_agg | N/A | QPSK | REFSENS | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | ALL RBs | Y | Low/CC2 |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 11 | X | Mid | N/A | ALL RBs | Y | High/CC1 | REFSENS | Lowest NRB_agg | N/A | QPSK | REFSENS | Lowest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB | QPSK | QPSK |  | ALL RBs | Y | High/CC2 |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 12 | X | Mid | N/A | ALL RBs | Y | High/CC1 | REFSENS | Highest NRB_agg | N/A | QPSK | REFSENS | Highest NRB_agg | N/A | QPSK |  | N/A |  | ALL RBs |  | Z |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | ALL RBs | Y | High/CC2 |  | ALL RBs | Z | Mid/CC1 |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| Default Test Settings for a CA_XA-YA-ZA-RA-SA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid |  | N/A |  | ALL RBs |  | S |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | Y | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid |  | N/A |  | ALL RBs |  | S |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | Z | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | R | Mid |  | N/A |  | ALL RBs |  | S |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | R | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | S |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 5 | S | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| Default Test Settings for CA_XC-YA-ZA-RA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | X | Low/CC1 | REFSENS | ALL RBs | X | Low/CC2 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | X | High/CC1 | REFSENS | ALL RBs | X | High/CC2 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 5 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 6 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 7 | R | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid |  | N/A |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| Default Test Settings for a CA_XA-YA-ZD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Z | Mid/CC1 | REFSENS | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 | N/A | ALL RBs | X | Mid |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 2 | Z | Mid/CC1 | REFSENS | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 | N/A | ALL RBs | X | Mid |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | X | Mid | REFSENS | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 4 | X | Mid | REFSENS | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 |  | N/A |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 5 | Y | Mid | REFSENS | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| 6 | Y | Mid | REFSENS | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC3 |  | N/A |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |
| Default Test Settings for CA_XA-ZA-RA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Y | Low/ CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 2 | Y | Low/ CC1 | REFSENS | ALL RBs | Y | Low/CC2 | N/A | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 3 | Y | High/ CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 4 | Y | High/ CC1 | REFSENS | ALL RBs | Y | High/CC2 | N/A | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 5 | X | Mid | REFSENS | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 6 | Z | Mid | REFSENS | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid |  | N/A |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| 7 | R | Mid | REFSENS | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid |  | N/A |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | Highest NRB | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |
| Test Settings for a CA_1A-3A-18A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 2 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 3 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 4 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 5 | 1 | Low | 50@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 10 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 6 | 1 | Low | 75@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 15 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 7 | 1 | Low | 100@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 8 | 1 | Low | 100@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 9 | 1 | Mid | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 10 | 1 | Low | 100@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 11 | 1 | Mid | 100@0 | ALL RBs | 3 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 12 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 13 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 14 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 15 | 3 | Low | 50@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 5 |  |
| 16 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Low |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 17 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 18 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 19 | 3 | High | 50@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 20 | 3 | Low | 50@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 18 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 10 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 21 | 3 | Low | 50@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 18 | Mid | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 15 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 22 | 18 | Low | 25@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 10 | N/A | QPSK |  | 10 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 5 |  |
| 23 | 18 | Low | 25@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 10 | N/A | QPSK |  | 15 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 24 | 18 | Low | 25@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | Mid |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 10 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 25 | 18 | Low | 25@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 10 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 26 | 18 | Mid | 25@25 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 15 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 27 | 18 | Mid | 25@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low | N/A | ALL RBs | 42 | High |  | N/A |  | ALL RBs |  | 42 |  | High |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 15 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 20 |  |
| 28 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 10 |  |
| 29 | 42 | Low | 100@0 | ALL RBs | 42 | Low | N/A | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 10 |  |
| 30 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 10 |  |
| 31 | 42 | Low | 100@0 | ALL RBs | 42 | Low | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 10 |  |
| 32 | 42 | Low | 100@0 | ALL RBs | 42 | Low | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 10 |  |
| 33 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 5 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 34 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 35 | 42 | Low | 100@0 | ALL RBs | 42 | Low | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 36 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Low | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 37 | 42 | Mid | 100@0 | ALL RBs | 42 | Mid | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 18 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 38 | 42 | Mid | 100@0 | ALL RBs | 42 | Mid | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| 39 | 42 | High | 100@0 | ALL RBs | 42 | High | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Low |  | N/A |  | ALL RBs |  | 18 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  | 20 | N/A | QPSK |  |  |  | 20 |  | N/A |  | QPSK |  |  |  | 15 |  |
| Test Settings for a CA_1A-3A-19A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 712 | 1 | Mid | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 1 | Mid | 50@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 1 | Mid | 75@0 | ALL RBs | 3 | High | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 1 | Mid | 100@0 | ALL RBs | 3 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 11 | 3 | High | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 12 | 3 | High | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 13 | 3 | High | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 14 | 3 | High | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1513 | 3 | High | 50@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1614 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1715 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1814 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1915 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 20 | 19 | High | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 21 | 19 | High | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 22 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 23 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 24 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 25 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 26 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 27 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 28 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | High |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 29 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 30 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 19 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 75 |  |
| Test Settings for a CA_1A-3A-41C-42A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 1 | Mid | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 | High | 100@0 | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 1 | Mid | 45@0 | ALL RBs | 3 | Low | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 3 | High | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 3 | Mid | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1014 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Low |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1115 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 |  | N/A |  | ALL RBs |  | 42 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 12 | 42 | Mid | 100@0 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 13 | 42 | Low | 100@0 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 14 | 42 | High | 100@0 | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for a CA_1A-3A-41A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 | Low | 25@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 1 | Mid | 45@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 1 | Mid | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 1 | Mid | 100@0 | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 1 | Low | 100@0 | ALL RBs | 3 | High | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 1 | High | 100@0 | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 3 | High | 50@50 | ALL RBs | 1 | Low | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 3 | Mid | 50@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1114 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1215 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1316 | 3 | Low | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 1417 | 3 | High | 50@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 15 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 41 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 25 | QPSK | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 16 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 41 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 17 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 41 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 25 | QPSK | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 18 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 41 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 19 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 3 | Mid |  | N/A |  | ALL RBs |  | 41 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | QPSK | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for a CA_1A-19A-21A-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Mid | 75@0 | ALL RBs | 19 | High | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 1 | Mid | 75@0 | ALL RBs | 19 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 | Mid | 100@0 | ALL RBs | 19 | High | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 1 | Mid | 100@0 | ALL RBs | 19 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 1 | Mid | 50@0 | ALL RBs | 19 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 1 | Mid | 75@0 | ALL RBs | 19 | Mid | N/A | ALL RBs | 21 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 19 | High | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 19 | High | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 11 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 12 | 19 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 21 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 13 | 21 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 14 | 21 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 15 | 21 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 16 | 21 | Mid | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 17 | 21 | High | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 75 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 18 | 21 | Mid | 25@25 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | High | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 19 | 21 | High | 25@50 | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 20 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid |  | N/A |  | ALL RBs |  | 21 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 21 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid |  | N/A |  | ALL RBs |  | 21 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 22 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid |  | N/A |  | ALL RBs |  | 21 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 23 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid |  | N/A |  | ALL RBs |  | 21 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 75 |  |
| 24 | 42 | High/CC2 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 19 | Mid |  | N/A |  | ALL RBs |  | 21 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 75 |  |
| Test Settings for CA_1A-41C-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | Low | 100@0 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 1 | Mid | 100@0 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 1 | High | 100@0 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  | ALL RBs |  | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 42 | Low/CC1 | 50@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 42 | High/CC1 | 50@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 1 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  | ALL RBs |  | 41 |  | Mid/CC1 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_2A-5B-30A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | Mid | 100@0 | ALL RBs | 5 | Mid/CC1 | N/A | ALL RBs | 5 | Mid/CC2 | N/A | ALL RBs | 30 | Mid |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 2 | 5 | Low/CC1 | 50@0 | ALL RBs | 5 | Low/CC2 | N/A | ALL RBs | 2 | Mid | N/A | ALL RBs | 30 | Mid |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 3 | 5 | Low/CC1 | 25@0 | ALL RBs | 5 | Low/CC2 | N/A | ALL RBs | 2 | Mid | N/A | ALL RBs | 30 | Mid |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 5 | High/CC1 | 50@0 | ALL RBs | 5 | High/CC2 | N/A | ALL RBs | 2 | Mid | N/A | ALL RBs | 30 | Mid |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 5 | High/CC1 | 25@0 | ALL RBs | 5 | High/CC2 | N/A | ALL RBs | 2 | Mid | N/A | ALL RBs | 30 | Mid |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 25 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 30 | Mid | 50@0 | ALL RBs | 2 | Mid | N/A | ALL RBs | 5 | Mid/CC1 | N/A | ALL RBs | 5 | Mid/CC2 |  | N/A |  |  | ALL RBs | 66 |  | Mid |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 100 |  |
| 7 | 66 | Mid | 100@0 | ALL RBs | 2 | Mid | N/A | ALL RBs | 5 | Mid/CC1 | N/A | ALL RBs | 5 | Mid/CC2 |  | N/A |  |  | ALL RBs | 30 |  | Mid |  | N/A |  | ALL RBs |  |
|  | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 50 | N/A |  | QPSK |  |  |  | 50 |  |
| Test Settings for CA_3A-41C-42C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Mid | 50@50 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 216 | 3 | Low | 50@25 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 317 | 3 | High | 50@25 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 4 | 41 | Mid/CC1 | 100@0 | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 5 | 41 | Low/CC1 | 100@0 | ALL RBs | 41 | Low/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  |  | N/A |  | QPSK |  |  |  | 100 |  |
| 6 | 41 | Low/CC1 | 100@0 | ALL RBs | 41 | Low/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  |  | N/A |  | QPSK |  |  |  |  |  |
| 7 | 41 | High/CC1 | 100@0 | ALL RBs | 41 | High/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 8 | 41 | High/CC1 | 100@0 | ALL RBs | 41 | High/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 9 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 10 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 11 | 42 | High/CC1 | 100@0 | ALL RBs | 42 | High/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 1218 | 3 | Low | 50@25 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Low/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Low/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 1319 | 3 | High | 50@25 | ALL RBs | 41 | Mid/CC1 | N/A | ALL RBs | 41 | Mid/CC2 | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 14 | 42 | Low/CC1 | 100@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 15 | 42 | Mid/CC1 | 100@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 25 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 16 | 41 | Low/CC1 | 50@0 | ALL RBs | 41 | Low/CC1 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 17 | 41 | High/CC1 | 50@0 | ALL RBs | 41 | High/CC1 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 42 | Mid/CC1 |  | N/A |  |  | ALL RBs | 42 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 18 | 42 | Low/CC1 | 50@0 | ALL RBs | 42 | Low/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| 19 | 42 | Mid/CC1 | 50@0 | ALL RBs | 42 | Mid/CC2 | N/A | ALL RBs | 3 | Mid | N/A | ALL RBs | 41 | Mid/CC1 |  | N/A |  |  | ALL RBs | 41 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_29A-66C-70C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Mid/CC1 | 100@0 | ALL RBs | 66 | Mid/CC2 | N/A | ALL RBs | 29 | Mid | N/A | ALL RBs | 70 | Mid/CC1 |  | N/A |  |  | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 75 | N/A |  | QPSK |  |  |  | 50 |  |
| 2 | 66 | High/CC1 | 100@0 | ALL RBs | 66 | High/CC2 | N/A | ALL RBs | 29 | Mid | N/A | ALL RBs | 70 | Mid/CC1 |  | N/A |  |  | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 75 | N/A |  | QPSK |  |  |  | 50 |  |
| 3 | 70 | Mid/CC1 | 100@0 | ALL RBs | 70 | Mid/CC2 | N/A | ALL RBs | 29 | Mid | N/A | ALL RBs | 66 | High/CC1 |  | N/A |  |  | ALL RBs | 66 |  | High/CC2 |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 50 | N/A | QPSK |  |  |  |  | 100 | N/A |  | QPSK |  |  |  | 100 |  |
| Test Settings for CA_66A-66A-70C-71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Low/CC1 | 100@0 | ALL RBs | 66 | High/CC2 | N/A | ALL RBs | 70 | Mid/CC1 | N/A | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 50 |
| 2 | 66 | Low/CC1 | 100@0 | ALL RBs | 66 | High/CC2 | N/A | ALL RBs | 70 | Mid/CC1 | N/A | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 50 |
| 3 | 70 | Mid/CC1 | 75@0 | ALL RBs | 70 | Mid/CC2 | N/A | ALL RBs | 66 | Low/CC1 | N/A | ALL RBs | 66 |  | High/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |
| 4 | 71 | Low | 16@12 | ALL RBs | 66 | Low/CC1 | N/A | ALL RBs | 66 | High/CC2 | N/A | ALL RBs | 70 |  | Mid/CC1 |  | N/A |  | ALL RBs |  | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 50 |
| Test Settings for CA_66C-70C-71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Mid/CC1 | 100@0 | ALL RBs | 66 | Mid/CC2 | N/A | ALL RBs | 70 | Mid/CC1 | N/A | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 50 |
| 2 | 66 | High/CC1 | 100@0 | ALL RBs | 66 | High/CC2 | N/A | ALL RBs | 70 | Mid/CC1 | N/A | ALL RBs | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A | QPSK |  | 75 | N/A |  | QPSK |  |  |  | 50 |  | N/A |  | QPSK |  |  |  | 50 |
| 3 | 70 | Mid/CC1 | 75@0 | ALL RBs | 70 | Mid/CC2 | N/A | ALL RBs | 66 | Mid/CC1 | N/A | ALL RBs | 66 |  | Mid/CC2 |  | N/A |  | ALL RBs |  | 71 |  | Mid |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 75 | N/A | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A |  | QPSK |  |  |  | 100 |  | N/A |  | QPSK |  |  |  | 50 |
| 4 | 71 | Low | 16@12 | ALL RBs | 66 | Mid/CC1 | N/A | ALL RBs | 66 | Mid/CC2 | N/A | ALL RBs | 70 |  | Mid/CC1 |  | N/A |  | ALL RBs |  | 70 |  | Mid/CC2 |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  | 50 | N/A | QPSK |  | 100 | N/A | QPSK |  | 100 | N/A |  | QPSK |  |  |  | 75 |  | N/A |  | QPSK |  |  |  | 50 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41F, X=41. Note 6: Inter-band: X,Y,Z,R,S correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-20A-42A,X=1,Y=3,Z=7,R=20,S=42. Note 7: Intra-band contiguous + Inter-band: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_3C-7A-28A, X=3,Y=7 and Z=28. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 10: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. Note 11: Test points that fulfil criteria of Note 4 in Table 7.3A.5.5-3. Note 12: Only Band 1 and Band 42 need to be tested and Band 3 does not need to be tested. Note 13: Only Band 1 and Band 3 need to be tested and Band 42 does not need to be tested. Note 14: Band 42: fUL/DL for SCC1 = 3430.2MHz (NUL/DL = 41892), fUL/DL for SCC2 = 3450MHz (NUL/DL = 42090) Note 15: Band 42: fUL/DL for SCC1 = 3500.2MHz (NUL/DL = 42592), fUL/DL for SCC2 = 3520MHz (NUL/DL = 42790) Note 16: Band 3: fUL = 1720MHz (NUL = 19300), fDL = 1815MHz (NDL = 1300) Band 42: fUL/DL = 3440MHz (NUL/DL = 41990) Note 17: Band 3: fUL = 1775MHz (NUL = 19850), fDL = 1870MHz (NDL = 1850) Band 42: fUL/DL = 3520MHz (NDL = 42790) Note 18: Band 42: fUL/DL for SCC1 = 3430.2MHz (NUL/DL = 41892), fUL/DL for SCC2 = 3450MHz (NUL/DL = 42090) Note 19: Band 42: fUL/DL for SCC1 = 3500.2MHz (NUL/DL = 42592), fUL/DL for SCC2 = 3520MHz (NUL/DL = 42790). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.10.4.1-2: Test Configuration Table with Wgap

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Highest NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | Wgap [MHz] | SCC1 |  |  |  | Wgap [MHz] | SCC2 |  |  |  | Wgap [MHz] | SCC3 |  |  |  | Wgap [MHz] | SCC4 |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | UL alloc (Note 2,3,4) | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | UL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |  | UL MOD | DL MOD | UL alloc | DL alloc |
| Default Test Settings for a CA_XA-XA-YA-ZA-RA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REFSENS | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 2 | Y | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 3 | Z | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| 4 | R | Mid | REFSENS | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | Mid | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |  | N/A | QPSK |  | NRB_agg |
| Default Test Settings for a CA_XA-XA-YA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | CC2 | REF SENS | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | CC2 | N/A | ALL RBs | Max | Y | CC1 | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |
| 2 | Y | CC2 | REF SENS | ALL RBs | Max | Y | CC1 | N/A | ALL RBs | - | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |
| 3 | Z | Mid | REF SENS | ALL RBs |  | X | CC2 | N/A | ALL RBs | Max | X | CC1 | N/A | ALL RBs | - | Y | CC2 | N/A | ALL RBs | Max | Y | CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | NRB_ agg |  | QPSK | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |  | N/A | QPSK |  | NRB_ agg |
| Test Settings for CA_2A-5B-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 3 | 2 | Mid | REFSENS | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 4 | 5 | Mid/CC1 | REFSENS | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs | - | 2 | Mid | N/A | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for CA_5B-30A-66A-66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 5 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 30 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 5 | Mid | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 30 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 3 | 30 | Mid | REFSENS | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs | - | 5 | Mid/CC1 | N/A | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |
| 4 | 5 | Mid/CC1 | REFSENS | ALL RBs | - | 5 | Mid/CC2 | N/A | ALL RBs | - | 30 | Mid | N/A | ALL RBs | - | 66 | CC2 | N/A | ALL RBs | Max | 66 | CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |
| Test Settings for a CA_29A-66A-66A-70C Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 70 | Mid/CC1 | N/A | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 75 |  | N/A | QPSK |  | 50 |
| 2 | 66 | CC1 | REFSENS | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 70 | Mid/CC1 | N/A | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 100 |  | N/A | QPSK |  | 25 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 75 |  | N/A | QPSK |  | 50 |
| 3 | 70 | Mid/CC1 | REFSENS | ALL RBs | - | 70 | Mid/CC2 | N/A | ALL RBs | - | 29 | Mid | N/A | ALL RBs | - | 66 | CC1 | N/A | ALL RBs | Max | 66 | CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | 75 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 50 |  | N/A | QPSK |  | 100 |  | N/A | QPSK |  | 100 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.55 if the UE supports 2 Rx antennas or Figure A.80 if the UE supports 4 Rx antennas in at least one band within the CA configuration.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.1, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 7.3A.10.4.1-1 through Table 7.3A.10.4.1-2 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 7.3A.10.4.3.

##### 7.3A.10.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.10.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 7.3A.10.4.1-1 through Table7.3A.10.4.1-2 as appropriate on PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.10.4.1-1 through Table 7.3A.10.4.1-2 as appropriate on PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in Table 7.3A.10.5-1 through Table7.3A.10.5-5. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for duration sufficient to achieve statistical significance according to Annex G.2A.

##### 7.3A.10.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

#### 7.3A.10.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.10.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.10.5-1: Reference sensitivity QPSK PREFSENS for CA_XF Configuration

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_41F | 41 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  |  |  | -1004 | -97.04 | -95.24 | -94.04 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.10.5-2 (originated from single carrier Tables 7.3.5-1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.10.5-2: Reference sensitivity QPSK PREFSENS for CA_XA-YA-ZA-RA-SA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_XA-YA-ZA-RA-SA |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

Unless otherwise specified, in case the UE supports more than one of the above 5DL inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one 5DL inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 5DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 5DL carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional 5DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 5DL CA configurations

The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration.

The reference receive sensitivity (REFSENS) requirement for inter-band specified in Table 7.3A.10.5-2 shall be met for an uplink CA configurations than or equal to that specified in Table 7.3.5-2.

NOTE:	Table 7.3A.10.5-3 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.10.5-3: Void

For combinations of intra-band contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.10.5-4. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.10.5-4: Reference sensitivity QPSK PREFSENS for CA_XA-YA-ZA-RC or XA-YC-ZC or XA-YE or XC-YD or XD-YC (intra-band contiguous + inter-band)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration |  | EUTRA band |  | 1.4 MHz (dBm) |  | 3 MHz (dBm) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  |  | 20 MHz (dBm) |  | Duplex mode |
| CA_1A-3A-18A-42C |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 35 |  |  |  |  |  | -93.3 |  | -90.8 |  | -89.3 |  |  | -88.3 |  |  |
|  |  |  |  |  |  |  |  | -96.04 |  | -93.54 |  | -92.04 |  |  | -91.04 |  |  |
|  |  | 36 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 18 |  |  |  |  |  |  |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_1A-3A-18A-42C7,8 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 18 |  |  |  |  |  |  |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -71.2 |  | -71.2 |  | -71.2 |  |  | -71.2 |  | TDD |
| CA_1A-3A-18A-42C9 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 18 |  |  |  |  |  |  |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -96.6 |  | -94.2 |  | -92.7 |  |  | -92.0 |  | TDD |
| CA_1A-3A-19A-42C |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  | 35 |  |  |  |  |  | -93.1 |  | -90.6 |  | -89.1 |  |  | -88.1 |  |  |
|  |  | 36 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  | 19 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  | TDD |
| CA_1A-3A-19A-42C7,8 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  | 19 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -70.7 |  | -70.7 |  | -70.7 |  |  | -70.7 |  | TDD |
| CA_1A-3A-19A-42C9 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  | 19 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -96.1 |  | -93.7 |  | -92.2 |  |  | -91.5 |  | TDD |
| CA_1A-3A-21A-42C |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 35 |  |  |  |  |  | -93.0 |  | -90.5 |  | -89.0 |  |  | -88.0 |  |  |
|  |  | 36 |  |  |  |  |  | -96.0 |  | -93.0 |  | -91.2 |  |  | -90.0 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 21 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | - |  |  |
|  |  | 42 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  | TDD |
|  |  |  |  |  |  |  |  | -99.74 |  | -96.74 |  | -94.94 |  |  | -93.74 |  |  |
| CA_1A-3A-21A-42C7,8 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.0 |  | -93.0 |  | -91.2 |  |  | -90.0 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 21 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | - |  |  |
|  |  | 42 |  |  |  |  |  | -70.7 |  | -70.7 |  | -70.7 |  |  | -70.7 |  | TDD |
| CA_1A-3A-21A-42C9 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.0 |  | -93.0 |  | -91.2 |  |  | -90.0 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 21 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | - |  |  |
|  |  | 42 |  |  |  |  |  | -96.1 |  | -93.7 |  | -92.2 |  |  | -91.5 |  | TDD |
| CA_1A-19A-21A-42C |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  | 1 |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 19 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  | 42 |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_1A-3A-41C-42A |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 35 |  |  |  |  |  | -93.1 |  | -90.6 |  | -89.1 |  |  | -88.1 |  |  |
|  |  | 36 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  |  |
| CA_1A-3A-41C-42A 7,8 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -70.7 |  | -70.7 |  | -70.7 |  |  | -70.7 |  |  |
| CA_1A-3A-41C-42A 9 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -96.1 |  | -93.7 |  | -92.2 |  |  | -91.5 |  |  |
| CA_1A-3A-41A-42C |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 35 |  |  |  |  |  | -93.1 |  | -90.6 |  | -89.1 |  |  | -88.1 |  |  |
|  |  | 36 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  |  |
| CA_1A-3A-41A-42C 7,8 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -70.7 |  | -70.7 |  | -70.7 |  |  | -70.7 |  |  |
| CA_1A-3A-41A-42C 9 |  | 1 |  |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  |  | -93.1 |  | FDD |
|  |  |  |  |  |  |  |  | -101.84 |  | -98.84 |  | -97.04 |  |  | -95.84 |  |  |
|  |  | 3 |  |  |  |  |  | -96.1 |  | -93.1 |  | -91.3 |  |  | -90.1 |  |  |
|  |  |  |  |  |  |  |  | -98.74 |  | -95.74 |  | -93.94 |  |  | -92.74 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -1004 |  | -974 |  | -95.24 |  |  | -944 |  |  |
|  |  | 42 |  |  |  |  |  | -96.1 |  | -93.7 |  | -92.2 |  |  | -91.5 |  |  |
| CA_1A-3A-42D5 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.54 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -93.3 |  | -90.8 |  | -89.3 |  |  | -88.3 |  |  |
|  |  |  |  |  |  |  |  | -96.04 |  | -93.54 |  | -92.04 |  |  | -91.04 |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_1A-3A-42D6 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.54 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_1A-3A-42D7,8 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.54 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 42 |  |  |  |  |  | -71.2 |  | -71.2 |  | -71.2 |  |  | -71.2 |  | TDD |
|  |  |  |  |  |  |  |  | -73.44 |  | -73.44 |  | -73.44 |  |  | -73.44 |  |  |
| CA_1A-3A-42D9 |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.54 |  |  | -96.04 |  |  |
|  |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  |  |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 42 |  |  |  |  |  | -96.6 |  | -94.2 |  | -92.7 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -98.84 |  | -96.44 |  | -94.94 |  |  | -94.24 |  |  |
| CA_1A-41C-42C |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 41 |  |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  |  |  |  |  |  |  | -100.04 |  | -97.04 |  | -95.24 |  |  | -94.04 |  |  |
|  |  | 42 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  |  |
|  |  |  |  |  |  |  |  | -99.74 |  | -96.74 |  | -94.94 |  |  | -93.74 |  |  |
| CA_1A-42E |  | 1 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  | FDD |
|  |  |  |  |  |  |  |  | -102.04 |  | -99.04 |  | -97.24 |  |  | -96.04 |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_2A-5B-30A-66A |  | 2 |  |  |  |  |  | -97,3 |  | -94,3 |  | -92.5 |  |  | -91,3 |  | FDD |
|  |  |  |  |  |  |  |  | -100.04 |  | -97.04 |  | -95.24 |  |  | -94.04 |  |  |
|  |  | 5 |  |  |  |  |  | -97.3 |  | -94.3 |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  | -98.3 |  | -95.3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -101.04 |  | -98.04 |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  |  |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
| CA_2A-5B-66A-66A |  | 2 |  |  |  |  |  | -97,3 |  | -94,3 |  | -92.5 |  |  | -91,3 |  | FDD |
|  |  |  |  |  |  |  |  | -100.04 |  | -97.04 |  | -95.24 |  |  | -94.04 |  |  |
|  |  | 5 |  |  |  |  |  | -97.3 |  | -94.3 |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  |  |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
| CA_3A-42E |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  | FDD |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 42 |  |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  |  | -92.0 |  | TDD |
|  |  |  |  |  |  |  |  | -100.24 |  | -97.24 |  | -95.44 |  |  | -94.24 |  |  |
| CA_3C-7C-28A |  | 3 |  |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  |  | -90.3 |  | FDD |
|  |  |  |  |  |  |  |  | -99.04 |  | -96.04 |  | -94.24 |  |  | -93.04 |  |  |
|  |  | 7 |  |  |  |  |  |  |  | -94.3 |  | -92.5 |  |  | -91.3 |  |  |
|  |  |  |  |  |  |  |  |  |  | -97.04 |  | -95.24 |  |  | -94.04 |  |  |
|  |  | 28 |  |  |  |  |  |  |  | -94.8 |  | -93.0 |  |  | -90.3 |  |  |
| CA_5B-30A-66A-66A |  | 5 |  |  |  |  |  | -97.3 |  | -94.3 |  |  |  |  |  |  | FDD |
|  |  | 30 |  |  |  |  |  | -98.3 |  | -95.3 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -101.04 |  | -98.04 |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  |  |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
| CA_29A-66C-70C |  | 29 |  |  |  |  |  | -96.3 |  | -93.3 |  |  |  |  |  |  | FDD |
|  |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  |  |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
|  |  | 70 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  |  |
| CA_41C-48D |  | 41 |  |  |  |  |  |  |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  | 48 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  |  |
| CA_41D-48C |  | 41 |  |  |  |  |  |  |  | -94.3 |  | -92.5 |  |  | -91.3 |  | TDD |
|  |  | 48 |  |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  |  | -91.5 |  |  |
| CA_66A-66A-70C-71A |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  | FDD |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
|  |  | 70 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  |  |
|  |  |  |  |  |  |  |  | -89.38,9 |  | -88.88,9 |  | -88.58,9 |  |  | -88.38,9 |  |  |
|  |  | 71 |  |  |  |  |  | -96.5 |  | -93.5 |  | -91.2 |  |  | -86.8 |  |  |
| CA_66C-70C-71A |  | 66 |  |  |  |  |  | -98.8 |  | -95.8 |  | -94.0 |  |  | -92.8 |  |  |
|  |  |  |  |  |  |  |  | -101.54 |  | -98.54 |  | -96.74 |  |  | -95.54 |  |  |
|  |  | 70 |  |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  |  | -93.3 |  |  |
|  |  |  |  |  |  |  |  | -89.38,9 |  | -88.88,9 |  | -88.58,9 |  |  | -88.38,9 |  |  |
|  |  | 71 |  |  |  |  |  | -96.5 |  | -93.5 |  | -91.2 |  |  | -86.8 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 5 or Band 8 or Band 19 or Band 20 or Band 26 or Band 28 or Band 42, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 5 or Band 8 or Band 19 or Band 20 or Band 26 or Band 28 or Band 42, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42C, CA_1A-3A-42C and CA_3A-19A-42C. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.10.5-5. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.10.5-5: Reference sensitivity QPSK PREFSENS for CA_XA-XA-XD or XA-XC-XC or XA-YC-YC or XA-YA-YA-YC or XA-XA-YD or XA-XA-YA-YC or XA-XA-YA-YA-YA or XA-XA-XA-YC or XA-YA-ZA-ZA-ZA or XA-YA-YA-ZC or XA-YA-YA-ZA-ZA or XA-YA-ZA-RA-RA or CA_XA-XA-YA-ZA-RA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-2A-12A-66A-66A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |
|  | 12 |  |  | -96.3 | -96.5 |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 | -88 |  |
|  |  |  |  | -92.24 | -91.74 | -91.24 | -90.74 |  |
| CA_2A-5A-30A-66A-66A | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_29A-66A-66A-70C | 29 |  |  | -96.3 | -93.3 |  |  | FDD |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

## {End of changes}