3GPP TSG-RAN5 Meeting #101	R5-236558

# Chicago, November 13 - 17 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2961 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for CA_n48B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Current test frequencies for NR contiguous CA configuration CA_n48b for SCS=15 KHz and SSB SCS=30 kHz were calculated based on a restriction that only even Kssb values could be used to align carrier frequency raster with the band’s synchronization raster. As this restriction is not valid and is removed by CR-2957 / R5-236489, there is a need to update the test frequencies to span the test frequencies over the full band width of the NR band. - CBW combos CC1 CBW+CC2 CBW (BCS): 20+40 (1,2), 20+50 (BSC 1,2) are not listed in correct order in Table 4.3.1.1.3.48.1-1. - According to TS 38.101-1, Table 5.5A.1-1 are the applicable CBW combinations for CA_n48B and BCS 0 ,1 and 2: For SCS=15 kHz are the applicable CBWs 5, 10, 15, 20, 30, 40 and 50 MHz (50 MHz is only applicable as SCell for NR CA). Following CBW combinations CC1 CBW+CC2 CBW (BCS) are missing in the test frequency Table 4.3.1.1.3.48.1-1 for SCS=15kHz: 5+15 (0), 5+20 (0), 30+10 (2), 30+15 (2), 30+20 (2), 30+40 (2), 30+50 (2), 40+10, (2), 40+15 (2) and 40+20 (2). - As CBW=50MHz for n48 is only applciable as downlink SCell part of CA configuration (TS 38.101-1, Table 5.5A.1-1, Note 6) shall the test frequency Table 4.3.1.1.3.48.1-1 not include any CORESET#0 parameters and Kssb be set to 31. - Note 2 in Table 4.3.1.1.3.48.1-1 need to be clarified that CC2 is to be used as Pcell for case CC1 CBW=5MHz (only applicable as SCell, TS 38.101-1, Table 5.5A.1-1 Note 5). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated test frequency parameters in Table 4.3.1.1.3.48.1-1 where the Low, Mid and High test frequencies been shifted due to incorrect restriction as described above. - Corrected order of CBW combos CC1 CBW+CC2 CBW (BCS): 20+40 (1,2), 20+50 (BSC 1,2) in Table 4.3.1.1.3.48.1-1. - Added CBW combinations CC1 CBW+CC2 CBW (BCS): 5+15 (0), 5+20 (0), 30+10 (2), 30+15 (2), 30+20 (2), 30+40 (2), 30+50 (2), 40+10, (2), 40+15 (2) and 40+20 (2) to Table 4.3.1.1.3.48.1-1. For CBW=5MHz clarifeied Note 5. - For CBW=50MHz removed CORESET#0 parameters and set Kssb to 31. Added Note 7 and marked all incatnces of CBW=50Mhz with th enote in the table. - Note 2 clarified that for the case CC1 CBW=5MHz then CC2 shall be used as Pcell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test frequencies will note cover the full n48 bandwidth. - Incorrect order in table remains. - No test frequencies for a number of CA_n48B and BCS 0, 1 and 2 applicable CBW cominations specified. - Misleading test frequency parameters for CBW=50 MHz remains. - Note 2 will not reflect the case of CC1 CBW=5MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.48.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on CR-2957 / R5-236489 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

###### 4.3.1.1.3.48	NR Intra-band contiguous configurations CA_n48

###### 4.3.1.1.3.48.1	CA_n48B

Editor's note:	Test frequencies for CA_n48B with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 15 kHz or 60kHz is FFS.

Table 4.3.1.1.3.48.1-1: NR Intra-Band contiguous CA configuration CA_n48B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 + 15 | CC1 | 5 | 15 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | 30 | - | 636924 | 31 | - | - | - |
| (0) |  | Note 5 |  |  | & | Mid | 3617.505 | 641167 | 3596.895 | 639793 | 102 |  | - | 641257 | 31 | - | - | - |
|  |  |  |  |  | Uplink | High | 3682.635 | 645509 | 3589.665 | 639311 | 504 |  | - | 645599 | 31 | - | - | - |
|  | Channel spacing CC1-CC2=9.855 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3562.365 | 637491 | 3555.255 | 637017 | 0 | 30 | 7888 | 637344 | 3 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3627.36 | 641824 | 3601.89 | 640126 | 102 |  | 7933 | 641664 | 2 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 5+20 | CC1 | 5 | 15 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | 30 | - | 636924 | 31 | - | - | - |
| (0) |  | Note 5 |  |  |  | Mid | 3615 | 641000 | 3594.39 | 639626 | 102 |  | - | 641090 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3677.715 | 645181 | 3584.745 | 638983 | 504 |  | - | 645271 | 31 | - | - | - |
|  | Channel spacing CC1-CC2=12.285 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3564.795 | 637653 | 3555.255 | 637017 | 0 | 30 | 7888 | 637344 | 3 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3627.285 | 641819 | 3599.385 | 639959 | 102 |  | 7931 | 641472 | 1 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 10+10 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3619.995 | 641333 | 3596.955 | 639797 | 102 |  | 7930 | 641376 | 7 | 3 | 1 (6) | 111 |
| Note 6 |  |  |  |  | Uplink | High | 3685.035 | 645669 | 3589.635 | 639309 | 504 |  | 7975 | 645696 | 3 | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=9.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3564.96 | 637664 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  | & | Mid | 3629.955 | 641997 | 3606.915 | 640461 | 102 |  | 7936 | 641952 | 3 | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3694.995 | 646333 | 3599.595 | 639973 | 504 |  | 7982 | 646368 | 11 | 2 | 1 (6) | 512 |
| 10+15 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.505 | 641167 | 3594.465 | 639631 | 102 |  | 7928 | 641184 | 5 | 1 | 1 (6) | 109 |
| Note 6 |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3584.61 | 638974 | 504 |  | 7971 | 645312 | 2 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3567.48 | 637832 | 3560.37 | 637358 | 0 | 30 | 7891 | 637632 | 10 | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3629.985 | 641999 | 3604.515 | 640301 | 102 |  | 7935 | 641856 | 7 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 10+20 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3591.96 | 639464 | 102 |  | 7926 | 640992 | 4 | 3 | 0 (2) | 107 |
| Note 6 |  |  |  |  | Uplink | High | 3675.36 | 645024 | 3579.96 | 638664 | 504 |  | 7968 | 645024 | 0 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3569.64 | 637976 | 3560.1 | 637340 | 0 | 30 | 7891 | 637632 | 4 | 2 | 0 (2) | 4 |
|  |  |  |  |  | & | Mid | 3629.64 | 641976 | 3601.74 | 640116 | 102 |  | 7933 | 641664 | 0 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 10+30 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3610.005 | 640667 | 3586.965 | 639131 | 102 |  | 7923 | 640704 | 1 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3665.31 | 644354 | 3569.91 | 637994 | 504 |  | 7961 | 644352 | 10 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3574.68 | 638312 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3629.685 | 641979 | 3596.925 | 639795 | 102 |  | 7930 | 641376 | 9 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 10+40 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3604.995 | 640333 | 3581.955 | 638797 | 102 |  | 7919 | 640320 | 11 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3655.635 | 643709 | 3560.235 | 637349 | 504 |  | 7954 | 643680 | 7 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3579.36 | 638624 | 3559.92 | 637328 | 0 | 30 | 7891 | 637632 | 4 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3629.355 | 641957 | 3591.555 | 639437 | 102 |  | 7926 | 640992 | 7 | 1 | 1 (6) | 109 |
|  |  |  |  |  |  | High | 3679.995 | 645333 | 3569.835 | 637989 | 504 |  | 7961 | 644352 | 3 | 0 | 1 (6) | 510 |
| 10+50 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3576.96 | 638464 | 102 |  | 7916 | 640032 | 8 | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3645.6 | 643040 | 3550.2 | 636680 | 504 |  | 7947 | 643008 | 4 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=29.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 Note 7 | 15 | 270 | Downlink | Low | 3584.4 | 638960 | 3560.1 | 637340 | 0 | 30 | - | 637632 | 31 | - | - | - |
|  |  |  |  |  |  | Mid | 3629.4 | 641960 | 3586.74 | 639116 | 102 |  | - | 640608 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 15+10 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (0) |  |  |  |  | & | Mid | 3619.995 | 641333 | 3594.525 | 639635 | 102 |  | 7928 | 641184 | 1 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3682.515 | 645501 | 3584.685 | 638979 | 504 |  | 7971 | 645312 | 9 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3569.985 | 637999 | 3565.305 | 637687 | 0 | 30 | 7895 | 638016 | 5 | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3632.475 | 642165 | 3609.435 | 640629 | 102 |  | 7938 | 642144 | 3 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3694.995 | 646333 | 3599.595 | 639973 | 504 |  | 7982 | 646368 | 11 | 2 | 1 (6) | 512 |
| 15+15 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.505 | 641167 | 3592.035 | 639469 | 102 |  | 7926 | 640992 | 11 | 2 | 0 (2) | 106 |
| Note 6 |  |  |  |  | Uplink | High | 3677.49 | 645166 | 3579.66 | 638644 | 504 |  | 7968 | 645024 | 8 | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3572.505 | 638167 | 3565.395 | 637693 | 0 | 30 | 7895 | 638016 | 11 | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3632.505 | 642167 | 3607.035 | 640469 | 102 |  | 7937 | 642048 | 7 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3572.505 | 638167 | 3565.395 | 637693 | 0 | 30 | 7895 | 638016 | 11 | 0 | 1 (6) | 6 |
| 15+20 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3589.53 | 639302 | 102 |  | 7924 | 640800 | 10 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3672.84 | 644856 | 3575.01 | 638334 | 504 |  | 7965 | 644736 | 6 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3574.665 | 638311 | 3565.125 | 637675 | 0 | 30 | 7895 | 638016 | 5 | 2 | 1 (6) | 8 |
|  |  |  |  |  | & | Mid | 3632.16 | 642144 | 3604.26 | 640284 | 102 |  | 7935 | 641856 | 0 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 15+30 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3610.005 | 640667 | 3584.535 | 638969 | 102 |  | 7921 | 640512 | 7 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3662.79 | 644186 | 3564.96 | 637664 | 504 |  | 7958 | 644064 | 4 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3579.705 | 638647 | 3565.305 | 637687 | 0 | 30 | 7895 | 638016 | 5 | 1 | 1 (6) | 7 |
|  |  |  |  |  |  | Mid | 3632.205 | 642147 | 3599.445 | 639963 | 102 |  | 7931 | 641472 | 9 | 1 | 0 (2) | 105 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 15+40 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (0,1,2) |  |  |  |  | & | Mid | 3604.995 | 640333 | 3579.525 | 638635 | 102 |  | 7917 | 640128 | 5 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3653.115 | 643541 | 3555.285 | 637019 | 504 |  | 7951 | 643392 | 1 | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3584.385 | 638959 | 3564.945 | 637663 | 0 | 30 | 7895 | 638016 | 5 | 3 | 1 (6) | 9 |
|  |  |  |  |  |  | Mid | 3631.875 | 642125 | 3594.075 | 639605 | 102 |  | 7928 | 641184 | 7 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.995 | 645333 | 3569.835 | 637989 | 504 |  | 7961 | 644352 | 3 | 0 | 1 (6) | 510 |
| 15+50 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.505 | 637167 | 3550.395 | 636693 | 0 | 30 | 7884 | 636960 | 3 | 0 | 0 (2) | 2 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3574.53 | 638302 | 102 |  | 7914 | 639840 | 2 | 0 | 1 (6) | 108 |
| Note 6 |  |  |  |  | Uplink | High | 3643.08 | 642872 | 3545.25 | 636350 | 504 |  | 7944 | 642720 | 10 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=31.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3589.425 | 639295 | 3565.125 | 637675 | 0 | 30 | - | 638016 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3631.92 | 642128 | 3589.26 | 639284 | 102 |  | - | 640800 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 20+10 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3619.995 | 641333 | 3592.095 | 639473 | 102 |  | 7926 | 640992 | 7 | 2 | 0 (2) | 106 |
| Note 6 |  |  |  |  | Uplink | High | 3680.355 | 645357 | 3580.095 | 638673 | 504 |  | 7968 | 645024 | 3 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3574.65 | 638310 | 3569.97 | 637998 | 0 | 30 | 7898 | 638304 | 6 | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.635 | 642309 | 3611.595 | 640773 | 102 |  | 7940 | 642336 | 3 | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3694.995 | 646333 | 3599.595 | 639973 | 504 |  | 7982 | 646368 | 11 | 2 | 1 (6) | 512 |
| 20+15 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3617.505 | 641167 | 3589.605 | 639307 | 102 |  | 7924 | 640800 | 5 | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3675.33 | 645022 | 3575.07 | 638338 | 504 |  | 7965 | 644736 | 2 | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3577.17 | 638478 | 3570.06 | 638004 | 0 | 30 | 7898 | 638304 | 0 | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.665 | 642311 | 3609.195 | 640613 | 102 |  | 7938 | 642144 | 7 | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20+20 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3587.1 | 639140 | 102 |  | 7923 | 640704 | 4 | 2 | 1 (6) | 110 |
| Note 6 |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3569.76 | 637984 | 504 |  | 7961 | 644352 | 8 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=19.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3579.99 | 638666 | 3570.45 | 638030 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3634.98 | 642332 | 3607.08 | 640472 | 102 |  | 7937 | 642048 | 4 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 20+30 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3610.005 | 640667 | 3582.105 | 638807 | 102 |  | 7919 | 640320 | 1 | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3660.03 | 644002 | 3559.77 | 637318 | 504 |  | 7954 | 643680 | 2 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3584.97 | 638998 | 3570.57 | 638038 | 0 | 30 | 7898 | 638304 | 2 | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3634.965 | 642331 | 3602.205 | 640147 | 102 |  | 7933 | 641664 | 5 | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 20+40 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.995 | 640333 | 3577.095 | 638473 | 102 |  | 7916 | 640032 | 11 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3650.295 | 643353 | 3550.035 | 636669 | 504 |  | 7947 | 643008 | 3 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3589.71 | 639314 | 3570.27 | 638018 | 0 | 30 | 7898 | 638304 | 10 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3634.695 | 642313 | 3596.895 | 639793 | 102 |  | 7930 | 641376 | 11 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.995 | 645333 | 3569.835 | 637989 | 504 |  | 7961 | 644352 | 3 | 0 | 1 (6) | 510 |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3572.1 | 638140 | 102 |  | 7912 | 639648 | 8 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3640.26 | 642684 | 3540 | 636000 | 504 |  | 7940 | 642336 | 0 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3594.75 | 639650 | 3570.45 | 638030 | 0 | 30 | - | 638304 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3634.74 | 642316 | 3592.08 | 639472 | 102 |  | - | 640992 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 30+10 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3619.995 | 641333 | 3587.235 | 639149 | 102 |  | 7923 | 640704 | 7 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3675.315 | 645021 | 3570.195 | 638013 | 504 |  | 7961 | 644352 | 3 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3584.685 | 638979 | 3580.005 | 638667 | 0 | 30 | 7905 | 638976 | 9 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3639.675 | 642645 | 3616.635 | 641109 | 102 |  | 7943 | 642624 | 3 | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3694.995 | 646333 | 3599.595 | 639973 | 504 |  | 7982 | 646368 | 11 | 2 | 1 (6) | 512 |
| 30+15 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3617.505 | 641167 | 3584.745 | 638983 | 102 |  | 7921 | 640512 | 5 | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3670.29 | 644686 | 3565.17 | 637678 | 504 |  | 7958 | 644064 | 2 | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3587.205 | 639147 | 3580.095 | 638673 | 0 | 30 | 7905 | 638976 | 3 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3639.705 | 642647 | 3614.235 | 640949 | 102 |  | 7942 | 642528 | 7 | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 30+20 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3582.24 | 638816 | 102 |  | 7919 | 640320 | 4 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3665.04 | 644336 | 3559.92 | 637328 | 504 |  | 7954 | 643680 | 4 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3589.965 | 639331 | 3580.425 | 638695 | 0 | 30 | 7905 | 638976 | 5 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3639.96 | 642664 | 3612.06 | 640804 | 102 |  | 7940 | 642336 | 8 | 3 | 0 (2) | 107 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 30+30 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3610.005 | 640667 | 3577.245 | 638483 | 102 |  | 7916 | 640032 | 1 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3654.99 | 643666 | 3549.87 | 636658 | 504 |  | 7947 | 643008 | 2 | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=30 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3595.005 | 639667 | 3580.605 | 638707 | 0 | 30 | 7905 | 638976 | 5 | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3640.005 | 642667 | 3607.245 | 640483 | 102 |  | 7937 | 642048 | 5 | 2 | 1 (6) | 110 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 30+40 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3604.995 | 640333 | 3572.235 | 638149 | 102 |  | 7912 | 639648 | 11 | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3645.315 | 643021 | 3540.195 | 636013 | 504 |  | 7940 | 642336 | 11 | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3599.685 | 639979 | 3580.245 | 638683 | 0 | 30 | 7905 | 638976 | 5 | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3639.675 | 642645 | 3601.875 | 640125 | 102 |  | 7933 | 641664 | 3 | 0 | 1 (6) | 108 |
|  |  |  |  |  |  | High | 3679.995 | 645333 | 3569.835 | 637989 | 504 |  | 7961 | 644352 | 3 | 0 | 1 (6) | 510 |
| 30+50 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.005 | 637667 | 3550.605 | 636707 | 0 | 30 | 7885 | 637056 | 1 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3600 | 640000 | 3567.24 | 637816 | 102 |  | 7909 | 639360 | 8 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3635.28 | 642352 | 3530.16 | 635344 | 504 |  | 7933 | 641664 | 8 | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=39.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3604.725 | 640315 | 3580.425 | 638695 | 0 | 30 | - | 638976 | 31 | - | - | - |
|  |  | Note 7 |  |  |  | Mid | 3639.72 | 642648 | 3597.06 | 639804 | 102 |  | - | 641376 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| 40+10 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3619.995 | 641333 | 3582.195 | 638813 | 102 |  | 7919 | 640320 | 7 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3670.635 | 644709 | 3560.475 | 637365 | 504 |  | 7954 | 643680 | 3 | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3594.36 | 639624 | 3589.68 | 639312 | 0 | 30 | 7912 | 639648 | 0 | 2 | 1 (6) | 8 |
|  |  |  |  |  |  | Mid | 3644.355 | 642957 | 3621.315 | 641421 | 102 |  | 7946 | 642912 | 3 | 0 | 0 (2) | 104 |
|  |  |  |  |  |  | High | 3694.995 | 646333 | 3599.595 | 639973 | 504 |  | 7982 | 646368 | 11 | 2 | 1 (6) | 512 |
| 40+15 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3617.505 | 641167 | 3579.705 | 638647 | 102 |  | 7918 | 640224 | 5 | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3665.61 | 644374 | 3555.45 | 637030 | 504 |  | 7951 | 643392 | 2 | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3596.88 | 639792 | 3589.77 | 639318 | 0 | 30 | 7912 | 639648 | 6 | 1 | 1 (6) | 7 |
|  |  |  |  |  |  | Mid | 3644.385 | 642959 | 3618.915 | 641261 | 102 |  | 7945 | 642816 | 7 | 1 | 1 (6) | 109 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 40+20 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3577.2 | 638480 | 102 |  | 7916 | 640032 | 4 | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3660.3 | 644020 | 3550.14 | 636676 | 504 |  | 7947 | 643008 | 8 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3599.7 | 639980 | 3590.16 | 639344 | 0 | 30 | 7912 | 639648 | 4 | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3644.7 | 642980 | 3616.8 | 641120 | 102 |  | 7943 | 642624 | 4 | 1 | 0 (2) | 105 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 40+30 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3610.005 | 640667 | 3572.205 | 638147 | 102 |  | 7912 | 639648 | 1 | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3650.31 | 643354 | 3540.15 | 636010 | 504 |  | 7940 | 642336 | 2 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3604.68 | 640312 | 3590.28 | 639352 | 0 | 30 | 7912 | 639648 | 8 | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3644.685 | 642979 | 3611.925 | 640795 | 102 |  | 7940 | 642336 | 5 | 0 | 1 (6) | 108 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 40+40 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.995 | 640333 | 3567.195 | 637813 | 102 |  | 7909 | 639360 | 11 | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3640.035 | 642669 | 3529.875 | 635325 | 504 |  | 7933 | 641664 | 3 | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=39.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3609.96 | 640664 | 3590.52 | 639368 | 0 | 30 | 7912 | 639648 | 4 | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3644.955 | 642997 | 3607.155 | 640477 | 102 |  | 7937 | 642048 | 11 | 2 | 1 (6) | 110 |
|  |  |  |  |  |  | High | 3679.995 | 645333 | 3569.835 | 637989 | 504 |  | 7961 | 644352 | 3 | 0 | 1 (6) | 510 |
| 40+50 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3562.2 | 637480 | 102 |  | 7905 | 638976 | 8 | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3630.06 | 642004 | 3519.9 | 634660 | 504 |  | 7926 | 640992 | 8 | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=44.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 Note 7 | 15 | 270 | Downlink | Low | 3614.94 | 640996 | 3590.64 | 639376 | 0 | 30 | -7912 | 639648 | 31 | - | - | - |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3602.28 | 640152 | 102 |  | - | 641664 | 31 | - | - | - |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  | - | 643680 | 31 | - | - | - |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell for CBW>5 MHz if nothing else is specified in the test case. For the case CC1 CBW=5 MHz then CC2 is used as Pcell- Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: This UE channel bandwidth is restricted for operation when carrier is configured as a downlink SCell part of a DC or CA configuration (TS 38.101-1, Table 5.3.5-1 Note 6). As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 6: UL CA is only supported for BCS0. I.e.the test frequencies for uplink CC2 are only supported for BCS0. Note 7: This UE channel bandwidth is restricted for operation when carrier is configured as a downlink SCell part of a DC or CA configuration (TS 38.101-1, Table 5.3.5-1 Note 7). As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### << End of changes>>